3GPP TSG-SA WG6 Meeting #19	S6-171158

Dubrovnik, Croatia, 9th – 13th October 2017	(revision of S6-17xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0082 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of interconnection affiliation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCSMI |  |  |  |  |  |  | Date: |  | 2017-09-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of group affiliation for interconnection of MC service systems |  |  |  |  |  |  |  |  |
| Summary of change: |  | Existing group affiliation procedures for multiple MC systems clarified to apply to case where MC systems are in the same trust domain, and new affiliation procedure introduced for case where MC systems are in different trust domains and require interconnection procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interconnection group affiliation will not be introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 10.8.3.2, 10.8.3.2.1, 10.8.3.2.2, 10.8.3.2a (new), 10.8.3.2a.1 (new), 10.8.3.2a.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 23.281 CRxx, TS 23.379 CRxx. |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Additional definitions in clause 3.1 are referred to in 23.379 CR0080 and in 23.281 CR0081 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Active MC service user profile: The MC service user profile that is currently used by an MC service client of an MC service user while receiving MC service.

Interconnection: A means of communication between MCPTT systems whereby MCPTT users obtaining service from one MCPTT system can communicate with other MCPTT users who are obtaining MCPTT service from one or more other MCPTT systems.

Interconnection group: An MCPTT group that is configured to allow inclusion of group members who are users from partner MCPTT system(s).

Location: The current physical location of the MC service UE.

MC service: A generic name for any one of the three mission critical services: either MCPTT, or MCVideo, or MCData.

MC service affiliated group member: An MC service user who has indicated an interest in a particular MC service group and has been accepted to participate in MC service group communication for that MC service group.

MC service client: A generic name for the client application function of a specific MC service. MC service client could be replaced by MCPTT client, or MCVideo client, or MCData client depending on the context.

MC service group: A defined set of MC service users with associated communication dispositions (e.g. media restrictions, default priority and commencement directions) configured for the use with one or more MC services.

MC service group affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is determined.

MC service group call: A mechanism by which an MC service user can make a one-to-many MC service(s) transmission to other users that are members of MC service group(s).

MC service group de-affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is removed.

MC service group home system: The mission critical system where the MC service group is defined.

MC service group host MC service server: The MC service server within a mission critical system which provides centralised support for a particular MC service of an MC service group defined in a MC service group home system.

MC service group member: An MC service user, whose MC service ID is listed in a particular MC service group.

MC service ID: A generic name for the user ID of a mission critical user within a specific MC service. MC service ID could be replaced by MCPTT ID, or MCVideo ID, or MCData ID depending on the context.

MC service server: A generic name for the server application function of a specific MC service. MC service server could be replaced by MCPTT server, MCVideo server, or MCData server depending on the context.

MC service user: An authorized user, who can use an MC service UE to participate in one or more MC services.

MC service user profile: The set of information associated to an MC service user that allows that user to employ one or more MC services in a given role and from a given MC service UE.

MC service UE: A UE that can be used to participate in one or more MC services.

MC user: A user, identified by an MC ID, who, after authorization, obtains mission critical service(s).

Mission critical system: The collection of applications, services, and enabling capabilities required to provide a single mission critical service or multiple mission critical services to one or more mission critical organizations.

Pre-selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile through configuration, and applicable for an authenticated MC service user upon MC service authorization.

Selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile for an MC service upon request by an MC service user.

Trust domain: The system or set of systems within which services may be provided, and signalling and data may be shared, on the basis of authorization from a single source.

For the purposes of the present document, the following terms given in 3GPP TS 22.280 [3] apply

Mission Critical

Mission Critical Applications

Mission Critical Organization

Mission Critical Service

* * * Next Change * * * *

#### 10.8.3.2	Affiliation to MC service group(s) defined in partner MC service system in the same trust domain

##### 10.8.3.2.1	Functional description

When an MC service client wants to affiliate to MC service group(s) which is defined in a partner MC service system for a single MC service and where that partner MC system is in the same trust domain as the primary MC service system of the MC service client, it shall be subject to authorization from the partner MC service system where the MC service group(s) is defined, and whether it subjects to authorization from the primary MC service system is conditional.

##### 10.8.3.2.2	Procedure

The procedure for affiliation to MC service group(s) which is defined in a partner MC service system in the same trust domain as the primary MC system of the MC service client for a single MC service is described in figure 10.8.3.2.2-1.

Pre-conditions:

1.	The MC service client has already been provisioned (statically or dynamically) with the group information, or a pointer to the group information, that the MC service client is allowed to be affiliated.

2.	The MC service server of the primary MC service system may have locally cached the MC service group affiliation status of the MC service user.

3.	The MC service server of the partner MC service system may have retrieved the group related information from the group management server.

4.	The MC service client may have indicated to the group management server of the partner MC service system that it wishes to receive updates of group configuration data for MC service group(s) for which it is authorized (as described in subclause 10.1.5.3).

5.	The MC service user triggers the affiliation procedure. This is an explicit affiliation caused by the MC service user.

Figure 10.8.3.2.2-1: Affiliation for an MC service group defined in partner MC service system in same trust domain as MC service client

1.	The MC service client requests the MC service server of the primary MC service system to affiliate to an MC service group or a set of MC service groups.

2.	The MC service server of the primary MC service system checks if the MC service client is authorized to affiliate to the requested MC service group(s) based on the user subscription. The MC service server also performs the check for the maximum limit of the total number (N2) of MC service groups that the user can be affiliated to simultaneously.

3.	Based on the group information included in the request, the MC service server of the primary MC service system, it determines to send group affiliation request to the corresponding MC service server of the partner MC service system. The request may be routed through intermediate signalling nodes.

4a.	The MC service server of the partner MC service system checks if the group policy is locally cached. If the group policy is not locally cached on the MC service server then MC service server subscribes to the group policy from the group management server.

4b.	The MC service server of the partner MC service system receives the group policy from the group management server via notification and locally caches the group policy information.

5.	Based on the group policy, the MC service server of the partner MC service system checks if the MC service group(s) is not disabled and if the user of the MC service client is authorised to affiliate to the requested MC service group(s).

6.	If the user of the MC service client is authorised to affiliate to the requested MC service group(s) then the MC service server of the partner MC service system stores the affiliation status of the user for the requested MC service group(s).

7.	The MC service server of the partner MC service system sends the affiliation status result of requested MC service group(s) to the MC service server of the primary MC service system (7a) and updates the group management server with the affiliation status of the user for the requested MC service group(s) (7b).

NOTE:	Steps 7a and 7b can occur in any order or in parallel.

8.	The MC service server of the primary MC service system stores the affiliation status of the user for the requested MC service group(s).

9.	The MC service server of the primary MC service system sends the group affiliation status result for the requested MC service group(s) to the MC service client.

#### 10.8.3.2a	Affiliation to MC service group(s) defined in partner MC service system in a different trust domain using interconnection

##### 10.8.3.2a.1	Functional description

When an MC service client wants to affiliate to MC service group(s) which is defined in a partner MC service system for a single MC service and where that partner MC system is in a different trust domain from the primary MC service system of the MC service client, it shall be subject to authorization from the partner MC service system where the MC service group(s) is defined, and may also be subject to authorization from the primary MC system of the MC service client.

##### 10.8.3.2a.2	Procedure

Figure 10.8.3.2a.2-1 illustrates the group affiliation procedure for an MC service group defined in the partner MC service system where interconnection procedures for different trust domains are required.

Pre-conditions:

1.	MC service client 1 is service authorized within its primary MC system.

2.	The group host MC service server of the MC service group to which the MC service user of MC service client 1 wishes to affiliate is located in the partner MC system of MC service client 1

Figure 10.8.3.2a.2-1: Group affiliation to an MCservice group defined in partner MC service system in different trust domain

1.	MC service client 1 initiates an MC service group affiliation request to the primary MC service server of MC service client 1 on behalf of its MC service user.

2.	The MC service server of the primary MC service system determines the group host MC service server of the MC service group. The MC service group is identified to have a different group home MC system, and the primary MC system gateway MC service server is determined to act as proxy group host MC service server for the MC service group.

3.	The MC service server of the primary MC service system forwards the MC service group affiliation request to the gateway MC service server of the primary MC service system.

4.	The gateway MC service server determines which MC system is the group home system for the MC service group.

5.	The gateway MC service server in the primary system of MC service client 1 forwards the MC service group affiliation request to the gateway MC service server of the group home MC system of the MC service group.

6.	The gateway MC service server of the group home MC system of the MC service group determines which MC service server is the group host MC service server for that MC service group.

7.	The gateway MC service server of the group home system of the MC service group forwards the MC service group affiliation request to the group host MC service server of the MC service group.

8.	The group host MC service server in the partner MC service system checks whether the user of MC service client 1 is authorized to affiliate to the MC service group.

9.	The group host MC service server sends an MC service group affiliation response to the gateway MC service server in the group home MC system of the MC service group

10.	The gateway MC service server in the group home MC system forwards the MC service group affiliation response to the gateway MC service server in the primary MC system of MC service client 1.

11.	The gateway MC service server in the primary MC system of MC service client 1 forwards the MC service group affiliation response to the primary MC service server of MC service client 1.

12.	The primary MC service server of MC service client 1 forwards the MC service group affiliation response to MC service client 1.

* * * End Changes * * * *