3GPP TSG-SA WG6 Meeting #19	S6-171162

Dubrovnik, Croatia, 9th – 13th October 2017	(revision of S6-17xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0080 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional model for MCVideo interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCSMI |  |  |  |  |  |  | Date: |  | 2017-09-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inclusion of gateway MCVideo server in functional model to permit interconnection, where gateway MCVideo server provides topology hiding at the application plane from interconnected MCVideo systems, and acts as a application proxy for the interconnected MCVideo system in a controlling or participating role in calls. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of gateway MCVideo server and new reference point MCVideo-10 to functional model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interconnection to partner MCVideo system in different trust domain is not satisfied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | SIP-4 reference point specified in 23.280 CR0080 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.1.1	On-network functional model

Figure 6.1.1-1 shows the functional model for the application plane of MCVideo service for on-network operations.

Figure 6.1.1-1: Functional model for application plane of MCVideo service

In the model shown in figure 6.1.1-1, the following apply:

-	The MCVideo server is an instantiation of a MC service server in accordance with 3GPP TS 23.280 [6].

-	The MCVideo server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [9].

-	MCVideo-9 carries signalling over multicast bearer between the transmission control server of the MCVideo server and the transmission control participant of the MCVideo UE.

-	MCVideo-4 carries signalling over unicast bearer between the transmission control server of the MCVideo server and the transmission control participant of the MCVideo UE.

* * * Next Change * * * *

##### 6.2.2.3.x	Gateway MCVideo server

The gateway MCVideo server provides support for MCVideo interconnection services with a partner MCVideo system in a different trust domain, and is based on the MCVideo server described in subclause 6.2.2.3.2 of the present document.

The gateway MCVideo server functional entity does not act as a GCS AS described in 3GPP TS 23.468 [9] and does not control bearers for calls in the MCVideo system.

The gateway MCVideo server is responsible for relaying call control and floor control signalling messages, and media between MCVideo servers within the MCVideo system and the interconnected MCVideo system.

The gateway MCVideo server supports the controlling and participating role on behalf of the interconnected MCVideo system. If the interconnected MCVideo system has the controlling role in an MCVideo call, the gateway MCVideo server acts in this role within its MCVideo system on behalf of the connected MCVideo system. If an MCVideo server within MCVideo system of the gateway MCVideo server has the controlling role within an MCVideo call, the gateway MCVideo server takes a participating role on behalf of the interconnected MCVideo system.

The gateway MCVideo server does not act as the primary MCVideo server for MCVideo users, or as group host MCVideo server for MCVideo groups. However when acting in a controlling or participating role in MCVideo calls on behalf of the interconnected MCVideo system, it acts as if it were the primary MCVideo server for users receiving service in the interconnected MCVideo system and as if it were the group host MCVideo server for groups hosted in the interconnected MCVideo system.

* * * Next Change * * * *

#### 6.3.x	Reference point MCVideo-10 (between the gateway MCVideo server and the gateway MCVideo server in a different MCVideo system)

The MCVideo-10 reference point, which exists between the gateway MCVideo server and the gateway MCVideo server in an interconnected MCVideo system for MCVideo application signalling for establishing MCVideo sessions, shall use the SIP-4 reference point for transport and routing of signalling. Floor control signalling and media are also transferred using the MCVideo-10 reference point between interconnected MCVideo systems.

* * * End Changes * * * *