3GPP TSG-SA WG6 Meeting #19	S6-171163

Dubrovnik, Croatia, 9th – 13th October 2017	(revision of S6-17xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0081 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of interconnection MCVideo group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCSMI |  |  |  |  |  |  | Date: |  | 2017-09-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of group call for interconnection of MCVideo systems |  |  |  |  |  |  |  |  |
| Summary of change: |  | Existing mulitple MCVideo system group call procedures clarified to apply to case where MCVideo systems are in the same trust domain, and new group call procedure introduced for case where MCVideo systems are in different trust domains and require interconnection procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interconnection MCVideo group call will not be introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Additional definitions in clause 3.1 refer to 23.280 CR0082 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

MCVideo client: An instance of an MC service client that provides the client application function for the MCVideo service.

MCVideo group: An MC service group configured for MCVideo service.

MCVideo group affiliation: An MC service group affiliation for MCVideo.

MCVideo group de-affiliation: An MC service group de-affiliation for MCVideo.

MCVideo group home system: The mission critical system where the MCVideo group is defined.

MCVideo group host server: The MCVideo server within a mission critical system which provides centralised support for MCVideo services of an MCVideo group defined in a MCVideo group home system.

MCVideo ID: An instance of an MC service ID within the MCVideo service.

MCVideo server: An instance of an MC service server that provides the server application function for the MCVideo service.

MCVideo service: A video communication service supporting applications for mission critical organizations and mission critical applications for other businesses and organizations (e.g., utilities, railways) with strong security, high availability, reliability and priority handling.

MCVideo system: The collection of applications, services, and enabling capabilities required to provide Mission Critical video for a Mission Critical Organization.

MCVideo UE: An MC service UE that can be used to participate in MCVideo services.

MCVideo user: An MC service user who is authorized for MCVideo services via an MCVideo UE.

Transmission control: Video transmitting control mechanism in an MCVideo service that determines which participants have the authority to transmit video, and determines the onward downlink video transmission during an video call.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.280 [2] apply:

Mission Critical

Mission Critical Applications

Mission Critical Service

Mission Critical Organization

Mission Critical System

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.281 [3] apply:

Real Time

Real Time Video

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.280 [6] apply:

Interconnection

Interconnection group

MC service client

MC service group

MC service group affiliation

MC service group de-affiliation

MC service group home system

MC service group host

MC service ID

MC service server

Trust domain

* * * Next Change * * * *

#### 7.1.2.x	Group call involving groups from multiple MCVideo systems in different trust domains

##### 7.1.2.x.1	Group call for an MCVideo group defined in the partner MCVideo system in a different trust domain

Figure 7.1.2.x.1-1 illustrates the group call setup procedure for an MCVideo group defined in the partner MCVideo system where interconnection procedures for different trust domains are required.

Pre-conditions:

1.	The group host MCVideo server of the MCVideo group is located in the partner MC system of MCVideo client 1

2.	MCVideo client 1 in the primary MC system and MCVideo client 2 in the partner MC system of MCVideo client 1 are affiliated to the MCVideo group.

Figure 7.1.2.x.1-1: Group call for an MCVideo group defined in partner MCVideo system (initiating)

1.	The affiliated MCVideo user initiates a group call to the MCVideo group using MCVideo client 1. A group call request message with the MCVideo group ID is routed to the MCVideo server in the primary MC system of MCVideo client 1.

2.	The MCVideo server of the primary MCVideo system determines the group host MCVideo server of the MCVideo group. The MCVideo group is identified to have a different group home system, and the primary MC system gateway MCVideo server is determined to act as proxy group host MCVideo server for the MCVideo group.

3.	The MCVideo server of the primary MCVideo system forwards the group call request to the gateway MCVideo server of the primary MCVideo system.

4.	The gateway MCVideo server determines which MC system is the group home system for the MCVideo group.

5.	The gateway MCVideo server in the primary MC system of MCVideo client 1 forwards the group call request to the gateway MCVideo server of the group home MC system of the MCVideo group.

6.	The gateway MCVideo server of the group home system of the MCVideo group determines which MCVideo server is the group host MCVideo server for that MCVideo group.

7.	The gateway MCVideo server of the group home system of the MCVideo group forwards the call request to the group host MCVideo server of the MCVideo group.

8.	The group host MCVideo server in the partner MCVideo system checks whether the user of MCVideo client 1 is authorized to initiate the group call. If authorized, it determines the other affiliated members of that group.

9.	The group host MCVideo server sends a call request to MCVideo client 2, which is receiving MCVideo service within the group home MCVideo system.

10.	MCVideo client 2 sends a group call response to the group host MCVideo server, to take part in the group call.

NOTE:	Steps 9 and 10 may each take place at any time before or after steps 11, 12, 13 and 14, provided step 9 takes place before step 10.

11.	The group host MCVideo server sends a group call response to the gateway MCVideo server in the group home MC system of the MCVideo group

12.	The gateway MCVideo server in the group home MC system forwards the group call response to the gateway MCVideo server in the primary MC system of MCVideo client 1.

13.	The gateway MCVideo server in the primary MC system of MCVideo client 1 forwards the group call response to the primary MCVideo server of MCVideo client 1.

14.	The primary MCVideo server of MCVideo client 1 forwards the call setup response to MCVideo client 1.

15.	The media and floor control paths are established to complete the group call setup.

* * * End Changes * * * *