3GPP TSG-SA WG6 Meeting #19                                                                   S6-171179

Dubrovnik, Croatia, 9-13 October 2017                                       (revision of S6-17tttt)

Source:	Motorola Solutions

Title:	Pseudo-CR on addition of new key issue for MC MBMS API: Notification of change of areas of service relevant to MBMS

Spec:	3GPP TR 23.792

Agenda item:	9.2

Document for:	Approval

Contact:	voprescu@motorolasolutions.com

1. Introduction

This pCR describes the key issue of the mission critical UE becoming aware of LTE-predefined area of MBMS service being  in (or moving out of) during operations. This information enables the UE to start/stop monitoring all or some of the MBMS TMGIs, and reporting to the MC service server its current ability to receive (or not) MBMS TMGI(s). The area of service (e.g. MBMS SAIs, PLMN id, cell id) is expected to be provided to the MC service client based on the content of the SIBs blocks advertised within the serving cell.

2. Reason for Change

Section 5 of TR 23.792 defines key issues to drive the design of the API.

3. Proposal

It is proposed to agree to inclusion in the TR 23.792.

## **** First Change ****

## 5.x	Notification of change of areas of service relevant to MBMS

The MBMS bearer announcement messages received by the MC service Client, indicate  for each TMGI a list of service areas identities where the TMGI could be used. The UE needs to be aware constantly whether or not is in such an area, in order to monitor the TMGI, its reception quality, and to report the current status to the MC Service server. Additional information (e.g. is MBMS supported within a certain PLMN) may also be of use to the mission critical entities to determine functionality and make decisions on sending location reports. The MC MBMS API needs to provide notifications to the MC service client, based on the content of the SIBs transmitted in the serving cell, of the current location, as it relates to MBMS functionality.

**** End of Changes ****