3GPP TSG-SA6 Meeting #19	S6-171217

# Dubrovnik, Croatia, 9th Oct 2017 - 13th Oct 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0072 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-server MBMS bearer coordination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MBMS_MCservices |  |  |  |  |  | Date: |  |  | 2017-10-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Modification of the procedure for multi-server MBMS coordination to resolve four editor’s note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are done to the procedure: - The information flows between a MC service server taking the participating role and MC service server taking the controlling role are removed. - The term call is removed and replaced in applicable cases with group communication session. - A new procedural step is added that covers the MBMS bearer announcement and the MBMS listening report aspects. - Some none MBMS related procedural steps are removed (e.g. affiliation). - Information flow tables are updated according to the changes mentioned above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure remains incompletly specified and can not be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.7.2.4, 10.7.2.5, 10.7.2.6, 10.7.2.7, 10.7.2.8, 10.7.2.9, 10.7.2.x (new), 10.7.3.9, 10.7.3.9.1, 10.7.3.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 10.7.2.4	Discover bearer request

Table 10.7.2.4-1 describes the information flow discover bearer request from the MC service server to another MC service server (MBMS bearer control role).

Table 10.7.2.4-1: Discover bearer request

| Information element | Status | Description |
| --- | --- | --- |
| List of service area identifiers | M | A list of service area identifier for the applicable MBMS broadcast area. |
| Bandwidth | M | Maximum bandwidth required |
| QCI | O | Desired QCI |

#### 10.7.2.5	Discover bearer response

Table 10.7.2.5-1 describes the information flow discover bearer response from an MC service server (MBMS bearer control role) to the MC service server.

Table 10.7.2.5-1: Discover bearer response

| Information element | Status | Description |
| --- | --- | --- |
| TMGIs | M | List of TMGIs and related information |
| List of service area identifiers | M | A list of service area identifiers for the applicable MBMS broadcast areas, corresponding to the listed TMGIs, over which the request was successful. |
| Frequency | O | Identification of the frequency if multi-carrier support is provided |
| QCI | O | QCI information used by the ProSe UE-Network relay to determine the ProSe per-packet priority value to be applied for the multicast packets relayed to a remote UE over PC5. |

#### 10.7.2.6	Media distribution request

Table 10.7.2.6-1 describes the information flow media distribution request from the MC service server to an MC service server (MBMS bearer control role) that has a desired bearer.

Table 10.7.2.6-1: Media distribution request

| Information element | Status | Description |
| --- | --- | --- |
| TMGI | M | TMGI information |
| Bandwidth | M | Maximum bandwidth required |
| Separate floor control | M | Whether or not a separate bearer is required for floor control |
| SDP information | M | SDP with media and floor control information applicable to groups that can use this bearer (e.g. codec, protocol id) |
| QCI | O | Desired QCI |

#### 10.7.2.7	Media distribution request response

Table 10.7.2.7-1 describes the information flow media distribution response from an MC service server (MBMS bearer control role) that has a desired bearer to the MC service server.

Table 10.7.2.7-1: Media distribution response

| Information element | Status | Description |
| --- | --- | --- |
| TMGI | M | TMGI information |
| Bandwidth | M | Maximum bandwidth required |
| SDP information | M | SDP with media and floor control information applicable to groups that can use this bearer (e.g. codec, protocol id) |
| QCI | O | Actual QCI |

#### 10.7.2.8	Identify multicast participants request

Void

#### 10.7.2.9	Remove call from bearer request

Void

#### 10.7.2.x	Media distribution release

Table 10.7.2.x-1 describes the information flow media distribution release from the MC service server to an MC service server (MBMS bearer control role) that has the used bearer.

Table 10.7.2.x-1: Media distribution request

| Information element | Status | Description |
| --- | --- | --- |
| TMGI | M | TMGI information |
| Bandwidth | M | The bandwidth released by the MC service server that used the bearer |

***** Next change *****

#### 10.7.3.9	Multi-server bearer coordination

##### 10.7.3.9.1	General

To avoid allocating duplicate bearers for an MBMS service area, a single MC service server may manage all the MBMS media transmission for all groups and users within a particular MBMS service area. An MC service server controlling the MBMS bearer has the MBMS bearer controlling role. Different MC service servers may allocate bearers as needed and make them available for other MC service servers to use.

NOTE:	Within on group communication session, multiple MC service servers (participating role) might be involved. Multiple MBMS service areas might have sufficient MC service group members to warrant multiple MBMS bearers to be used and therefore multiple MC service servers (MBMS bearer controlling role) might be involved. For brevity, this clause only illustrates the simplest case.

##### 10.7.3.9.2	Procedures

NOTE 1:	This procedure is not fully specified in this version of the document.

Pre-conditions:

-	All MC service servers are configured with the contact information of those MC service servers that have, or in the case of dynamic bearer allocation, might make MBMS bearers available for use by other MC service servers.

Figure 10.7.3.9.2-1: Multiple server MBMS procedure

1.	The MC service server 1 evaluates whether multicast is desired for each service area in which MC service group members are located, based upon the locations, affiliation status and other factors.

2.	The MC service server 1 determines whether another MC service server has already established a bearer with coverage for the MBMS service area where multicast is desired. To do this, the MC service server 1 consults a pre-configured list of MC service servers and sends them a discover bearer request. This request may be sent to several MC service servers.

NOTE 2:	MC service servers of the same type can be configured to discover bearers from a single server. The single server then becomes a centralized MBMS bearer controller for the MC service. Similarly, all MC service servers of all types can be configured to discover bearers from a single server. The single server then becomes a centralized MBMS bearer controller for all MC services.

3.	The MC service server 2 (MBMS bearer controlling role) responds with a discover bearer response indicating whether there is an MBMS bearer available in the specific MBMS service area with the requested bandwidth. If the bearer of interest has insufficient bandwidth, the polling MC service server 1 may resort to unicast, or may allocate another bearer for the congested area. If a duplicate bearer is allocated for the same area, the bearer should not be shared with other servers and may be torn down as soon as the congestion on the original bearer clears up, in order to conserve resources.

For any MBMS service areas not covered by another MC service server, the MC service server 1 prepares to distribute media to those MBMS service areas via multicast by setting up a bearer. The bearer set up by the MC service server 1 may then become available for other MC service servers (controlling role) for other MC service groups.

4.	The MC service server 1 performs the MBMS bearer announcement and the MBMS listening reporting according relevant procedures specified in this specification. If the MC service server 2 is authorized to receive MBMS related location information from the users utilizing the services from MC service server 1, the MC service server 2 may optionally do the MBMS bearer announcement and handling the listening reports on behalf of MC service server 1. Listening reports shall in this case be sent to both MC service server 1 and MC service server 2.

5.	The MC service server 1 sends a media distribution request to the MC service server 2 (MBMS bearer controlling role). The media distribution request is sent to reserve the specified capacity in the MBMS bearer.

6.	MC service server 2 (MBMS bearer controlling role) sends a media distribution response to the MC service server 2 (controlling role) indicating whether the request can be supported and supplies details about the bearer.

7.	The MC service server 1 establishes a group communication session via the bearer, informing MBMS connected MC service clients 1 and 2 that a group communication session is about to start on the MBMS bearer. This step is equivalent to MapGroupToBearer in MCPTT.

8.	MC service client 2 sends media on the uplink to the MC service server 1.

9.	The MC service server 1 forwards the media to MC service server 2 (MBMS bearer controlling role).

10.	The MC service server 2 (MBMS bearer controlling role) distributes the media to MBMS served MC service client 1 via multicast.

Editor’s note:	Whether the MC service server 1 can send the media directly on MB2U is FFS.

11.	The MC service server 1 sends a media distribution release request, informing the MC service server 2 (MBMB bearer controlling role) to request the MC service server 2 (MBMS bearer controlling role) to release the capacity that was reserved in step 5.

***** End of changes *****