3GPP TSG-SA WG6 Meeting #26	S6-181448

Vilnius, Lithuania, 15th – 19th October 2018	(revision of S6-181340)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0121 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | There is no file download when using media plane for FD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  |  | Date: |  | 2018-10-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FD using media plane is a real-time transmission; there is no file to upload or download in procedures. The information flows for FD requests using media plane define “Download indication” IE to tell the recipient that s/he needs to download the file mandatory. This term works fine with HTTP base FD as it does need to download the file by the recipient. However, in the FD using media plane case, what it really means is the recipient needs to accept the media plane (MSRP) invite request automatically so the file will be transmitted and received automatically. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the description of the “Download indication” appropriately to reflect the real meaning of this IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 development might be confused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.1.8, 7.5.2.1.12, 7.5.3.2.1, 7.5.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.5.2.1.8	MCData FD request (using media plane)

Table 7.5.2.1.8-1 describes the information flow for the MCData FD request (in subclause 7.5.2.5.2) sent from the MCData client to the MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.8-1: MCData FD request (using media plane)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| MCData ID | M | The identity of the MCData user receiving file |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download. (i.e. auto accept this media plane setup request) |
| SDP offer | M | Media parameters offered |
| Content reference | M | URL reference to the content and file metadata information |

* * * Second Change * * * *

##### 7.5.2.1.12	MCData group standalone FD request (using media plane)

Table 7.5.2.1.12-1 describes the information flow for the MCData group standalone FD request (in subclause 7.5.2.7.2) sent from the MCData client to the MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.12-1: MCData group standalone FD request (using media plane)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download. (i.e. auto accept this media plane setup request) |
| SDP offer | M | Media parameters offered |
| Content reference | M | URL reference to the content and file metadata information |

* * * Third Change * * * *

##### 7.5.3.2.1	MCData FD request (using media plane)

Table 7.5.3.2.1-1 describes the information flow for the MCData FD request sent from the MCData client to another MCData client.

Table 7.5.3.2.1-1: MCData FD request (using media plane)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| MCData ID | M | The identity of the MCData user receiving file |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download. (i.e. auto accept this media plane setup request) |
| SDP offer | M | Media parameters offered |
| Content reference | M | URL reference to the content and file metadata information |

* * * Fourth Change * * * *

##### 7.5.3.2.4	MCData group standalone FD request (using media plane)

Table 7.5.3.2.4-1 describes the information flow for the MCData group standalone FD request sent from the MCData client to another MCData client.

Table 7.5.3.2.4-1: MCData group standalone FD request (using media plane)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download. (i.e. auto accept this media plane setup request) |
| SDP offer | M | Media parameters offered |
| Content reference | M | URL reference to the content and file metadata information |