3GPP TSG-SA WG6 Meeting #27	S6-181706

West Palm Beach, FL, USA, 26th – 30th November 2018	(revision of S6-18xxxx)

Source:	Motorola Solutions

Title:	pCR Solution MCVideo push and pull from server

Spec:	3GPP TR 23.784

Agenda item:	10.4

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

1. Introduction

This pCR proposes a solution for discreet listening of MCVideo push and pull to and from the MCVideo server.

2. Reason for Change

To introduce a solution for the MCVideo aspects of key issue 3.

3. Proposal

It is proposed to agree the changes listed in this pCR to 3GPP TR 23.784

* * * First Change * * * *

## 6.x	Solution x: Discreet listening of MCVideo push and pull

### 6.x.1	Description

#### 6.x.1.1	Overview

This solution addresses the MCVideo aspects of key issue 3 for DiL of MCVideo push and pull.

#### 6.x.1.2	Invocation

Discreet listening is activated by the MC service client of the authorized user according to the invocation procedures described in subclause 6.1.1.3.2 of the present document.

#### 6.x.1.3	MCVideo push discreet listening procedure

The procedure for DiL of a one to server push is shown in figure 6.x.1.3-1 below.

Figure 6.x.1.3-1: MCVideo push to server discreet listening

Steps 1 to 7 are described in 3GPP TS 23.281 [3] subclause 7.4.2.4, the procedure for one to server video push.  Additional steps to support discreet listening are as follow:

11.	The MCVideo server notifies MCVideo client 2 of the authorized user that MCVideo client 1 has been authorized to push video to the MCVideo server. This step may take place at any time following step 2.

12.	The authorized user is notified and decides whether to receive the MCVideo stream, or whether to just remain aware that an MCVideo push is taking place from the target MCVideo user of MCVideo client 1.

13.	MCVideo client 2 provides the response to the MCVideo server, indicating whether the authorized user wishes to receive the video stream or not.

If the authorized user decides to receive the video stream, steps 14 and 15 establish the media plane and transmission control, and transmit the video to the authorized user.  Steps 16 and 17 complete the transaction.

The same procedure can be followed between the MCVideo server and MCVideo client 2 of the authorized user where the MCVideo push is remotely initiated as described in subclause 7.4.2.5 of 3GPP TS 23.281 [3], and where the MCVIdeo push is sent to a group as described in subclause 7.4.2.6 of 3GPP TS 23.281 [3].

#### 6.x.1.4	MCVideo pull discreet listening procedure

The procedure for DiL of a one from server pull is shown in figure 6.x.1.4-1 below.

Figure 6.x.1.4-1: MCVideo pull from server discreet listening

Steps 1 to 7 are described in 3GPP TS 23.281 [3] subclause 7.3.2.4, the procedure for one from server video pull.  Additional steps to support discreet listening are as follow:

11.	The MCVideo server notifies MCVideo client 2 of the authorized user that MCVideo client 1 has been authorized to pull video from the MCVideo server. This step may take place at any time following step 2.

12.	The authorized user is notified and decides whether to receive the MCVideo stream, or whether to just remain aware that an MCVideo pull is taking place to the target MCVideo user of MCVideo client 1.

13.	MCVideo client 2 provides the response to the MCVideo server, indicating whether the authorized user wishes to receive the video stream or not.

If the authorized user decides to receive the video stream, steps 14 and 15 establish the media plane and transmission control, and transmit the video to the authorized user.  Steps 16 and 17 complete the transaction.

#### 6.x.1.5	Information flows for discreet listening of MCVideo push and pull

A single pair of information flows can support the request and response for discreet listening of MCVideo push and pull video transfers. Suitable information elements are shown in tables 6.x.1.5-1 and 6.x.1.5-2 below.

Table 6.x.1.5-1: MCVideo push-pull DiL request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | MCVideo ID of the target of DL |
| MCVideo ID | O | MCVideo ID of third party requesting remote MCVideo push, if applicable |
| Push/pull | M | Indicates whether the video is being pushed to or pulled from the target MCVideo client |
| MC service group ID | O | Provided if the communication is a push to an MCVideo push |

Table 6.x.1.5-2: MCVideo push-pull DiL response

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | MCVideo ID of the target of DL |
| Video stream reception | M | Indicates whether the authorized user wishes to receive the video stream |

The information elements for MCVideo push-pull DiL complete request and response are derived from those employed in steps 6 and 7 of the MCVideo push and pull procedures described in 3GPP TS 23.281 [3], and are illustrated in tables 6.x.1.5-3 and 6.x.1.5-4 below.

Table 6.x.1.5-3: MCVideo push-pull DiL complete request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | MCVideo ID of the target of DL |
| Cause | M | End of communication cause |

Table 6.x.1.5-4: MCVideo push-pull DiL complete response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | MC service ID of the target of DL |

### 6.x.2	Impacts on existing nodes and functionality

This solution results in the following impacts on existing entities and reference points:

-	MC service clients and MC service-1 reference points: addition of functionality to offer and accept DiL MCVideo push and pull content.

-	New procedures to provide DiL of MCVideo push and pull transactions.

### 6.x.3	Solution Evaluation

This solution allows discreet listening of MCVideo push and pull transactions to be achieved with minimal changes to existing MCVideo functionality, as the provision of DiL is based on existing MCVideo push and pull procedures.