3GPP TSG-SA WG6 Meeting #28	S6-190228

Kochi, India, 21st – 25th January 2019	(revision of S6-190124)

Source:	Motorola Solutions

Title:	pCR Overall evaluation

Spec:	3GPP 23.784

Agenda item:	10.4

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

1. Introduction

This pCR presents an overall evaluation to capture the evaluations of each indivual solution, and to indicate a mapping of key issues to solutions.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.784 v0.4.0.

* * * First Change * * * *

# 7	Overall Evaluation

## 7.1	General

The following subclauses contain an overall evalation of the solutions presented in this technical report, and their applicability to the key issues raised. Subclause 7.2 provides an evaluation of the solutions provided in clause 6, and their impact on other entities and working groups, and shows the mapping of the solutions to the key issues outlined in clause 5. Subclause 7.3 lists the key issues that do not have explicit solutions in this Technical Report, together with comments concerning their resolution. Subclause 7.4 lists the key issues with security implications that will need consideration in 3GPP SA WG3.

Editor's note: the response from SA1 on requirements for discreet listening of groups may cause additional solutions and evaluation.

## 7.2	Solution evaluation

Table 7.2-1 below shows the mapping of key issues to solutions.

Table 7.2-1: Mapping of key issues to solutions

| Key issue | Solution |
| --- | --- |
| 1: Selection of MC service for discreet listening | 1: Functionality for discreet listening |
| 2: Group membership of authorized MC user for discreet listening | 1: Functionality for discreet listening |
| 3: Discreet listening for MCVideo push and pull and MCData file upload and download | 2: Discreet listening of MCVideo push and pull |
| 4: Discreet listening of end to end encrypted calls | Out of scope (see note) |
| 5: Logging and replay of end to end encrypted calls | Out of scope (see note) |
| 6: Source MC service server for discreet listening of user in group call | 1: Functionality for discreet listening |
| 7: Discreet listening and logging for interconnected MC system |  |
| 8: Discreet listening and logging for migrated MC service user |  |
| x: Discreet listening and logging for user receiving MC service on multiple devices |  |
| NOTE: Security solutions are within the remit of SA3 |  |

The solutions specified in this technical report are described in table 7.2-2, together with the evaluation text for each solution.

Table 7.2-2: Solution evaluations

| Solution | Applicable key issues | Evaluation | Impact on other entities and working groups |
| --- | --- | --- | --- |
| Solution 1: Functionality for discreet listening | 1: Selection of MC service for discreet listening 2: Group membership of authorized MC user for discreet listening 6: Source MC service server for discreet listening of user in group call | This solution allows discreet listening to be achieved with minimal changes to the functional model as it only requires enhancement to existing functional entities and reference points. It also allows discreet listening to be invoked on a service dependent basis as it is achieved as a function of the MC service specific MC service clients. | Minor impact on downstream groups to add functionality to existing functional entities and reference points. |
| Solution 2: Discreet listening of MCVideo push and pull | 3: Discreet listening for MCVideo push and pull and MCData file upload and download | This solution allows discreet listening of MCVideo push and pull transactions to be achieved with minimal changes to existing MCVideo functionality, as the provision of discreet listening is based on existing MCVideo push and pull procedures. Additional configuration parameters are likely to be needed to support the service. | Minor impact on downstream groups to add functionality to existing functional entities and reference points. |
| Solution x: Functionality for logging and replay |  | This solution defines new functional entities and reference points to fulfil the logging requirement together with reuse of existing common services core functional entities and reference points. The new functional entities and reference points will be based on the functionality of existing MC service clients and associated reference points. A set of procedures will need to be defined to show the invocation of logging functions on logged calls. | Impact on downstream groups to add new functionality, based on existing call related functionality. |

## 7.3	Key issues with no associated solutions

Table 7.3-1 lists the key issues described in this technical report that have no specific solutions, together with comments concerning their resolution.

Editor's note: This subclause will be removed if there are no key issues without solutions when the TR is concluded.

Table 7.3-1: Key issues with no associated solutions

| Key issue | Comment |
| --- | --- |

## 7.4	Key issues with security implications

The following key issues have security considerations that will need review and possible solutions by SA3:

-	Key issue 4: Discreet listening of end to end encrypted calls; and

-	Key issue 5: Logging and replay of end to end encrypted calls.