3GPP TSG-SA WG6 Meeting #29	S6-190344

Montréal, Canada, 25th February – 1st March 2019	(revision of S6-19xxxx)

Source:	Motorola Solutions

Title:	Interconnection discussion for MCData

Agenda Item:	9.4

Contact:	david.chater-lea@motorolasolutions.com

Discussion of unfulfilled interconnection functions for MCData

### Introduction

3GPP TS 23.282 CR130 made changes to the MCData user profile to permit migration to take place.  However, there are still unresolved items to permit interconnection to take place:

-	The functional model needs the addition of the MC gateway server to permit interconnection with topology hiding, as has been done for MCPTT and MCVideo. CR0132 is brought forward together with this discussion paper to make this change to the functional model.

-	Interconnection for services between two MCData clients via an MCData server, or between an MCData client and an MCData server will be straightforward, and will follow the generic procedures for interconnection with topology hiding in 3GPP TS 23.280 subclause 10.14.

-	Interconnection where an MCData client accesses a message store or a content server in a partner MC system, e.g. when the MCData client has migrated to another system, is the subject of further discussion in this paper.

### Interconnection, content servers and message stores

It is possible that an MCData user will need to access a content server in another MC system. It is FFS whether an MCData user will need to access a message store in another MC system when receiving service in the primary MCData system of the MCData user, however during migration, it can be expected that a migrated MCData user will need to access from the partner MCdata system to the message store that the MCData user has access to in the primary MCData system.

Topology hiding will need to be in place between different MC systems in different trust domains.  The architecture for interconnection for MCData services needs to permit this. This is illustrated in figure 1.

Figure 1: Interconnection with topology hiding

The MC gateway server provides topology hiding between MC service servers in different trust domains for application plane and SIP signalling plane protocols. HTTP signalling plane protocols use HTTP proxy functions in each MC system. The MC gateway server role is in a server to server environment, and is not expected to provide topology hiding for a client to server interaction. Figure 2 illustrates this non permissible architecture.

Figure 2: Non permissible architecture for content server and message store interconnection

This therefore leaves two possibilities: the MCData client relies on MCData server to server reference points to access the message store and content server in another MC system as shown in Figure 3 below, or there need to be some additional reference points between content servers and message stores to satisfy the need for interconnection as illustrated in figure 4 below.

Figure 3: Reference points if MCData servers provide interconnection to message store and content server in another MC system

In figure 3, the functionality of existing reference points between MCData client and MCData server, and between MCData server and message store and content server is expanded as necessary to permit the interconnected functions.

Figure 4: Reference points to permit message stores and content servers provide interconnection directly

In figure 4, R1 is a new reference point between content server and MC gateway server, and R2 is a new reference point between gateway servers which enables R1. R3 is a new reference point between message store and MC gateway server, and R2 is a new reference point between gateway servers which enables R3.

A final architectural option would be for the MCData client to interface with the message store and content server in an  interconnection environment by using the HTTP aspect of the signalling plane, and rely on HTTP proxies to provide topology hiding. This is illustrated in figure 5 below. This would not be able to make use of the application plane controlled bearer priorities, and would rely on the prioritisation used by the signalling plane.

Figure 5: Use of signalling plane HTTP connection to enable interconnection with message store and content server

### Discreet listening

Discreet listening provides an additional consideration, as the authorized user will expect to be able to monitor communications between the MCData user and the message store or content server. If the discreet listening service relies on the authorized user communicating with the MCData server, the architecture shown in figure 3 appears to be the simplest to implement. However an alternative could be to allow the authorized user direct connections to the message store and content server, and have an architecture similar to that shown in figure 5.

### Discussion

The architecture shown in figure 3 requires the least changes to the existing functional models to implement interconnection where connection to a message store or content server in another MC system are required. This is also the simplest architecture to permit discreet listening. It is recommended that this approach is taken forward for interconnection for MCData.