3GPP TSG-SA WG6 Meeting #29	S6-190347

Montréal, Canada, 25th February – 1st March 2019	(revision of S6-19xxxx)

Source:	Motorola Solutions

Title:	Key issue for discreet listening of MCData services

Spec:	3GPP TR 23.784 v050

Agenda item:	10.4

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

1. Introduction

This pCR introduces a key issue for discreet listening of MCData services where a content server or message store is used.

2. Reason for Change

To add a key issue for related to MCData where a content server or message store is used.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.784 v050.

* * * First Change * * * *

## 5.z	Key Issue z:	Discreet listening of MCData content server and message store services

An MCData user may make use of a content server for upload and download of MCData content. An MCData user may also have the use of a message store where content of MCData transactions can be stored for later retrieval. An authorized user may require discreet listening of transactions between the MCData user and the the content server or message store. If provided, the authorized user may need access to the content of information exchanged, or may require notification that the information is being exchanged without needing the content.

Issues:

-	Whether discreet listening of transactions to and from an MCData content server is required.

-	Whether discreet listening of transactions to and from a message store is required.

-	Whether discreet listening requires notification that transactions are taking place without providing the content, or whether the content itself is also needed.

-	Whether the content of transactions needs to follow the same path as notification of the transactions, if both notification and content are required, or whether content can be accessible by the authorized user directly from the message store or content server.