3GPP TSG-SA WG6 Meeting #29	S6-190470

Montréal, Canada, 25th February – 1st March 2019	(revision of S6-190348)

Source:	Motorola Solutions

Title:	Group management for logging solution 3

Spec:	3GPP TR 23.784 v050

Agenda item:	10.4

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

1. Introduction

Solution 3, 'Functionality for on-network logging and replay', did not include a group management client or group management server, as it was considered that the group configuration was irrelevant to a logging equipment, which simply is provided for communications information for any group call that the logger is configured to log. However, the following configuration parameters may assist the development of a logging solution:

- 	Participant type for the group in the group member list: it could aid the MC service server and possibly the logging equipment to know that the logging equipment is defined as such

-	Media and floor control confidentiality and integrity protection; whereas the logging equipment will probably not be able to decrypt the media, it will need to decrypt the floor control signalling

-	Support for multi talker control, as the logging equipment may need to warned that it may receive multiple simultaneous media streams

-	Codec type(s), at least for the purpose of identifying storage requirements.

There may be others as well, although most group configuration parameters are not relevant to logging equipment. Therefore this pCR adds the functional entities for group managment into the solution.

2. Reason for Change

To add group configuration entities and reference points into solution 3.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.784 v050.

* * * First Change * * * *

## 6.3	Solution 3:	Functionality for on-network logging and replay

### 6.3.1	Description

#### 6.3.1.1	Functional model

To support on-network logging and replay, existing common service core functional entities are reused. The MC logging function makes use of a new MC logging client which has a subset of the functionality of an existing MC service client, in that it can receive communications information and communications content relating to logged users, but cannot set up calls or demand to take part in calls. Provision of call related information and call content is achieved by configuration in the MC service server. The MC logging client uses an MC service ID to allow call related information and call content to be routed to the MC logging client, reusing functionality from existing reference points.

NOTE 1:	An MC logging function may contain multiple MC service clients to allow the separate services MCPTT, MCVideo and MCData to be logged within the same function. However this is out of scope of the description of the functional model.

Figure 6.3.1.1-1 below illustrates the functional entities and reference points of the common services core in the application plane that are used for logging.

Figure 6.3.1.1-1: Common services core functional model for logging

In Figure 6.3.1.1-1, the MC logging function is configured with the list of MC service IDs and MC service groups that the logging function will log using reference point CSC-4. The group management client receives configuration for groups that are to be logged. The identity management client allows the logging function to be authorized for logging service using reference point CSC-1, using either security credentials supplied by a user or supplied by the logging application. The key management client obtains keys using reference point CSC-8 to protect signalling information between MC service server and MC service client. Keys may be obtained to protect media if the solution is intended to log communications with any end to end encryption removed.

NOTE 2:	The means of protecting media and communications related information during storage within the MC logging function is outside the scope of this solution.

The MC replay function receives the necessary security information needed to replay logged communications by CSC-8. A user of the replay function, or an automatic function of the replay function, will perform the necessary authorizations to retrieve logged communications, and CSC-1 will be used to verify the authenticity and authorization of the user or function.

NOTE 3:	Inclusion of the mechanisms for MC service user authentication and authorization for use by the replay function within this solution allows the user organization to require the same security processes to access logged information as the security processes that are used to communicate within the MC service. Alternative solutions are possible, but not considered within this solution.

The MC logging function and MC replay function are in the same security domain as the MC system for which communications are logged.

The interface MCLog-10 between the logging recorder and the replay client to retrieve communications is out of scope of this solution. MCLog-10 does not need to use 3GPP access for connectivity. Information relating to the user of the logging replay function and any profile for this replay function user are out of scope of this solution.

Figure 6.3.1.1-2 below shows the MC service related application plane entities used to log communications.

Figure 6.3.1.1-2: MC service related application plane functional model for logging

In figure 6.3.1.1-2, the MC logging function may comprise MC logging clients dedicated to MCPTT, MCVideo and MCData. Each MC logging client is identified by a service specific identity, namely the MCPTT ID, MCVideo ID or MCData ID of the client. Call related information concerning the targets of logging are sent to the MC logging function via MClog-1, which is based on the MCservice-1 reference points (i.e. MCPTT-1, MCVideo-1 etc) but does not permit calls to be initiated by the MC logging function. Media plane communications content is provided to the media reception function of the MC logging client via MCLog-2, which is based on unicast media distribution reference points MC service-7 (i.e. MCPTT-7, MCVideo-7 etc), and floor control or media control information provided during calls is provided via MCLog-3.

Editor's note:	MCData services may require further functionality to be described.