3GPP TSG-SA WG6 Meeting #30	S6-190564

Newport Beach, CA, USA, 8th – 12th April 2019	(revision of S6-19xxxx)

Source:	Motorola Solutions

Title:	Update requirements annex

Spec:	3GPP TR 23.784 v100

Agenda item:	10.3

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

1. Introduction

This pCR aligns the reproduced requirements in Annex A with updated SA1 requirements for discreet listening in 3GPP TS 22.280.

2. Reason for Change

Following the update to requirements for discreet listening in 3GPP TS 22.280 v16.5.0, presented to SA6 #29 in S6-190431 as CR107 to 3GPP TS 22.280 v16.4.0, the discreet listening requirements reproduced in Annex A need to be updated with the new and changed requirements.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.784 v100.

* * * First Change * * * *

## A.1	Requirements for discreet listening

Requirements listed in this subclause are reproduced from the specifications referenced within this subclause. The referenced specifications are the normative source for the requirements and have precedence over the information reproduced within this subclause.

The requirements in 3GPP TS 22.280 [2] that apply for discreet listening are provided in table A.1-1. They are applied to all services MCPTT, MCVideo and MCData according to the reference tables in Annexes A, B and C of [2].

Table A.1-1: Discreet listening requirement

| Requirement | Description |
| --- | --- |
| [R-6.15.1-001a] | The MCX Service shall provide discreet listening capabilities without noticeable impact on or knowledge of the target MCX User, or the members of the target MCX Service Group, and all other unauthorized MCX Users. |
| [R-6.15.1-001] | The MCX Service shall provide a mechanism for an authorized MCX User to receive MCX Service Private Communication transmissions to and from a specific target MCX User that is within the authority of the authorized MCX User. |
| [R-6.15.1-002] | The MCX Service shall provide a mechanism for an authorized MCX User to receive MCX Service Group Communication transmissions to and from a specific target MCX User that is within the authority of the authorized MCX User. |
| [R-6.15.1-003] | Subject to regulatory constraints and operator security policies, the MCX Service shall allow to be configured not to provide transmissions from MCX Users who are communicating with the discreet listening target MCX User, and who are not themselves targets of discreet listening. |
| [R-6.15.1-004] | The MCX Service shall provide a mechanism for an authorized MCX User to receive MCX Service Group transmissions from a specific MCX Service Group that is within the authority of the authorized MCX User. |
| NOTE: Permission to activate discreet listening can include multiple levels of authorization, but this is outside the scope of 3GPP. |  |