3GPP TSG-SA WG6 Meeting #35	S6-200xxx

# Hyderabad, India, 13th - 17th Jan 2020											(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 23.379 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancing MCPTT communication requests with application priority capabilities in on-network mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2020-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP TS 22.280 requirement [R-6.8.7.2-001] asks to apply application priorities to each communication. This is only partially done in MCPTT service 3GPP TS 23.379 at the moment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR adds the missing requested priority to these MCPTT flows which does not encompass the application priority. In detail following will be addressed: - Emergency as well as imminent peril communication can also use the requested priority in the future, but only the available indicator or the requested priority can be used. - In order to prioritize the emergency alert request as desired in the 3GPP transport system, this can be determined by the request. - In some cases, the missing requested priority is updated in some server to server flows in order to ensure the application priority also in the target server and its 3GPP system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing capabilities in the MCPTT service context. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.2.1, 10.6.2.2.3, 10.6.2.2.5, 10.6.2.2.8, 10.6.2.2.34, 10.6.2.2.36, 10.7.2.1.2, 10.7.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********************************************1st Change********************************************

##### 10.6.2.2.1	MCPTT emergency group call request

Table 10.6.2.2.1-1 describes the information flow MCPTT emergency group call request from the MCPTT client to the MCPTT server and from the MCPTT server to the MCPTT client.

Table 10.6.2.2.1-1 MCPTT emergency group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID on which the call is to be conducted |
| Emergency indicator (NOTE 2) | O | Indicates that the group call request is an MCPTT emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Implicit floor request (NOTE 1) | O | Indicates that the originating client requests the floor. |
| Requested priority (NOTE 2) | O | Priority level requested for the call. |
| Location | O | Location of the calling party |
| NOTE 1: This element shall be included only when this information flow is from the client to the server and the originating client requests the floor. NOTE 2: Either the emergency indicator or the requested priority is present. |  |  |

********************************************2nd Change*******************************************

##### 10.6.2.2.3	MCPTT emergency alert request

Table 10.6.2.2.3-1 describes the information flow MCPTT emergency alert request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.3-1 MCPTT emergency alert request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the alerting party |
| Functional alias | O | The functional alias of the alerting party |
| MCPTT group ID | M | The MCPTT group ID with which the alert is associated |
| Requested priority | O | Priority level requested for the emergency alert indication. |
| Location | O | The alerting MCPTT client's location |

********************************************3rd Change*******************************************

##### 10.6.2.2.5	MCPTT imminent peril group call request

Table 10.6.2.2.5-1 describes the information flow MCPTT imminent peril group call request from the MCPTT client to the MCPTT server and from the MCPTT server to the MCPTT client.

Table 10.6.2.2.5-1 MCPTT imminent peril group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID on which the call is to be conducted |
| Imminent peril indicator (NOTE 2) | O | Indicates that the group call request is an imminent peril call |
| Implicit floor request (NOTE 1) | O | Indicates that the originating client requests the floor |
| Requested priority (NOTE 2) | O | Priority level requested for the call. |
| Location | O | Location of the calling party |
| NOTE 1: This element shall be included only when this information flow is from the client to the server and the originating client requests the floor. NOTE 2: Either the imminent peril indicator or the requested priority is present. |  |  |

********************************************4th Change*******************************************

##### 10.6.2.2.8	Group call request (MCPTT server – MCPTT server)

Table 10.6.2.2.8-1 describes the information flow group call request between the MCPTT servers.

Table 10.6.2.2.8-1 Group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is initiated |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |
| Implicit floor request (NOTE) | O | Indicates that the originating client requests the floor. |
| Requested priority | O | Priority level requested for the call. |
| Location information | O | Location of the calling party |
| NOTE: This element shall be included only when the originating client requests the floor. |  |  |

********************************************5th Change*******************************************

##### 10.6.2.2.34	Preconfigured regroup request (MCPTT client – MCPTT server)

Table 10.6.2.2.34-1 describes the information flow preconfigured regroup request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.34-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| MCPTT group ID | M | MCPTT group ID of the MCPTT group from which configuration is to be taken |
| MCPTT group ID list | O (see NOTE) | List of MCPTT groups to be regrouped into the preconfigured group regroup group |
| MCPTT user ID list | O (see NOTE) | List of MCPTT users to be regrouped into the preconfigured user regroup group |
| Requested priority | O | Priority level requested for the call. |
| NOTE: One and only one of these shall be present. |  |  |

********************************************6th Change*******************************************

##### 10.6.2.2.36	Preconfigured regroup request (MCPTT server – MCPTT server)

Table 10.6.2.2.36-1 describes the information flow preconfigured regroup request from the MCPTT server to the MCPTT server.

Table 10.6.2.2.36-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| MCPTT group ID | M | MCPTT group ID of the MCPTT group from which configuration is to be taken |
| MCPTT group ID list | O (see NOTE) | List of MCPTT groups to be regrouped into the preconfigured regroup group |
| MCPTT user ID list | O (see NOTE) | List of MCPTT users to be regrouped into the preconfigured user regroup group |
| Requested priority | O | Priority level requested for the call. |
| NOTE: One and only one of these shall be present. |  |  |

********************************************7th Change*******************************************

##### 10.7.2.1.2	MCPTT private call request (MCPTT server to MCPTT server)

Table 10.7.2.1.2-1 describes the information flow MCPTT private call request from the MCPTT server to the MCPTT server.

Table 10.7.2.1.2-1: MCPTT private call request (MCPTT server to MCPTT server) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID | M | The MCPTT ID of the called party |
| Functional alias | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | M | Media parameters of MCPTT client. |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| Requested priority | O | Priority level requested for the call. |
| Location information | O | Location of the calling party |

********************************************8th Change*******************************************

##### 10.7.2.1.5	MCPTT emergency private call request (MCPTT client to MCPTT server)

Table 10.7.2.1.5-1 describes the information flow MCPTT emergency private call request from the MCPTT client to the MCPTT server.

Table 10.7.2.1.5-1: MCPTT emergency private call request (MCPTT client to MCPTT server) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID (NOTE) | O | The MCPTT ID of the called party |
| Functional alias (NOTE 1) | O | The functional alias of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCPTT emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| SDP offer | M | Media parameters of MCPTT client. |
| Requested priority (NOTE 2) | O | Priority level requested for the call. |
| Location information | O | Location of the calling party |
| NOTE 1: Only one identity shall be present. NOTE 2: Either the emergency indicator or the requested priority is present. |  |  |

*******************************************End of Changes****************************************