3GPP TSG-SA WG6 Meeting #35	S6-200151

# Hyderabad, India, 13th - 17th Jan 2020											(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 204 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | Y | Radio Access Network |  | Core Network | Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Local policies at Partner MCData system is not applied |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsug Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2020-01-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The NOTE 6 of subclause 7.5.2.4.3 describes the partner MCData content server may store the local copy of the file in case future requests arises in the partner MCData system for interconnected MCData systems. If the local policies in the partner MCData system are applied to the file received from the primary MCData system and expiry time dictated by local policy in partner system is shorter than the expiry time sent from the primary MCData system, then the intended availability of file is not met and can result in early expiry of file availability timer. The participant(s) at partner system will not be able to download the file even though the file availability at the primary MCData system is still valid. - The defination/description of ‘Nominated’ expiry time is missing and to our understanding, ‘Nominated’ expiry time is same as ‘expiry time sent from the primary MCData system’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The usage of local policy is removed at MCData partner system. - The sentence ‘nominated expiry time’ is replaced with ‘expiry time sent from the primary MCData system’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The system will become more complicated and difficult in implimentation based on the policies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.5.2.4.3	Procedure with interconnection between MCData systems

The procedure in figure 7.5.2.4.3-1 describes the case where a MCData user initiates a one-to-one data communication for sending a file to another MCData user where that other MCData user is receiving MCData service on a partner MCData system, and where interconnection is in use between the two MCData systems. In this procedure, the file has not previously been downloaded in the partner MC system.

Pre-conditions:

1.	The MCData users on the MCData client 1 and the MCData client 2 are already service authorized and receiving MCData service. MCData client 1 is receiving service on its primary MCData system, and MCData client 2 is receiving MCData service in the partner MCData system of MCData client 1.

2.	The file to be distributed has been uploaded to the media storage function on the MCData content server in the primary MCData system of MCData client 1 using the procedures defined in subclause 7.5.2.2.

3.	There is a service agreement between the primary and partner MCData systems to allow files to be shared between MCData content servers in the two systems.

Figure 7.5.2.4.3-1: One-to-one file distribution using HTTP with interconnection

1.	The user at the MCData client 1 initiates a file distribution request to the MCData user at MCData client 2.

2.	MCData client 1 sends an MCData FD request towards the primary MCData server. The MCData FD request contains content payload in the form of a file URL with the necessary access authorization information and may contain the file metadata information. The MCData FD request indicates the target MCData user for the one-to-one data communication. The MCData FD request contains a conversation identifier for message thread indication. If the MCData user at MCData client 1 has requested to mandatory download at the recipient side, then the MCData FD request contains the mandatory download indication. The MCData FD request may contain a request for a download completed report indication if selected by the user at MCData client 1.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send the MCData FD request and that the size of the file is below maximum data size for FD from the service configuration.

4.	The MCData server in the primary MCData system initiates the MCData FD request towards the MCData server in the partner MCData system, which contains the URL of the file which is stored in the primary MCData content server. The request includes the necessary access authorization information as MCData client 2 will retrieve the file while receiving service in the partner MCData system.

NOTE 1:	The contents of and mechanisms to use the authorization information are outside the scope of the present document.

5.	The partner MCData server sends the MCData FD request to MCData client 2.

6.	The receiving MCData client 2 may notify the user about the incoming MCData FD request (including file metadata, if present) which may be either accepted, rejected or ignored.

7.	MCData user 2 may provide a response (accept or reject) or not (ignore) to the notification, then MCData client 2 sends the MCData FD response to the partner MCData server. MCData client 2 automatically sends an accepted MCData FD response when the incoming request includes mandatory download indication.

8.	The partner MCData server forwards the MCData FD response to the MCData server in the primary MCData system.

9.	The primary MCData server forwards the MCData FD response to MCData client 1.

10.	MCData client 2 requests the file from the partner MCData content server. The URL provided in step 5 is prepended with server URI of the partner MCData content server, such that the URL identifies a file location in the partner MCData content server.

NOTE 2:	Step 10 may occur any time after step 7, before or after steps 8 and 9.

11.	The partner MCData content server checks whether the file is stored locally, and if this is not the case, sends an MCData file retrieve request to the primary MCData content server.  The MCData file retrieve request contains the URL of the file location in the primary MCData system, generated by removing the prepended local path from the requested URL.

NOTE 3:	The means of proving authorization for the request is outside the scope of the present document.

12.	The primary MCData content server responds to the partner MCData content server with an MCData file retrieve response which contains the content of the file to be retrieved. File metadata may include the lifetime of the file. The primary MCData content server records that the file has been sent to the indicated partner MCData system.

NOTE 4:	The partner MCData content server may store the local copy of the file in case future requests arise until the expiry time sent from the primary MCData system for the file is reached or until a request is received to delete the file.

13.	The partner MCData content server sends the file to MCData client 2 in the MCData download data response. MCData client 2 records file download completed and notifies MCData user 2.

14.	MCData client 2 initiates an MCData download complete report for reporting file download completed, if this was requested by the user at MCData client 1 in the initial MCData FD request.

15.	The MCData download complete report is sent to the primary MCData server. The partner MCData server may store the download complete report for download history interrogation from authorized MCData users in the partner MCData system.

16.	The MCData download completed report is sent by the primary MCData server to the MCData user at MCData client 1. The MCData file download completed report from MCData user may be stored by the primary MCData server for download history interrogation from authorized MCData users in the primary MCData system.