3GPP TSG-SA WG6 Meeting #35	S6-200178

# Hyderabad, India, 13th - 17th Jan 2020											(revision of S6-200024)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0136 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancing MCVideo communication requests with application priority capabilities in on-network mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2020-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP TS 22.280 requirement [R-6.8.7.2-001] asks to apply application priorities to each communication. This is only partially done in MCVideo service 3GPP TS 23.281 at the moment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR adds the missing requested priority to these MCVideo flows which does not encompass the application priority. In detail following will be addressed: - Emergency as well as imminent peril communication can also use the requested priority in the future, but only the available indicator or the requested priority can be used. - In order to prioritize the emergency alert request as desired in the 3GPP transport system, this can be determined by the request. - In some cases, the missing requested priority is updated in some server to server flows in order to ensure the application priority also in the target server and its 3GPP system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing capabilities in the MCVideo service context. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2.8, 7.1.2.2.14, 7.1.2.2.18, 7.1.2.2.22, 7.2.2.2.6; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-200024 |  |  |  |  |  |  |  |  |

********************************************1st Change********************************************

##### 7.1.2.2.8	Group call rejoin request (MCVideo client – MCVideo server)

Table 7.1.2.2.8-1 describes the information flow group call rejoin request from the MCVideo client to the MCVideo server.

Table 7.1.2.2.8-1: Group call rejoin request (MCVideo client – MCVideo server))

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the re-joining MCVideo group member |
| Functional alias | O | The functional alias of the re-joining MCVideo group member |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is on-going |
| SDP offer | M | Media parameters of MCVideo client |

********************************************2nd Change*******************************************

##### 7.1.2.2.14	MCVideo emergency alert request

Table 7.1.2.2.14-1 describes the information flow MCVideo emergency alert request from the MCVideo client to the MCVideo server, for the procedures defined in 3GPP TS 23.280 [6].

Table 7.1.2.2.14-1: MCVideo emergency alert request (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the alerting party |
| Functional alias | O | The functional alias of the alerting party |
| MCVideo group ID | M | The MCVideo group ID with which the alert is associated |
| Requested priority | O | Priority level requested for the emergency alert indication. |
| Location | O | The alerting MCVideo client's location |

********************************************3rd Change*******************************************

##### 7.1.2.2.18	MCVideo emergency group call request

Table 7.1.2.2.18-1 describes the information flow emergency group call request from the MCVideo client to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.18-1: MCVideo emergency group call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Emergency indicator | M | Indicates that the group call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested priority (NOTE 2) | O | Priority level requested for the call. |
| Implicit transmit media request (see NOTE 1) | O | Indicates that the originating client requests the permission to transmit media. |
| NOTE 1: This element shall be included only when the originating client requests the the permission to transmit media. NOTE 2: The use of the requested priority depends on operator’s policy. |  |  |

********************************************4th Change*******************************************

##### 7.1.2.2.22	MCVideo imminent peril group call request

Table 7.1.2.2.22-1 describes the information flow MCVideo imminent peril group call request from the MCVideo client to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.22-1: MCVideo imminent peril group call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Requested priority (NOTE) | O | Priority level requested for the call. |
| Imminent peril indicator | M | Indicates that the group call request is an imminent peril call |
| NOTE: The use of the requested priority depends on operator’s policy. |  |  |

********************************************5th Change*******************************************

##### 7.2.2.2.6	MCVideo emergency private call request (MCVideo client to MCVideo server)

Table 7.2.2.2.6-1 describes the information flow MCVideo emergency private call request from the MCVideo client to the MCVideo server.

Table 7.2.2.2.6-1: MCVideo emergency private call request (MCVideo client to MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit transmit media request | O | An indication that the user is also requesting to transmit media |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested priority (NOTE) | O | Priority level requested for the call. |
| NOTE: The use of the requested priority depends on operator’s policy. |  |  |

*******************************************End of Changes****************************************