3GPP TSG-SA WG6 Meeting #35	S6-200199

# Hyderabad, India, 13th - 17th Jan 2020											(revision of S6-200126)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0232 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement to group creation at GMS using preconfigured group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2020-01-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) ) Rel-17 (Release 17) |  |
| Reason for change: |  | The creation of user regroup may take extra time for GMS to create a totoally new temporary user regrouped group profile, the group notification may also need to convey the newly temporary user regroup group profile to UE. This CR introduces a enhancement to the user regroup procedure, which the users are regrouped with the precondigured group profile, which the newly temporary user regrouped group profile is not required to be distributed to the users being regrouped. However, the procedure in 10.2.3 Group creation is applicable for both scenarios of normal group creation by an MC service administrator and user regrouping operations by authorized user/dispatcher, which should be splited into two separate procedures, i.e., one for normal group creation, and the other one for user regrouping operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the scentence in 10.2.3 “It applies to the scenario of normal group creation by an MC service administrator and user regrouping operations by authorized user/dispatcher.” to “It applies to the scenario of normal group creation by an MC service administrator” - Add the user grouping procedure derived from existing 10.2.3 with modifications to reflect it is applicable for user grouping operation including: 2.1) add a MC service ID of the preconfigured group whose configuration data will be taken for the user regrouped group in the user regroup notification 2.2) in the case of using the preconfigured group for user regrouping, the group management clients being regrouped will not further pull the group profile from the GMS. 2.3) update the pre-conditions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Traffic load might be high. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2.7A (new), 10.2.2.a (new), 10.2.2.b (new), 10.2.2.c (new), 10.2.2.d (new), 10.2.3, 10.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * * *

10.2.2.7A	Group creation notification

Table 10.2.2.7A-1 describes the information flow group creation notification from the group management server to the group management client.

Table 10.2.2.7A-1: Group regroup notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service group ID | M | MC service group ID of the MC service group being created |
| MC service ID list | M | List of MC service IDs that are enabled of the created group |

* * * First Change * * * *

10.2.2.a	User regroup request

Table 10.2.2.a-1 describes the information flow user regroup request from the group management client to the group management server.

Table 10.2.2.a-1: User regroup request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list | M | List of MC service IDs that are part of the group to be created corresponding to the list of the configured services (see NOTE 1) |
| MC service list (see NOTE 2) | O | List of MC services whose service communications are to be enabled on the group. |
| Proposed MC service group ID | O | MC service group ID proposed for the requested group |
| MC service group configuration data | O (NOTE 4) | Configuration data for the requested group (see NOTE 3) |
| MC service group ID | O (NOTE 4) | MC service group ID of the MC service group from which configuration is to be taken for user regroup operation |
| NOTE 1: The MC service ID list can be an empty list. NOTE 2: This information element shall be included in the message for creating a group configured for multiple MC services. NOTE 3: If the MC service group configuration data is not present it can be provided later using the "Store group configuration request" (see subclause 10.1.2.1). NOTE 4: Either MC service group configuration data information element or MC service group ID information element is present. |  |  |

10.2.2.b	User regroup response

Table 10.2.2.b-1 describes the information flow user regroup response from the group management server to the group management client.

Table 10.2.2.b-1: User regroup response

| Information element | Status | Description |
| --- | --- | --- |
| MC service group ID | M (see NOTE) | MC service group ID of the user regrouped group |
| Result | M | Indicates success or reason for failure |
| NOTE: If the Result information element indicates failure then the value of MC service group ID information element has no meaning. |  |  |

10.2.2.c	User regroup notify

Table 10.2.2.c-1 describes the information flow user regroup notify from the group management server to the MC service server(s).

NOTE:	When group is configured for multiple MC services, the reuser regroup notify message is sent from the group management server to the MC service servers configured for the group.

Table 10.2.2.c-1: User regroup notify

| Information element | Status | Description |
| --- | --- | --- |
| MC service group ID | M | MC service group ID that was created from user regroup operation based on the MC service ID list and the MC services enabled on them |
| MC service ID list | M | List of MC service IDs that are part of the created group from user regroup operation |

10.2.2.d	User regroup notification

Table 10.2.2.d-1 describes the information flow user regroup notification from the group management server to the group management client.

Table 10.2.2.d-1: User regroup notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service group ID | M | MC service group ID of the MC service group being created from user regroup operation |
| MC service group ID | O | MC service group ID of the MC service group from which configuration is to be taken |
| MC service ID list | M | List of MC service IDs that are enabled of the created group |

* * * 3rd Change * * * *

10.2.3	Group creation

Figure 10.2.3-1 below illustrates the group creation operations by authorized MC service user/ MC service administrator to create a group. It applies to the scenario of normal group creation by an MC service administrator.

Pre-conditions:

1.	The group management client, group management server, MC service server and the MC service group members belong to the same MC system.

2.	The administrator/authorized user/dispatcher is aware of the users' identities which will be combined to form the MC service group.

Figure 10.2.3-1: Group creation

1.	The group management client of the administrator/dispatcher/authorized MC service user requests group create operation to the group management server. The identities of the users being combined and the information of the MC services that are enabled on the group shall be included in this message.

NOTE 1:	The administrator/dispatcher/authorized MC service user can include a proposed MC service group ID in the group creation request. The proposed MC service group ID can be derived from a set of rules previously agreed between the administrator/dispatcher/authorized MC service user and the group management server. The method by which the the administrator/dispatcher/authorized MC service user obtains the set of rules is outside the scope of the present document.

NOTE 2: The administrator/dispatcher/authorized MC service user can include include configuration data for the requested group. The method by which the administrator/dispatcher/authorized MC service user prepares the configuration data for the requested group is outside the scope of the present document, but it could, for example, be derived from pre-prepared templates available to the administrator/dispatcher/authorized MC service user.

2a. If the proposed MC service group ID information element is not present in the group creation request, the group management server generates the MC service group ID. If the proposed MC service group ID information element is present and acceptable to the group management server, the group management server uses the proposed MC service group ID for the new group. If the proposed MC service group ID information element is present but not acceptable to the group management server, the group management server modifies proposed MC servicegroup ID or generates the MC service group ID or rejects the group creation request.

2b.	During the group creation, the group management server stores or creates and stores the information of the group as group configuration data as described in subclause 10.1.5.4. The group management server performs the check on the maximum limit of the total number (Nc6) of MC service group members for the MC service group(s).

3.	The group management server may conditionally notify the MC service server regarding the group creation with the information of the group members (if any). During user regroup, the group management server notifies the MC service server regarding the group creation with the information of the temporary group members. The MC service users of the temporary group may be automatically affiliated, if configured on the MC service server.

4.	The MC service group members  (if any) of the MC service group are notified about the newly created MC service group configuration data.

5.	The group management server provides a group creation response to the group management client of the administrator/dispatcher/authorized MC service user. The result information element indicates:

-	success (using proposed MC service group ID); or

-	success (MC service group ID provided by the group management server); or

-	failure (proposed MC service group ID not acceptable); or

-	failure for other reason.

* * * 3rd Change * * * *

10.2.x	User regrouping

A user regroup may be achieved by regrouping MC service users into a new MC service regroup group which uses the configuration of a separate preconfigured MC service group. The MC service regroup group configuration needs to be provided to the relevant MC service users who will be regrouped in advance of the regrouping operation.

NOTE 1:	A preconfigured group which is intended only to provide configuration for the user regroup process is identified by a parameter in group configuration described in this specification.

NOTE 2:	The configuration may alternatively be taken from any MC service group that has been configured in the user profile of all the relevant MC service users who will be regrouped.

NOTE 3:	Regroup users may be defined according to the organizational structure of a mission critical organization, or by some other means which allows the MC service client of an authorized user to be aware of an appropriate regroup group for MC service users that will be regrouped together.

The preconfigured MC service group that provides the configuration is not used as the MC service user regroup group itself, it only provides configuration for one or more MC service regroup users. The MC service group ID of the user regroup group may be provided by the authorized user when the user regrouping is carried out.

After the user regrouping procedure, the regrouping remains in effect until explicitly teardown.

Figure 10.2.x-1 below illustrates the user regrouping operations by authorized MC service user/ MC service administrator to create a group.

Pre-conditions:

1.	The group management client, group management server, MC service server and the MC service group members belong to the same MC system.

2.	The administrator/authorized user/dispatcher is aware of the users' identities which will be combined to form the MC service group.

3.	Initiating group management client  is authorized to initiate a user regroup procedure using a preconfigured group.

4.	Initiating group management client is aware of a suitable MC service users who will be regrouped.

5.	The MC service group identity and group configuration for the preconfigured MC service group have been preconfigured in group management clients of which MC service users to be regrouped.

Figure 10.2.x-1: User regroup creation

1.	The group management client of the administrator/dispatcher/authorized MC service user requests user regroup operation to the group management server. The identities of the users being combined and the information of the MC services that are enabled on the group shall be included in this message.The group management client may specify the MC service group ID of the group from which configuration information for the  user regroup is to be taken if using the preconfigured group for user regrouping.

NOTE 1:	The administrator/dispatcher/authorized MC service user can include a proposed MC service group ID in the user regroup request. The proposed MC service group ID can be derived from a set of rules previously agreed between the administrator/dispatcher/authorized MC service user and the group management server. The method by which the the administrator/dispatcher/authorized MC service user obtains the set of rules is outside the scope of the present document.

NOTE 2: The administrator/dispatcher/authorized MC service user can include configuration data for the requested group. The method by which the administrator/dispatcher/authorized MC service user prepares the configuration data for the requested group is outside the scope of the present document, but it could, for example, be derived from pre-prepared templates available to the administrator/dispatcher/authorized MC service user.

2a. If the proposed MC service group ID information element is not present in the user regroup request, the group management server generates the MC service group ID. If the proposed MC service group ID information element is present and acceptable to the group management server, the group management server uses the proposed MC service group ID for the new group. If the proposed MC service group ID information element is present but not acceptable to the group management server, the group management server modifies proposed MC servicegroup ID or generates the MC service group ID or rejects the user regroup request.

2b.	During the user regrouping, the group management server stores or creates and stores the information of the group as group configuration data as described in subclause 10.1.5.4. The group management server performs the check on the maximum limit of the total number (Nc6) of MC service group members for the MC service group(s). If the MC service group ID of the group from which configuration information for the  user regroup is to be taken, the group management server uses the preconfigured group configuration as the user regrouped group configuration.

3.	The group management server may conditionally notify the MC service server regarding the user regroup with the information of the group members (if any). During user regroup, the group management server notifies the MC service server regarding the user regroup with the information of the temporary group members. The MC service users of the temporary group may be automatically affiliated, if configured on the MC service server.

4.	The MC service group members  (if any) of the MC service group are notified about the newly created MC service group configuration data. If the preconfigured group is used for user regrouping, the MC service group ID of the group from which configuration information for the user regroup group is to be taken is included, and the MC service users being regrouped will not pull the regrouped group data from the group management server. The MC service users use the preconfigured group configuration as the regrouped group configuration.

5.	The group management server provides a user regroup response to the group management client of the administrator/dispatcher/authorized MC service user. The result information element indicates:

-	success (using proposed MC service group ID); or

-	success (MC service group ID provided by the group management server); or

-	failure (proposed MC service group ID not acceptable); or

-	failure for other reason.

If the group management client of the administrator/dispatcher/authorized MC service user is being regrouped, the initiating group management client will not pull the group data from the group management server. The MC service users uses the preconfigured group configuration as the user regrouped group configuration.