3GPP TSG-SA WG6 Meeting #36BIS-e	S6-200503

E-meeting, 31st Mar 2020 – 8th Apr 2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	Service provisioning overview and triggers

Spec:	3GPP TS 23.558 v0.1.2

Agenda item:	7.4

Document for:	Approval

Contact:	Niranth (namogh@huawei.com)

1. Introduction

The contribution provides a proposal for service provisioning overview and triggers.

2. Reason for Change

It is required to understand the impact of service provisioning during the entire lifecycle of the UE when it requires to consume edge computing services. The triggers associated for initiating service provisioning is also required to be captured.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.1.2.

* * * First Change * * * *

## 8.3	Service provisioning

### 8.3.1	General

Service Provisioning allows the Edge Enabler Clients to be configured with information about available edge computing services, based on the hosting UEs location, service requirements, service preferences and connectivity. This procedure provides Edge Enabler Clients with the necessary address information for establishing connections to Edge Enabler Servers.

Figure 8.3.1-1 illustrates the service provisioning in the general life-cycle of a UE consuming edge computing services.

Figure 8.3.1-1: Service provisioning in the lifecycle of a UE consuming edge computing services

The UE is initially provisioned with the configurations required to connect to the Edge Data Network. Upon initial provisioning, the Edge Enabler Client of the UE may register with the provisioned Edge Enabler Server(s). UE further consumes the edge computing services and performs various operations like Edge Application Server discovery, Edge application communications, Application context relocation, etc. While UE is consuming edge computing services, there may be several triggers which causes service provisioning to be initiated by the UE or by the Edge Configuration Server. This cycle of service provisioning continues until the UE consumes edge computing services.

The triggers for service provisioning are classified as:

a.	Triggers at UE - Some examples are:

-	Application Client related updates available at the Edge Enabler Client due to AC installation/re-installation, Application Client requesting application server access (e.g. via internet browser)

-	Edge Enabler Client supporting one or more Application Clients may be updated due to Edge Enabler Client re-installation.

b.	Triggers at Edge Configuration Server – Some example are:

-	Edge Enabler Server updates due to Edge Application Server installation/re-installation/re-location, EDN/DNAI changes of the UE.

NOTE:	The DNAI changes can be detected at the Edge Configuration Server if the Core Network type is 5GC.