3GPP TSG-SA WG6 Meeting #36BIS-e	S6-200504

E-meeting, 31st Mar 2020 – 8th Apr 2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	Service provisioning input

Spec:	3GPP TS 23.558 v0.1.2

Agenda item:	7.4

Document for:	Approval

Contact:	Niranth (namogh@huawei.com)

1. Introduction

The contribution provides a proposal for service provisioning input.

2. Reason for Change

An Application Client is configured to connect to an EAS by default. This information is usually the FQDN of the EAS which is resolved by the network for the intial connectivity to the EAS. An example of Default EAS ID is an FQDN.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.1.2.

* * * First Change * * * *

### 7.2.4	Edge Application Server ID (EAS ID)

The Edge Application Server ID identifies a particular application for e.g. SA6Video, SA6Game etc. For example, all Edge SA6Video Servers will share the same Edge Application Server ID.

NOTE:	The format for the EAS ID is the responsibility of stage 3.

* * * Next Change * * * *

### 8.2.2	Application Client Profile

An Application Client Profile includes information about Application Client used to determine services and service characteristics required.

Table 8.2.2-1: Application Client Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The associated EAS identifier for the application client |
| Application Client Type | M | The category or type of Application Client (e.g. V2X) |
| Preferred ECSP list | O | The identity of Edge Computing Service Provider(s) whose associated EES(s) the EEC prefers to connect to. If this field is present, the ECS may customise its response based on this preference. |
| Application Client Schedule | O | The expected operation schedule of the Application Client (e.g. time windows) |
| Application Client Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the Application Client's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| Application Client Service KPIs | O | KPIs required in order for Application Clients to receive services form the EAS, as described in Table 8.2.3-1 |