3GPP TSG-SA WG6 Meeting #36-BIS-e	S6-200512

E-meeting, 31st March – 8th April 2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	Clarification on EAS registration

Spec:	3GPP TS 23.558 v0.1.2

Agenda item:	7.4

Document for:	Approval

Contact:	Cuili Ge – geuili@huawei.com

1. Introduction

This pCR is proposed to provide clarification on EAS registration.

2. Reason for Change

In the Edge Application Server Registration response, the Expiration time IE is mandatory. However, for some EAS, the registration or the registration update is triggered by events e.g., because the Edge Application Server's status or availability schedule has changed as described in clause 8.4.3.2, rather than periodic timer. It is proposed to make this IE optional.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.1.2.

* * * First Change * * * *

#### 8.4.3.2	Procedure

Pre-conditions:

1.	The Edge Application Server has been configured with an Edge Application Server Identity;

2.	The Edge Application Server has been configured with the address (e.g. URI) of the Edge Enabler Server; and

3.	Both the Edge Application Server and Edge Enabler Server have been configured with the necessary credentials to enable communications.

Figure 8.4.3.2-1: Edge Application Server Registration procedure

1.	The Edge Application Server determines that registration to the Edge Enabler Server is needed (e.g. the Edge Application Server is instantiated and started up). The Edge Application Server may also determine that its existing registration needs to be updated (e.g. because the Edge Application Server's status or availability schedule has changed).

2.	The Edge Application Server sends a Registration Request to the Edge Enabler Server. The request indicates if the request is for a new registration or an update to an existing registration.

3.	The Edge Enabler Server performs an authorization check to verify whether the Edge Application Server has the proper privileges to register.

4.	Upon successful authorization, the Edge Enabler Server stores the Service Profile for later use (e.g. for serving Edge Application Server discovery requests received from Edge Enabler Clients, etc.) and replies to the Edge Application Server with a Registration Response. The Edge Enabler Server may provide an expiration time to indicate to the Edge Application Server when the registration will automatically expire. The Edge Application Server shall send a Registration Request message with a Registration Type Indicator of type Update prior to the expiration to maintain the current registration. The Edge Enabler Server will treat the registration expiration as an implicit de-registration.

##### 8.4.3.3.2	Edge Application Server Registration response

Table 8.4.3.3.2-1 describes information elements in the Edge Application Server Registration Response from the Edge Enabler Server to the Edge Application Server.

Table 8.4.3.3.2-1: Edge Application Server Registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the registration operation |
| Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| NOTE: If the Expiration time IE is not included, it indicates that the registration never expires. |  |  |