3GPP TSG-SA WG6 Meeting #36-BIS-e	S6-200513

E-meeting, 31st March – 8th April 2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	EAS discovery update

Spec:	3GPP TS 23.558 v0.1.2

Agenda item:	7.4

Document for:	Approval

Contact:	Cuili Ge – geuili@huawei.com

1. Introduction

This pCR is proposed to update the EAS discovery procedure.

2. Reason for Change

(1) EAS Endpoint can not be the FQDN, which is not sufficient enough to establish connection the discovered EAS.

(2) which EES should the EEC connect to for EAS discovery ?

Currently, one or more EES(s) are provisioned to the UE located in different geographical area or at different instances of time. Also one EES may not have all the EAS(s) associated with the Application Clients at the UE. The UE has to connect to the EES which has the EAS registered and can serve the Application Client at the UE. This aspect has to be clarified.

(3) Parameters overlapping with each other in EAS discovery request

The EAS IDs and EAS ID list in table 8.5.2.3.1-2: Query Filters seems refer to the same concept defined in 7.2.4 and identify a particular application as FQDN does.

As defined in 7.2.5, the AC ID identifies the client side of a particular application, which still indicates a particular application.

So EAS IDs, EAS ID list, Application Client ID(s) are all used to identify a particular application.

The user’s preference for different application is different, so each application should be provide its own query filter if required.

(4) The following EN is to be resolved by using EAS profile to replace several of the IEs.

Editor's Note:	It is FFS whether EAS Profiles should replace several of the IEs in the Discovered EAS List.

(5) It is not clear a single EAS or multiple EASs is returned to EEC.

As discussed, the EES determines the single EAS for AC to access via the EAS discovery procedure.

(6) Align the message name in the figure with the information flow tables

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.1.2.

* * * First Change * * * *

### 8.2.2	Application Client Profile

An Application Client Profile includes information about Application Client used to determine services and service characteristics required.

Table 8.2.2-1: Application Client Profile

| Information element | Status | Description |
| --- | --- | --- |
| Default EAS ID | M | The default EAS for the application client |
| Application Client Type | M | The category or type of Application Client (e.g. V2X) |
| Application Client ID | M | Identity of Application Client |
| ECSP Filter | O | The identity of Edge Computing Service Provider(s) that the EEC is willing to connect to. If this field is present, the ECS may filter its response based on this preference. |
| Application Client Schedule | O | The expected operation schedule of the Application Client (e.g. time windows) |
| Application Client Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the Application Client's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| Application Client Service KPIs | O | KPIs required in order for Application Clients to receive services form the EAS, as described in Table 8.2.3-1 |

* * * Next Change * * * *

### 8.2.4	Edge Application Server Profiles

Table 8.2.4-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| Supported service features | O | Supported service features e.g. single vs. multi-player gaming service |
| Supported service permission levels | O | Supported level of service permissions e.g. trial, gold-class |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Service Area | O | The geographical service area that the EAS serves |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heart beat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Editor's Note: The inclusion of EAS Instance ID (including optionality) is dependent on how the ID is defined and assigned, which is FFS.

* * * Next Change * * * *

### 8.5.2	Edge Application Server Discovery

#### 8.5.2.1	General

Edge Application Server discovery enables Edge Enabler Clients to obtain information about available Edge Application Servers of interest. The identification of the Edge Application Servers is based on matching query filters or Application Client Profiles provided in the request.

#### 8.5.2.2	Procedure

Pre-conditions:

1.	Information related to the Edge Enabler Server is available with the Edge Enabler Client.

Figure 8.5.2.1-1: Edge Application Server Discovery procedure

1.	The Edge Enabler Client sends an Edge Application Server discovery request to the Edge Enabler Server. The discovery request may contain query filters to retrieve Edge Application Server information corresponding to a particular Application Client or a category of  applications, e.g. gaming applications. Query filters may also indicate a desired level of service permissions (e.g. trial, gold-class, etc.), desired features (e.g. multi-player or single player), or desired location availability.

2.	Upon receiving the request from the Edge Enabler Client, the Edge Enabler Server checks if the Edge Enabler Client is authorized to discover the requested Edge Application Server(s). The authorization check may apply to an individual Edge Application Server, a category of Edge Application Servers or to the Edge Data Network, i.e. to all the Edge Application Servers. If the discovery request contained query filters that indicated a desired level of service permissions, features, and/or location availability, then the Edge Enabler Server may also check if the discovered Edge Application Servers are able to provide the desired level of service permissions, features, and/or location availability.

The Edge Enabler Server determines a set of registered Edge Application Servers that match the query filters (e.g. Application Client Profiles) provided in the request.

3.	The Edge Enabler Server sends an Edge Application Server discovery response including information about the discovered Edge Application Servers. For registered discovered Edge Application Servers, this includes endpoint information. Depending on the query filters or Application Client Profiles in the discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, Application Client locations(s) that the Edge Application Servers can support, KPIs, etc.

Upon receiving the Edge Application Server discovery response, the Edge Enabler Client uses the endpoint information for routing of the outgoing application data traffic to Edge Application Server(s), as needed, and may provide necessary notifications to the Application Client(s).

#### 8.5.2.3	Information flows

##### 8.5.2.3.1	Edge Application Server discovery request

Table 8.5.2.3.1-1 describes information elements for the Edge Application Server Discovery Request from the Edge Enabler Client to the Edge Enabler Server. Table 8.5.2.3.1-2 provides further detail about the Query Filter information element.

Table 8.5.2.3.1-1: Edge Application Server discovery request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the Edge Enabler Client. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | O | Security credentials resulting from a successful authorization for the edge computing service. |
| List of Application Query Filter(s) | M | List of characteristics per Application for discovery purposes, as detailed in Table 8.5.2.3.1-2. |

Table 8.5.2.3.1-2: Application Query Filter

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | Identifier of the Edge Application Server whose details are to be discovered. |
| Application provider(s) | O | Provider of the Application Client |
| Service permission level(s) | O | Desired level of service permissions e.g. trial, gold-class |
| Desired feature(s) | O | Desired service features e.g. single vs. multi-player gaming service |
| Location availability | O | The desired location(s) (e.g. geographical area, route) where the service should be available. |
| Application Client Profile(s) | O | Profiles of Application Clients with parameters used to determine matching EA. Application Client Profiles are further described in Table 8.2.2-1. |

Editor's Note:	The logic on how the query filters are applied in conjunction with the Application Client Profiles is to be clarified.

Editor's Note:	If and how complex Information Elements may be broken down into multiple elements is FFS.

##### 8.5.2.3.2	Edge Application Server discovery response

Table 8.5.2.3.2-1 describes information elements for the Edge Application Server Discovery Response from the Edge Enabler Server to the Edge Enabler Client.

Table 8.5.2.3.2-1: Edge Application Server discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Discovered EAS list | O | List of discovered EAS. Each element corresponds to the information related to one EAS and includes the information described below. |
| >EAS profile | M | The EAS profile as specified in clause 8.2.4 corresponding to the discovered EAS |
| >EAS Available Compute | O | The maximum compute resource available for the Application Client |
| >EAS Available Graphical Compute | O | The maximum graphical compute resource available for the Application Client |
| >EAS Available Memory | O | The maximum memory resource available for the Application Client |
| >EAS Available Storage | O | The maximum storage resource available for the Application Client |

Editor's Note:	The use of EAS Instance ID (including optionality) is dependent on how the ID is defined and assigned, which is FFS.

Editor's Note:	If and how complex Information Elements may be broken down into multiple elements is FFS

* * * End of Change * * * *