3GPP TSG-SA WG6 Meeting #36-BIS-e	S6-200535

E-meeting, 31st March – 08th April 2020	(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0247 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding a procedure for off-network storing of event-triggered location information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This procedure describes the event where the location management client is not able to send location information reports to the location management server because the UE is off-network. In that situation it is required that the location management client is able to locally store the event-triggered location information reports. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a procedure for off-network storing of event-triggered location information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Required off-network location information reports will not be stored in the location management client when the UE is off-network and lost for processing after the UE becomes on-network again. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.3.1.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 10.9.3	Procedure

#### 10.9.3.1	Event-triggered location reporting procedure

NOTE 1: This procedure is valid for single MC system operation only.

The location management server provides location reporting configuration to the location management clients, indicating what information the location management server expects and what events will trigger the sending of this information to the location management server. The decision to report location information can be triggered at the location management client by different conditions. The conditions could include, for example, the reception of the location reporting configuration, initial registration, distance travelled, elapsed time, cell change, MBMS SAI change, MBMS session change, leaving a specific MBMS bearer service area, tracking area change, PLMN change, call initiation, or other types of events such as emergency alert, emergency call or imminent peril calls. The location report can include information described as ECGI, MBMS SAIs, geographic coordinates and other location information.

Figure 10.9.3.1-1: Event-triggered location reporting procedure

1.	The location management server sends location reporting configuration message to the location management client(s) containing the initial configuration (or a subsequent update) for reporting the location of the MC service UE. This message can be sent over a unicast bearer to a specific location management client or as a group message over an MBMS bearer to update the location reporting configuration for multiple location management clients at the same time.

NOTE 2:	The location reporting configuration information can be made part of the user profile, in which case the sending of the message is not necessary.

NOTE 3:	Different location management clients may be given different location reporting criteria.

2.	A location reporting event occurs, triggering step 3.

3.	The location management client sends a location information report to the location management server, containing location information identified by the location management server and available to the location management client.

4.	Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client.

##### 10.9.3.1.1	Off-network storing of event-triggered location information procedure

The location management client may get into an off-network situation at any time after the initial location reporting configuration was provided by the location management server. If any location information trigger applies, the location management client locally stores the corresponding location information report. The conditions in off-network situation may vary from the conditions in an on-network situation and may e.g. include distance travelled, elapsed time, call initiation, off-network emergency alert, off-network emergency group call, off-network imminent peril group call and off-network emergency private call.

NOTE 1:	The procedure may also apply in some scenarios of isolated operation for public safety. Which conditions are applicable in this case, may depend on the particular isolated operation scenario. They may include all or a subset of the on-network conditions and e.g. in case only one single base station is available, the conditions entering cell and leaving cell may be relevant instead of cell change.

Figure 10.9.3.1.1-1 illustrates the procedure for the location management client to store event-triggered location reports while being off-network.

Figure 10.9.3.1.1-1: Off-network storing of event-triggered location information procedure

1.	The location management server sends location reporting configuration message to the location management client(s) containing the initial configuration (or a subsequent update) for reporting the location of the MC service UE.

NOTE 2:	The location reporting configuration information can be made part of the user profile, in which case the sending of the message is not necessary.

NOTE 3:	Different location management clients may be given different location reporting criteria.

2.	The location management client loses connection to the location management server or changes to off-network operation.

3.	A location reporting event occurs.

4.	The location management client locally stores the location information report, containing location information identified by the location reporting configuration and available to the location management client.

NOTE 4:	It is proposed, that the location management client stores location information based on a configurable period of time or number of reports.

NOTE 5:	The location information report may include timestamp, ECGI, MBMS SAIs, longitude, latitude, accuracy, speed, bearing, altitude, measurement code and additional other location information.

* * * Next Change * * * *

* * * End of Change * * * *