3GPP TSG-SA WG6 Meeting #36-BIS-e	S6-200540

# E-meeting, 31st March – 8th April 2020											(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 258 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minor changes to ambient listening procedures on user indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-03-26 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NOTE related to user indication about the ambient listening call was wrong |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR corrects those NOTE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proecures could be interpreted wrongly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14.3.1, 10.14.3.2, 10.14.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

### 10.14.3	Ambient listening call procedures

#### 10.14.3.1	Remotely initiated ambient listening call setup

The MCPTT service provides the capability for an authorised user to initiate a remotely initiated ambient listening call at an MCPTT client.

Figure 10.14.3.1-1 illustrates the information flow for remotely initiated ambient listening call setup.

Pre-conditions:

-	MCPTT client 1 is the client of the authorized user who is authorized to invoke a remotely initiated ambient listening call to be set up at the requested MCPTT client 2.

-	MCPTT user 1 is the "listening" user at MCPTT client 1, and MCPTT user 2 is the "listened to" user at MCPTT client 2.

Figure 10.14.3.1-1: Remotely initiated ambient listening call

1.	MCPTT client 1 initiates a remotely initiated ambient listening call by sending the ambient listening call request to the MCPTT server. The remotely initiated ambient listening call type is included.

2.	The MCPTT server performs an authorization check for the authorized user 1 for the remotely initiated ambient listening call. If authorization fails, the MCPTT server provides a failure response to MCPTT client 1.

3.	The MCPTT server sends the ambient listening call request to MCPTT client 2.

NOTE:	MCPTT client 2 does not provide any indication of the ambient listening call request to its user.

4.	MCPTT client 2 returns the ambient listening call response to the MCPTT server.

5.	MCPTT server provides an ambient listening call response to MCPTT client 1, indicating whether the call is set up successfully or not.

6.	The floor control server of the MCPTT server then sends a floor granted to MCPTT client 2 according to the ambient listening type received in step 1.

7.	Accordingly, the floor control server of the MCPTT server sends a floor taken to MCPTT client 1.

8.	After receiving the floor granted message at the MCPTT client 2, the media is transmitted from MCPTT client 2 to MCPTT client 1.

#### 10.14.3.2	Locally initiated ambient listening call setup

The MCPTT service provides the capability for an authorised user to initiate a locally initiated ambient listening call at an MCPTT client.

Figure 10.14.3.2-1 illustrates the information flow for locally initiated ambient listening call setup.

Pre-conditions:

-	MCPTT client 2 is the client of the authorized user who is authorized to invoke a locally initiated ambient listening call to be set up at the requested MCPTT client 1.

-	MCPTT user 1 is the "listening" user at MCPTT client 1, and MCPTT user 2 is the "listened to" user at MCPTT client 2.

Figure 10.14.3.1-1: Locally initiated ambient listening call

1.	MCPTT client 2 initiates a locally initiated ambient listening call by sending the ambient listening call request to the MCPTT server. The locally initiated ambient listening call type is included.

2.	The MCPTT server performs an authorization check for the authorized user 2 for the locally initiated ambient listening call. If authorization fails, the MCPTT server provides a failure response to MCPTT client 2.

3.	The MCPTT server sends the ambient listening call request to MCPTT client 1.

4.	MCPTT client 1 returns the ambient listening call response to the MCPTT server.

5.	MCPTT server provides an ambient listening call response to MCPTT client 2, indicating whether the call is set up successfully or not.

6.	The floor control server of the MCPTT server then sends a floor granted to MCPTT client 2 according to the ambient listening type received in step 1.

7.	Accordingly, the floor control server of the MCPTT server sends a floor taken to MCPTT client 1.

8.	After receiving the floor granted message at the MCPTT client 2, the media is transmitted from MCPTT client 2 to MCPTT client 1.

NOTE:	MCPTT client 2 does not provide any indication of the ambient listening call request to its user.

* * * * * * * SECOND CHANGE * * * * * * *

#### 10.14.3.5	Ambient listening call release – "listened to" user initiated

Figure 10.14.3.5-1 illustrates the information flow for ambient listening call release – "listened to" user initiated. This procedure is only applied for the locally initiated ambient listening call.

Pre-conditions:

-	There is an ongoing ambient listening call between MCPTT client 1 and MCPTT client 2.

-	MCPTT user 1 is the "listening" user at MCPTT client 1, and MCPTT user 2 is the "listened to" user at MCPTT client 2.

-	MCPTT client 2 is the MCPTT client of the authorized user, who is authorized to release the locally initiated ambient listening call at MCPTT client 2.

Figure 10.14.3.5-1: Ambient listening call release – "listened to" user initiated

1.	The authorized user 2 at MCPTT client 2 initiates the ambient listening call release by sending an ambient listening call release request to the MCPTT server.

2.	The MCPTT server provides an ambient listening call release request to MCPTT client 1.

3.	The authorized user 1 at MCPTT client 1 is notified about the ambient listening call release.

4.	MCPTT client 1 provides an ambient listening call release response to the MCPTT server.

5.	The MCPTT server provides the ambient listening call release response to MCPTT client 2.

6.	MCPTT client 2 stops transmitting media to MCPTT client 1.

NOTE:	MCPTT client 2 does not provide any indication of the ambient listening call release to its user.

* * * * * * * END CHANGES * * * * * * *