3GPP TSG-SA WG6 Meeting #36-e	S6-200380

# E-meeting, 24th – 28th Feb 2020								         				(revision of S6-200xxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0209 | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IP Connectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To apply appropriate changes made to R17 in CR0199 rev2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align and correct message names with the procedure used. - Add new information elements that are needed for the procedure. - Add precondition to calrify the use case in procedure. - Correct text in procedure step for clarity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 development might be wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.2, 7.14.2.1, 7.14.2.1.1, 7.14.2.1.2, 7.14.2.2, 7.14.2.2.1, 7.14.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Same changes for R-17 are captured in CR0199rev2 agreed in SA6 #35 meeting in Hyderabad, India. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.14.2	IP connectivity for on-network

#### 7.14.2.1	Information flows for IP connectivity

##### 7.14.2.1.1	MCData IPcon point-to-point request

Table 7.14.2.1.1-1 describes the information flow of the MCData IPcon point-to-point request sent from the MCData client to the MCData server and from the MCData server to the addressed MCData client.

Table 7.14.2.1.1-1: MCData IPcon point-to-point request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the originator MCData user; |
| Functional alias | O | The associated functional alias of the originator MCData user; |
| MCData ID | M | The MCData identity of the target MCData client IP connectivity is requested. |
| Application Priority | O | Contains the required application priority for the IP data communication. If not present, the MCData applies the predefined default application priority after reception of MCData IPcon response. |
| Location Information | O (see NOTE) | Actual location information of the originating MCData user; |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite); |
| Establishment reason | O | IP connectivity establishment reason |
| NOTE: This information contains the latest available location information of the requesting MCData user that may be different to the latest available location information in the MC system. |  |  |

##### 7.14.2.1.2	MCData IPcon point-to-point response

Table 7.14.2.1.2-1 describes the information content of the MCData IPcon point-to-pointresponse as answer to MCData IPcon point-to-point data request.

Table 7.14.2.1.2-1: MCData IPcon point-to-point response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the targeted MCData user. |
| MCData ID | M | The MCData identity of the requesting MCData user. |
| Time Limit | O | Negotiated time (1 min – infinite) |
| IP connectivity status | M | IP connectivity establishment result |

#### 7.14.2.2	IP connectivity point-to-point MCData transport service

##### 7.14.2.2.1	General

IP connectivity service capabilities enables MCData unaware data host to use usual MCData service capabilities, e.g. data communication between them.. This subsection describes the establishment of a point-to-point connection between two IP connectivity clients using the media plane for IP Data transmission.

##### 7.14.2.2.2	Procedure

The procedure in figure 7.14.2.2.2-1 describes the case where an IP connectivity capable MCData client is initiating a point-to-point IP connectivity with another IP connectivity capable MCData client.

Pre-conditions:

-	The total data volume limit, e.g. daily time limit or total data volume per day does not restrict the establishment of an IP connectivity IP data exchange.

-	MCData clients are linked with individual data hosts.

-	MCData clients belong to the same MCData system.

-	The data hosts linked with the MCData clients already have an IP address allocated.

-	MCData clients have IP connectivity capabilities.

-	The linked data hosts are authorized to use the MCData clients to establish an IP connectivity.

NOTE: How the data host is authorized to use the MCData client is out of the scope of the present document.

-	The MCData server has subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

-	MCData client 1 understands the correspondence between the IP addresses of target data hosts and MCData client 2. How this relationship is determined is out of scope of the present document.

Figure 7.14.2.2.2-1: Establishment of a point-to-point IP connectivity

1.	MCData client 1 has IP Data to send to MCData client 2 and initiates an IP connectivity point-to-point request.

2.	MCData client 1 sends a MCData IPcon point-to-point request towards the MCData server. The MCData IPcon point-to-point request contains the MCData ID of MCData client 2. MCData user at MCData client 1 may include its associated functional alias.

3.	MCData server checks whether MCData user at MCData client 1 is authorized to send an MCData IPcon point-to-point request and checks if MCData client 2 is authorised to receive the IP connectivity service.

4.	MCData server initiates the MCData IPcon point-to-point request towards the MCData client 2.

5.	MCData client 2 sends a MCData IPcon point-to-point response to the MCData server that contains the information if the request is accepted or the reason of rejection. If accepted, the MCData client 2 may include the data transmission time limit.

6.	MCData server forwards the MCData IPcon point-to-point response of MCData client 2 to MCData client 1.

7.	The MCData server applies transmission and reception control and the necessary policy to ensure that appropriate data is transmitted between the MCData clients.

8.	MCData client 1 and MCData Client 2 have successfully established media plane for data communication and MCData client 1 and MCData client 2 exchange IP Data.