3GPP TSG-SA WG6 Meeting #36-e	S6-200391

# E-meeting, 24th – 28th Feb 2020													(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0017 | rev | <Rev#> | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on V2X USD and V2X server USD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Datang Mobile Com. Equipment |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The V2X USD and V2X server USD are specified as two separate configurations in TS 23.285: the V2X USD is per PLMN specific for receiving MBMS based V2X traffic, and the V2X server USD is local V2X application server specific for receiving MBMS based V2X traffic. Both the V2X USD and V2X server USD should be able to be provisioned by the VAE server by means of configuration management service and the V2X USD announcement. For the V2X USD provisioning, using multicast is different than unicast in that the message is usually sent periodically over MBMS broadcast channel, with which the VAE server doesn’t necessarily receive any acknowledgement from the clients. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add V2X USD as the configuration information. Add V2X server USD case in V2X USD provisioning procedure. Clarify the procedure of V2X USD provisioning using multicast. Fix the description regarding configuration management service for multicast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is possible that the V2X communication over multicast cannot setup because of the incomplete provisioning. Redundant signaling for V2X USD provisioning over multicast exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 9.1.2, 9.6.3, C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AS	Application Server

AID	Application Object Identifier

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EPS	Evolved Packet System

ETSI	European Telecommunications Standards Institute

LTE	Long-Term Evolution

MBMS	Multimedia Broadcast Multicast Service

PSID	Provider Service Identifier

SAE	Society of Automotive Engineers

SCEF	Service Capability Exposure Function

SCS	Services Capability Server

UE	User Equipment

USD	User Service Description

V2I	Vehicle-to-Infrastructure

V2N	Vehicle-to-Network

V2P	Vehicle-to-Pedestrian

V2V	Vehicle-to-Vehicle

V2X	Vehicle-to-Everything

VAE	V2X Application Enabler

* * * Next Change * * * *

9.1.2	Configuration management service

9.1.2.1	General

The VAE capabilities (VAE client and VAE server) utilize configuration management service procedures of SEAL to support V2X services for unicast delivery over LTE-Uu. The configuration management service of SEAL is utilized for offline configuration (before the V2X UE is receiving the V2X service from the V2X application specific server) and online configuration (during the V2X UE is receiving the V2X service from the V2X application specific server).

9.1.2.2	Information flows

The following information flows of configuration management service are applicable for the V2X applications:

-	Get VAL UE configuration request specified in subclause 11.3.2.1 of 3GPP TS 23.434 [6];

-	Get VAL UE configuration response specified in subclause 11.3.2.2 of 3GPP TS 23.434 [6];

The usage of the above information flows are clarified as below:

-	The VAL UE ID is the V2X UE ID;

-	VAL service ID is the V2X service ID; and

-	VAL UE configuration data is the V2X UE configuration data. The V2X UE configuration data includes:

--	V2X USD information as specified in 3GPP TS 23.285 [5]; and

--	V2X server USD information as specified in 3GPP TS 23.285 [5]; and

--	V2X application server address information as specified in 3GPP TS 23.285 [5] and transport port for unicast.

9.1.2.3	Procedures

The following procedures of configuration management service are applicable for the V2X applications:

-	VAL UE configuration data specified in subclause 11.3.3 of 3GPP TS 23.434 [6].

--	Once the V2X UEs are connected to the serving PLMN, the V2X UE is configured with configuration data (e.g. local service information) for receiving V2X services.

* * * Next Change * * * *

9.6.3	V2X USD provisioning

9.6.3.1	General

This subclause describes the procedure for VAE server provisioning the V2X USD information to V2X UE via V1-AE reference point for V2X communication using MBMS.

9.6.3.2	Procedure

Figure 9.6.3.2-1 illustrates the procedure for provisioning V2X USD to the V2X UE via V1-AE reference point. This procedure also applies for provisioning of the V2X server USD.

Pre-conditions:

1.	The V2X UE has connected to the VAE server.

2.	If multicast delivery mode is used, the MBMS bearer being used is activated by the VAE server.

Figure 9.6.3.2-1: V2X USD provisioning

1.	The VAE server is triggered for providing V2X USD to V2X UE.

2.	The VAE server sends the V2X USD announcement to the VAE client in the V2X UE with the information of the V2X USDs corresponding to the V2X applications. The V2X USD information consists of TMGI, list of SAIs, frequency and SDP information for V2X applications' communication using MBMS. The details of V2X USD are specified in subclause 4.4.7.2 in 3GPP TS 23.285 [5]. This message can be sent via unicast or multicast.

3.	Upon receiving the V2X USD announcement, the VAE client of the V2X UE stores the received V2X USDs.

NOTE:	The principles for storing and applying precedence when V2X USD is configured via different mechanisms (e.g. MBMS service announcement or pre-configuration or by VAE server via V1-AE reference point) is as per V2X service provider's policy.

4.	The VAE client of the V2X UE provides an acknowledgement to the VAE server of the V2X AS. This step is omitted when the announcement is sent via multicast.

* * * Next Change * * * *

C.2	VAE client configuration data

The procedure of VAE client related configuration are specified in 3GPP TS 23.434 [6]. Data in tables C.2-1 has to be known by the VAE client.

Data in table C.2-1 can be configured using the CM-UU reference point as specified in 3GPP TS 23.434 [6].

Table C.2-1: VAE client related configuration data

| Reference | Parameter description |
| --- | --- |
| Clause 8.1 and clause 9.1.2.2 | V2X user/UE identity |
| Clause 9.1.2.2 | V2X service ID |
| Clause 9.1.2.2 | V2X USD |
| Clause 9.1.2.2 | V2X server USD |
| Clause 9.2.3 and Clause 9.1.2.3 | IP address and port information of the initial VAE server serving the VAE client |
| Clause 9.3.3 | GEO ID information |