3GPP TSG-SA WG6 Meeting #36-e	S6-200427

# E-meeting, 24th – 28th Feb 2020								         				(revision of S6-200387)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0238 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuring APN and PDN-Credentials details in UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently the MC Service APNs and the credentials to access the PDNs identified by MC Service APNs being used for MC services are configured either via UE (pre configuration) or via initial UE configuration. In case if any of the APN configuration/PDN Credentials changes either in HPLMN or VPLMN there is no way for the UE to get these changes without manual intervention. This will be a major advantage to the device vendors and Operators to make any changes to the existing configuration hassle free. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed one another way in addition to the existing mechanism to push the configuration changes to MC UE. This CR proposes that these configuration can be pushed when MC UE is registering with PLMN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No way for pushing the changes related to configuration either in HPLMN or VPLMN to UE without manual intervention. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

### 5.2.7	Bearer management

#### 5.2.7.0	General

The MC service UE shall use the following APNs:

-	an MC services APN for the SIP-1 reference point;

-	an MC common core services APN for the HTTP-1 reference point; and

-	an MC identity management service APN for the CSC-1 reference point.

The value of each of these APNs:

-	may be the same or may differ;

-	may be the same as other non-MC services that have compatible QoS and PDN (see NOTE); and

-	shall be made available to the UE either via UE (pre)configuration or via initial UE configuration (see subclause 10.1.1) on a per HPLMN and optionally also a per VPLMN basis or while UE registering to PLMN (when MC service provider and the home PLMN operator are part of the same organization).

NOTE:	The APN value of "IMS" is a well-known APN, whose PDN connection characteristics are defined in GSMA PRD IR.92 [23] and GSMA PRD IR.88 [24], and which is used in some deployments for operator IMS-based services e.g. Voice over LTE. This well-known APN can be used for the MC service APN if the SIP core belongs to the PLMN operator and both the PLMN operator and MC service provider have agreed which QoS aspects to utilise i.e. either the QoS aspects defined in subclause 5.2.7.2 or the QoS aspects defined in GSMA PRD IR.92 [23] and GSMA PRD IR.88 [24].

The MC service UE may utilise PDN access credentials as specified in 3GPP TS 23.401 [17] (e.g. PAP, CHAP) to access the PDNs identified by the MC service APN, the MC common core services APN and the MC identity management service APN. If PDN access credentials are required, then they shall be made available to the MC service UE either via initial MC service UE configuration (see subclause 10.1.1) or while UE registering to PLMN (when MC service provider and the home PLMN operator are part of the same organization) on a per APN basis.

The PDN connection to the APNs defined within the present subclause can be of type "IPv4", "IPv6" or "IPv4v6" (see 3GPP TS 23.401 [17]). If a PDN connection to an APN defined within the present subclause is of type "IPv4v6" then the MC service client shall use configuration data to determine whether to use IPv4 or IPv6.

Editor’s note: Making the configurations available to UE while registering to PLMN when MC service provider and the home PLMN operator are not part of the same organization is FFS.

* * * * * * * END CHANGES * * * * * * *