3GPP TSG-SA WG6 Meeting #38-e	S6-201010

# e-meeting, 20th – 31th July 2020													(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0223 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update information flows to support interconnection for SDS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2020-07-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Railway communication requires support interconnection for the short data service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds missing server to server information flows for the short data service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No server to server SDS messages available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.1.1, 7.4.2.1.2, 7.4.2.1.3, 7.4.2.1.4, 7.4.2.1.5, 7.4.2.1.6, 7.4.2.1.8, 7.4.2.1.9, 7.4.2.1.11, 7.4.2.1.12, 7.4.2.1.14, 7.4.2.1.15, 7.4.2.1.16, 7.4.2.1.17, 7.4.2.1.18, 7.4.2.1.19, 7.4.2.1.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.4.2.1.1	MCData standalone data request

Table 7.4.2.1.1-1 describes the information flow for the MCData standalone data request sent from the MCData client to the MCData server, from MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.1-1: MCData standalone data request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ID (see NOTE 1) | O | The identity of the MCData user towards which the data is sent |
| Functional alias (see NOTE 1) | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 2) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Payload | M | SDS content |
| NOTE 1: Either the MCData ID or the functional alias must be present. NOTE 2: The application identifier shall be included only if the payload destination type indicates that the payload is for application consumption. |  |  |

Table 7.4.2.1.1-2: MCData standalone data request (MCData server to MCData server and MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData client consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Payload | M | SDS content |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the payload is for application consumption. |  |  |

##### 7.4.2.1.2	MCData data disposition notification

Table 7.4.2.1.2-1 describes the information flow for the MCData data disposition notification sent from the MCData client to the MCData server, from MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.2-1: MCData data disposition notification

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user towards which the notification is sent |
| MCData ID | M | The identity of the MCData user sending notification |
| Conversation Identifier | M | Identifies the conversation |
| Disposition association | M | Identity of the original MCData transaction |
| Disposition | M | Disposition which is delivered or read or both |

##### 7.4.2.1.3	MCData standalone session data request

Table 7.4.2.1.3-1 describes the information flow for the MCData standalone session data request sent from the MCData client to the MCData server, from MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.3-1: MCData standalone session data request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ID (see NOTE 1) | O | The identity of the MCData user towards which the data is sent |
| Functional alias (see NOTE 1) | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Standalone transaction |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 2) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Requested Priority | O | Application priority level requested for this communication. |
| SDP offer | M | Media parameters offered |
| NOTE 1: Either the MCData ID or the functional alias must be present. NOTE 2: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

Table 7.4.2.1.3-2: MCData standalone session data request (MCData server to the MCData server and MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Transaction type | M | Standalone transaction |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.4	MCData standalone session data response

Table 7.4.2.1.4-1 describes the information flow for the MCData standalone session data response sent from the MCData client to the MCData server, from MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.4-1: MCData standalone session data response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user receiving data |
| MCData ID | M | The identity of the MCData user sent data |
| Conversation Identifier | M | Identifies the conversation |
| SDP answer | M | Media parameters selected |
| Establishment reason | M | Reason for establishment or rejection |

##### 7.4.2.1.5	MCData session data request

Table 7.4.2.1.5-1 describes the information flow for the MCData session data request sent from the MCData client to the MCData server, from MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.5-1: MCData session data request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ID (see NOTE 1) | O | The identity of the MCData user towards which the data is sent |
| Functional alias (see NOTE 1) | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Session based transactions |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 2) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE 1: Either the MCData ID or the functional alias must be present. NOTE 2: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

Table 7.4.2.1.5-2: MCData session data request (MCData server to the MCData server and MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| MCData ID | O | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Session based transactions |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.6	MCData session data response

Table 7.4.2.1.6-1 describes the information flow for the MCData session data response sent from the MCData client to the MCData server, from MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.6-1: MCData session data response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user receiving data |
| MCData ID | M | The identity of the MCData user sent data |
| Conversation Identifier | M | Identifies the conversation |
| SDP answer | M | Media parameters selected |

##### 7.4.2.1.7	MCData group standalone data request (MCData client – MCData server)

Table 7.4.2.1.7-1 describes the information flow for the MCData group standalone data request (in subclause 7.4.2.5.2) sent from the MCData client to the MCData server.

Table 7.4.2.1.7-1: MCData group standalone data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Payload | M | SDS content |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.8	MCData group standalone data request (MCData server - MCData server and MCData server – MCData client)

Table 7.4.2.1.8-1 describes the information flow for the MCData group standalone data request (in subclause 7.4.2.5.2) sent from the MCData server to the MCData server and from the MCData server to the MCData client.

Table 7.4.2.1.8-1: MCData group standalone data request (MCData server - MCData server and MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Payload | M | SDS content |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the payload is for application consumption. |  |  |

##### 7.4.2.1.9	MCData data disposition notification(s)

Table 7.4.2.1.9-1 describes the information flow for the MCData data disposition notification(s) sent from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to the MCData client.

Table 7.4.2.1.9-1: MCData data disposition notification(s)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user towards which the notification is sent |
| MCData ID | M | The identity of the MCData user sending notification |
| Conversation Identifier | M | Identifies the conversation |
| Disposition association | M | Identity of the original MCData transaction |
| Disposition | M | Disposition which is delivered or read or both |

##### 7.4.2.1.10	MCData group session standalone data request (MCData client – MCData server)

Table 7.4.2.1.10-1 describes the information flow for the MCData group session standalone data request (in subclause 7.4.2.6.2) sent from the MCData client to the MCData server.

Table 7.4.2.1.10-1: MCData group session standalone data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Standalone transaction |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI, attached data hosts) |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption or IP data in IP connectivity sessions are for data host consumption. |  |  |

##### 7.4.2.1.11	MCData group session standalone data request (MCData server – MCData server and MCData server – MCData client)

Table 7.4.2.1.11-1 describes the information flow for the MCData group session standalone data request (in subclause 7.4.2.6.2) sent from the MCData server to the MCData server and from the MCData server to the MCData client.

Table 7.4.2.1.11-1: MCData group session standalone data request (MCData server – MCData server and MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Standalone transaction |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI, attached data hosts) |
| SDP offer | M | Media parameters offered |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption or IP data in IP connectivity sessions are for data host consumption. |  |  |

##### 7.4.2.1.12	MCData group session standalone data response

Table 7.4.2.1.12-1 describes the information flow for the MCData group standalone data response (in subclause 7.4.2.6.2) sent from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.12-1: MCData group session standalone data response

| Information element | Status | Description |  |
| --- | --- | --- | --- |
| MCData ID | M | The identity of the MCData user receiving data |  |
| MCData group ID | M |  | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the MCData user sent data |  |
| Conversation Identifier | M | Identifies the conversation |  |
| SDP answer | M | Media parameters selected |  |

##### 7.4.2.1.13	MCData group data request (MCData client – MCData server)

Table 7.4.2.1.13-1 describes the information flow for the MCData group data request sent from the MCData client to the MCData server.

Table 7.4.2.1.13-1: MCData group data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Session based transactions |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.14	MCData group data request (MCData server – MCData server and MCData server – MCData client)

Table 7.4.2.1.14-1 describes the information flow for the MCData group data request sent from the MCData server to the MCData server and from the MCData server to the MCData client.

Table 7.4.2.1.14-1: MCData group data request (MCData server – MCData server and MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the recipient MCData user |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Session based transactions |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| SDP offer | M | Media parameters offered |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.4.2.1.15	MCData group data response

Table 7.4.2.1.15-1 describes the information flow for the MCData group data response sent from the MCData client to the MCData server from the MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.15-1: MCData group data response

| Information element | Status | Description |  |
| --- | --- | --- | --- |
| MCData ID | M | The identity of the MCData user receiving data |  |
| MCData group ID | M |  | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the MCData user sent data |  |
| Conversation Identifier | M | Identifies the conversation |  |
| SDP answer | M | Media parameters selected |  |

##### 7.4.2.1.16	MCData one-to-one SDS communication upgrade request

Table 7.4.2.1.16-1 describes the information flow for the MCData one-to-one SDS communication upgrade request sent from the MCData client to the MCData server from the MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.16-1: MCData one-to-one SDS communication upgrade request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data (when initiated by MCData client); The identity of the MCData user receiving data (when initiated by MCData server). |
| Functional alias | O | The associated functional alias of the MCData user sending data or receiving data. |
| Conversation Identifier | M | Identifies the conversation |
| Emergency indicator | M | Indicates that the data request is for MCData emergency communication |

##### 7.4.2.1.17	MCData one-to-one SDS communication upgrade response

Table 7.4.2.1.17-1 describes the information flow for the MCData one-to-one SDS communication upgrade response sent from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.17-1: MCData one-to-one SDS communication upgrade response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data (when initiated by MCData client); The identity of the MCData user receiving data (when initiated by MCData server). |
| Conversation Identifier | M | Identifies the conversation |

##### 7.4.2.1.18	MCData group SDS communication upgrade request

Table 7.4.2.1.18-1 describes the information flow for the MCData group SDS communication upgrade request sent from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.18-1: MCData group SDS communication upgrade request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending upgrade request |
| Functional alias | O | The associated functional alias of the MCData user sending data or receiving data. |
| MCData group ID | M | The MCData group ID on which the emergency upgrade request is made |
| Conversation Identifier | M | Identifies the conversation |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

Table 7.4.2.1.18-2: MCData group SDS communication upgrade request (MCData server to MCData server and MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending upgrade request |
| Functional alias | O | The associated functional alias of the MCData user sending data or receiving data. |
| MCData group ID | M | The MCData group ID on which the emergency upgrade request is made |
| MCData ID | M | The identity of the MCData user receiving the upgrade request |
| Conversation Identifier | M | Identifies the conversation |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

##### 7.4.2.1.19	MCData group SDS communication upgrade response

Table 7.4.2.1.19-1 describes the information flow for the MCData group SDS communication upgrade response sent from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.19-1: MCData group SDS communication upgrade response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data (when initiated by MCData client); The identify of the MCData user receiving data (when initiated by MCData server). |
| MCData group ID | M | The MCData group ID on which the emergency upgrade request is made |
| Conversation Identifier | M | Identifies the conversation |

##### 7.4.2.1.20	MCData group SDS communication in-progress priority state cancel request

Table 7.4.2.1.20-1 describes the information for the MCData group SDS communication in-progress priority state cancel request sent from the MCData client to the MCData server, from the MCData server to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.20-1: MCData group SDS communication in-progress priority state cancel request (MCData client to MCData server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the cancelling party |
| MCData group ID | M | The MCData group ID on which the MCData in-progress emergency state is to be cancelled. |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Conversation Identifier | M | Identifies the conversation |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

Table 7.4.2.1.20-2 MCData group SDS communication in-progress priority state cancel request (MCData server to MCData server and MCData server to MCData client)

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the cancelling party |
| MCData group ID | M | The MCData group ID on which the MCData in-progress emergency state is to be cancelled. |
| MCData ID | M | The identity of the recipient MCData user |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Conversation Identifier | M | Identifies the conversation |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

##### 7.4.2.1.21	MCData group SDS communication in-progress priority state cancel response

Table 7.4.2.1.21-1 describes the information flow for the MCData group SDS communication in-progress priority state cancel response sent from the MCData server to the MCData client.

Table 7.4.2.1.21-1: MCData group SDS communication in-progress priority state cancel response

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the cancelling party |
| MCData group ID | M | The MCData group ID on which the MCData in-progress emergency in-progress is to be cancelled. |
| Conversation Identifier | M | Identifies the conversation |

* * * End of Changes * * * *