3GPP TSG-SA WG6 Meeting #38-e	S6-201011

# e-meeting, 20th – 31th July 2020													(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0224 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update information flows to support interconnection for FD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2020-07-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Railway communication requires support interconnection for the file distribution service. Currently not all information flows are indicated as applicable for server to server information exchange. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds missing server to server information flows for the file distribution service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No server to server file distribution messages available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.1.8; 7.5.2.1.9, 7.5.2.1.10, 7.5.2.1.11, 7.5.2.1.12, 7.5.2.1.13, 7.5.2.1.21, 7.5.2.1.22, 7.5.2.1.23, 7.5.2.1.24, 7.5.2.1.25, 7.5.2.1.26, 7.5.2.1.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.5.2.1.8	MCData FD request (using media plane)

Table 7.5.2.1.8-1 describes the information flow for the MCData FD request (in subclause 7.5.2.5.2) sent from the MCData client to the MCData server and, from the MCData server to another MCData client and from an MCData server to a partner MCData server.

Table 7.5.2.1.8-1: MCData FD request (using media plane/MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| Functional alias | O | The functional alias associated with MCData user sending the file. |
| MCData ID (see NOTE) | O | The identity of the MCData user receiving file |
| Functional alias (see NOTE) | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download. (i.e. auto accept this media plane setup request) |
| SDP offer | M | Media parameters offered |
| Content reference | M | URL reference to the content and file metadata information |
| Requested priority | O | Application priority level requested for this communication session |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| NOTE: Either the MCData ID or the functional alias must be present. |  |  |

Table 7.5.2.1.8-2: MCData FD request (using media plane/MCData server to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| Functional alias | O | The associated functional alias of the MCData user identity sending the file. |
| MCData ID | M | The identity of the MCData user receiving file |
| Functional alias | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download. (i.e. auto accept this media plane setup request) |
| SDP offer | M | Media parameters offered |
| Content reference | M | URL reference to the content and file metadata information |
| Requested priority | O | Application priority level requested for this communication session |

Table 7.5.2.1.8-3: MCData FD request (using media plane/MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| Functional alias | O | The associated functional alias of the MCData user identity sending the file. |
| MCData ID | M | The identity of the MCData user receiving file |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download. (i.e. auto accept this media plane setup request) |
| SDP offer | M | Media parameters offered |
| Content reference | M | URL reference to the content and file metadata information |
| Requested priority | O | Application priority level requested for this communication session |

##### 7.5.2.1.9	MCData FD response (using media plane)

Table 7.5.2.1.9-1 describes the information flow for the MCData FD response (in subclause 7.5.2.5.2) sent from the MCData client to the MCData server, from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.9-1: MCData FD response (using media plane)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending FD request |
| MCData ID | M | The identity of the MCData user sending response |
| Conversation Identifier | M | Identifies the conversation |
| SDP answer | M | Media parameters selected |
| Establishment reason | O | Reason for establishment or rejection |

##### 7.5.2.1.10	MCData group standalone FD request (using HTTP)

Table 7.5.2.1.10-1 describes the information flow for the MCData group standalone FD request (in subclause 7.5.2.6.2) sent from the MCData client to the MCData server, from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.10-1: MCData group standalone FD request (using HTTP/MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| Functional alias | O | The functional alias associated with MCData user sending the file. |
| MCData group ID | M | The MCData group ID to which the file is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download |
| Content reference | M | URL reference to the content and file metadata information |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

Table 7.5.2.1.10-2: MCData group standalone FD request (using HTTP/MCData server to MCData client and MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| Functional alias | O | The functional alias associated with MCData user sending the file. |
| MCData group ID | M | The MCData group ID to which the file is to be sent |
| MCData ID | M | The identity of the MCData user receiving the file |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download |
| Content reference | M | URL reference to the content and file metadata information |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

##### 7.5.2.1.11	MCData group standalone FD response (using HTTP or MBMS download delivery method)

Table 7.5.2.1.11-1 describes the information flow for the MCData group standalone FD response (in subclause 7.5.2.6.2) sent from the MCData client to the MCData server, from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.11-1: MCData group standalone FD response (using HTTP)

| Information element | Status |  | Description |
| --- | --- | --- | --- |
| MCData ID | M |  | The identity of the MCData user sending FD request |
| MCData group ID | M | The MCData group ID to which the file is to be sent |  |
| MCData ID | M |  | The identity of the MCData user sending response |
| Conversation Identifier | M |  | Identifies the conversation |

##### 7.5.2.1.12	MCData group standalone FD request (using media plane)

Table 7.5.2.1.12-1 describes the information flow for the MCData group standalone FD request (in subclause 7.5.2.7.2) sent from the MCData client to the MCData server from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.12-1: MCData group standalone FD request (using media plane/MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| Functional alias | O | The functional alias associated with MCData user sending the file. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download. (i.e. auto accept this media plane setup request) |
| SDP offer | M | Media parameters offered |
| Content reference | M | URL reference to the content and file metadata information |
| Requested priority | O | Application priority level requested for this communication session |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

Table 7.5.2.1.12-2: MCData group standalone FD request (using media plane/MCData server to MCData client and MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| Functional alias | O | The functional alias associated with MCData user sending the file. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the MCData user receiving file |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | O | Indicates mandatory download. (i.e. auto accept this media plane setup request) |
| SDP offer | M | Media parameters offered |
| Content reference | M | URL reference to the content and file metadata information |
| Requested priority | O | Application priority level requested for this communication session |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

##### 7.5.2.1.13	MCData group standalone FD response (using media plane)

Table 7.5.2.1.13-1 describes the information flow for the MCData group standalone FD response (in subclause 7.5.2.7.2) sent from the MCData client to the MCData server, from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.13-1: MCData group standalone FD response (using media plane)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending FD request |
| MCData group ID | M | The MCData group ID to which the file is to be sent |
| MCData ID | M | The identity of the MCData user sending response |
| Conversation Identifier | M | Identifies the conversation |
| SDP answer | M | Media parameters selected |

* * * Next Change * * * *

##### 7.5.2.1.21	MCData group standalone FD over MBMS request

Table 7.5.2.1.21-1 describes the information flow for the MCData group standalone FD request (in subclause 7.5.2.6.2) sent from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.21-1: MCData group standalone FD over MBMS request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| MCData group ID | M | The MCData group ID to which the file is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | M | Indicates mandatory download |
| Content reference | M | URL reference to the content and file metadata information |
| MBMS user service id | M | Id of the MBMS user service delivering the file |
| MBMS content URI | M | URI upon which the content is delivered in the MBMS user service |

##### 7.5.2.1.22	MCData one-to-one FD upgrade request

Table 7.5.2.1.22-1 describes the information flow for the MCData one-to-one FD upgrade request sent from the MCData client to the MCData server from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.22-1: MCData one-to-one FD upgrade request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data (when initiated by MCData client); The identity of the MCData user receiving data (when initiated by MCData server). |
| Functional alias | O | The associated functional alias of the MCData user sending data or receiving data. |
| Conversation Identifier | M | Identifies the conversation |
| Emergency indicator | M | Indicates that the data request is for MCData emergency communication |

##### 7.5.2.1.23	MCData one-to-one FD upgrade response

Table 7.5.2.1.23-1 describes the information flow for the MCData one-to-one FD upgrade response sent from the MCData client to the MCData server from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.23-1: MCData one-to-one FD upgrade response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data (when initiated by MCData client); The identity of the MCData user receiving data (when initiated by MCData server). |
| Conversation Identifier | M | Identifies the conversation |

##### 7.5.2.1.24	MCData group FD upgrade request

Table 7.5.2.1.24-1 describes the information flow for the MCData group FD upgrade request sent from the MCData client to the MCData server from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.24-1: MCData group FD upgrade request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the upgrade request |
| Functional alias | O | The associated functional alias of the MCData user sending data or receiving data. |
| MCData group ID | M | The MCData group ID on which the emergency upgrade request is made |
| Conversation Identifier | M | Identifies the conversation |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

Table 7.5.2.1.24-2: MCData group FD upgrade request (MCData server to MCData server and MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the upgrade request |
| Functional alias | O | The associated functional alias of the MCData user sending data or receiving data. |
| MCData group ID | M | The MCData group ID on which the emergency upgrade request is made |
| MCData ID | M | The identity of the MCData user receiving the upgrade request |
| Conversation Identifier | M | Identifies the conversation |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

##### 7.5.2.1.25	MCData group FD upgrade response

Table 7.5.2.1.25-1 describes the information flow for the MCData group FD upgrade response sent from the MCData client to the MCData server from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.25-1: MCData group FD upgrade response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data (when initiated by MCData client); The identify of the MCData user receiving data (when initiated by MCData server). |
| MCData group ID | M | The MCData group ID on which the emergency upgrade request is made |
| Conversation Identifier | M | Identifies the conversation |

##### 7.5.2.1.26	MCData group FD in-progress priority state cancel request

Table 7.5.2.1.26-1 describes the information for the MCData group FD in-progress priority state cancel request sent from the MCData client to the MCData server from an MCData server to a partner MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.26-1: MCData group FD in-progress priority state cancel request (MCData client to MCData server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the cancelling MCData User |
| MCData group ID | M | The MCData group ID on which the MCData in-progress emergency state is to be cancelled. |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Conversation Identifier | M | Identifies the conversation |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

Table 7.5.2.1.26-2: MCData group FD in-progress priority state cancel request (MCData server to MCData server and MCData server to MCData client)

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the cancelling MCData User |
| MCData group ID | M | The MCData group ID on which the MCData in-progress emergency state is to be cancelled. |
| MCData ID | M | The identity of the MCData user receiving the cancel request |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Conversation Identifier | M | Identifies the conversation |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

##### 7.5.2.1.27	MCData group FD in-progress priority state cancel response

Table 7.5.2.1.27-1 describes the information flow for the MCData group FD in-progress priority state cancel response sent from an MCData server to a partner MCData server and from the MCData server to the MCData client.

Table 7.5.2.1.27-1: MCData group FD in-progress priority state cancel response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the cancelling party |
| MCData group ID | M | The MCData group ID on which the MCData in-progress emergency in-progress is to be cancelled. |
| Conversation Identifier | M | Identifies the conversation |

* * * End of Changes * * * *