3GPP TSG-SA WG6 Meeting #38-e                                                         S6-201013

e-meeting, 20th – 31th July 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0146 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the IE of priority in Information flows for transmission control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TD Tech |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCVideo |  |  |  |  |  | Date: |  |  | 2020-07-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the transmission control server receives a transmit media request from transmission control participant ,it decides wether to give a grant or not based on transmission control criteria.The priority should be included in transmission control criteria to indicate the importance of the transmission media.When the maximum media transmission limit for the MCVideo session is reached, transmission control server determines to accept the transmit media request which has a higher priority and decides to pre-empt the on-going video transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the IE of priority in transmit media request message and remote transmit media request message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors remain in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1.2.2, 7.7.1.2.17, 7.7.1.3.1, 7.7.1.3.3, 7.7.1.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.7.1.2.2      Transmit media request

Table 7.7.1.2.2-1 describes the information flow transmit media request, from the transmission control participant to the transmission control server, which is used to request the transmit media request. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.2-1: Transmit media request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose media transmission is requested |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Media priority | O | Priority of the media |

* * * End of Change * * * *

* * * Next Change * * * *

7.7.1.2.17      Remote transmit media request

Table 7.7.1.2.17-1 describes the information flow remote transmit media request, from the transmission control participant to the transmission control server. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.17-1: Remote transmit media request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user who remotely initiated the media transmission of another user. |
| MCVideo ID | M | Identity of the user whose media transmission is requested |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Media priority | O | Priority of the media |

* * * End of Change * * * *