3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201703

e-meeting, 12th – 20th October 2020	(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 1703 | rev | 17 | Current version: | 4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription to Group Location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Systems require location of group members of certain groups (where certain groups may have certain roles within the organization). Dispatcher may need to quickly and efficiently target certain resources by knowing these group member’ respective locations. See TS 22.280 [R-8.4.6-002] |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the location subscription / notification information elements to support group location subscription and notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current interface requries listing of each group members list in the location subscribe request. This list may be extermely large list of users . When the group changes, the location subscription has to be updated again with the updated member list. By introducing a group ID, the above use case may be optimized. |  |  |  |  |  |  |  |  |
|  |  | . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.5,10.9.2.7,10.9.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.9.2.5	Location information subscription request

Table 10.9.2.5-1 describes the information flow from the MC service server or location management client to the location management server for location information subscription request.

Table 10.9.2.5-1: Location information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the requesting MC service user |
| MC service ID list | O (see NOTE 2 | List of MC service users whose location information is requested. |
| MC service group ID | O (see NOTE 2) | MC service group ID for whose affiliated members location information is requested |
| Time between consecutive reports | M (see NOTE1) | Indicates the interval time between consecutive reports |
| NOTE1: If the interval time has a value of zero then the location management server will send the Location information notification immediately the location information report is received from the MC service user in the MC service ID list. NOTE 2: At least one of these elements shall be present |  |  |

* * * Next Change * * * *

#### 10.9.2.7	Location information notification

Table 10.9.2.7-1 describes the information flow from the location management server to the MC service server.

Table 10.9.2.7-1: Location information notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list | M | List of the MC service IDs (e.g. MCPTT ID, MCData ID, MCVideo ID) of the MC service users whose location information needs to be notified |
| MC service group ID | O | Identity of the MC service group ID for which location of MC service ID(s) is being notified |
| MC service ID | M | Identity of the MC service user subscribed to location information of another MC service user (see NOTE 1) |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information (see NOTE 2) | M | Location information |
| NOTE 1: This is only used when the location management server sends location information notification to the MC service user who has subscribed the location information. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

* * * Next Change * * * *

#### 10.9.2.8	Location information cancel subscription request

Table 10.9.2.8-1 describes the information flow from the MC service server or location management client to the location management server for location information cancel subscription request.

Table 10.9.2.8-1: Location information cancel subscription request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the requesting MC service user |
| MC service ID list | O (see NOTE) | List of MC service IDs (e.g. MCPTT ID, MCData ID, MCVideo ID) of the MC service users for whom location information is no longer required. |
| MC service group ID | O (see NOTE) | Identity of the MC service Group ID whose members’ location is no longer needed. |
| NOTE: At least one of these elements shall be present |  |  |