3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201794

e-meeting, 12th – 20th October 2020	(revision of S6-20xxxx)

Source:	Tencent

Title:	Pseudo-CR on key issue x: UAS media application session management

Spec:	3GPP TR 23.755

Agenda item:	8.5

Document for:	Approval

Contact:	shuaiizhao@tencent.com

1. Introduction

Certain UAV media applications have specified KPIs need to be fulfilled using a 5G system. Table 7.1-1 and 7.2-2 in TS 22.125 listed a few use cases where KPIs are mattered. In cases where live streaming is used such as live broadcasting, real-time video streaming, and remote UAV controller through HD video, the UAS application layer needs certain network infrastructure support to establish media sessions: such as NAT traversal, Media codec selection/negotiation using SDP, SIP signaling for live UAS control either by video or voice, etc.

Meanwhile, the stage 1 requirement also supports remote UAV control using video. Such interactive means of UAV control require specific network protocols to support it such as RTP/RTSP.

Currently, SEAL support signaling control using SIP and HTTP. However, there is no support on how to deal with media-centric UAS applications mentioned above.

2. Reason for Change

New KI to address media-centric UAV applications using a 3GPP network.

3. Conclusions

This proposal identifies a new KI relating to UAS media application session management

4. Proposal

It is proposed to agree on the following changes to 3GPP TR 23.755

* * * First Change * * * *

## 5.x 	Key issue #X:  UAS media application session management

Certain UAV applications have specified KPIs need to be fulfilled using a 5G system. Table 7.1-1 and 7.2-2 in TS 22.125 listed a few use cases where KPIs are mattered. In cases where live streaming is used such as live broadcasting, real-time video streaming, and remote UAV controller through HD video, the UAS application layer needs certain network infrastructure support to establish media sessions: such as NAT traversal, Media codec selection/negotiation using SDP, SIP signaling for live UAS control either by video or voice, etc.,

Meanwhile, the stage 1 requirement also supports remote UAV control using video. Such interactive means of UAV control require specific network protocols to support it such as RTP/RTSP.

Currently, SEAL support signaling control using SIP and HTTP. However, there is no support on how to deal with media-centric UAS applications mentioned above.

Hence, it is required to study the following:

- Whether and How SEAL can be used to fulfill stage 1 requirement for live media session management.

- How SEAL can be enhanced to support interactive media streaming