3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201857

e-meeting, 12th – 20th October 2020	(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0027 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Tracking UE and obtaining dynamic UE information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2020-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LMS is enhanced with features for tracking UE and obtaining dynamic UE information at a location. Such features are useful for advanced/remote driving mode with slow to moderate speed and deployed in areas like campus (e.g. autonomous shuttle vehicles), factories or ports (e.g. autonomous/remotely controlled guided vehicles). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding features for tracking UE and obtaining dynamic UE information at a location. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new features for tracking UE and obtaining dynamic UE information at a location cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.x	Tracking UE location

##### 9.3.x.1	Tracking UE location subscription

Figure 9.3.x.1-1 describes the procedure for tracking UE location subscription.

Figure 9.3.x.1-1: Tracking UE location subscription

1.	The VAL server sends track UE location request to the location management server. The request contains the VAL UE ID of the UE (which is to be tracked).

2.	The location management server initiates location reporting configuration with the location management client of the UE.

3.	The location management server may optionally subscribe for UE location information from 3GPP core network for the UE.

4.	The location management server determines the UE location information of the UE as received in steps 3 and 4.

5.	The location management sends track UE location response to the VAL server with the UE location information of the UE.

##### 9.3.x.2	Tracking UE location notification

Figure 9.3.x.2-1 describes the procedure for tracking UE location notification.

Pre-condition:

-	VAL server has performed the subscription procedure as specified in clause 9.3.x.1.

Figure 9.3.x.2-1: Tracking UE location notification

1.	The location management server receives location information of the UE (which is being tracked).

2.	The location management server may optionally receive the location information of the UE from 3GPP core network.

3.	The location management server determines the location information of UE as received in steps 2 and 3.

4.	The location management serveer sends UE location notification to the VAL server with the location information of the UE.

* * * Next Change * * * *

#### 9.3.y	Obtaining UE(s) information at a location

Figure 9.3.y-1 describes the procedure for obtaining UE(s) information at a location.

Pre-condition:

-	The VAL server has a jurisdiction over a geographical area for which the location management server is configured to operate.

-	The UE(s) in the geographical area have provided its location information to the location management server.

Figure 9.3.y-1: Obtaining UE(s) information at a location

1.	The VAL server sends get UE infomation request to the location management server. The request contains a location information and application defined proximity range.

2.	The location management server determines the UE(s) whose location are within the application defined proximity range of the location information provided in step 1.

3.	The location management server sends get UE information response to the VAL server with a list of UE(s) and its corresponding location information as determined in step 1.