3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201889

e-meeting, 12th – 20th October 2020	(revision of S6-201740)

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on connection authorisation

Spec:	3GPP TR 23.700-79 version 0.1.0

Agenda item:	8.7

Document for:	Approval

Contact:	Martin Öttl, martin.oettl@nokia.com

1. Introduction

A control mechanism for connecting non-3GPP devices with gateway UE functions is needed as not all devices should be connected with all gateway UE functions. An authorized usage of gateway UE functions by clients and users is needed as well as a control mechanism on how many gateway UE functions can be used by the same MC service user/client.

2. Reason for Change

Adding a solution for authorisation for connection of non-3GPP devices with an MC gateway UE.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-79 version 0.1.0.

* * * First Change * * * *

## 7.x	Connection authorisation with the MC gateway UE

### 7.x.1	General

This solution addresses the key issue 2 described in clause 5.2 on authorisation for connection of non-3GPP devices with an MC gateway UE. The solution only applies to non-3GPP devices which can host an MC client.

### 7.x.2	Information flows

#### 7.x.2.1	Connection authorization request

Table 7.x.2.1-1 describes the information flow connection authorization request sent from the MC client, which resides on a non-3GPP device, to the MC gateway UE.

Table 7.x.2.1-1: Connection authorization request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user. |

#### 7.x.2.2	Connection authorization response

Table 7.x.2.2-1 describes the information flow connection authorization response sent from the MC gateway UE to the MC client.

Table 7.x.2.2-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user. |
| Result | M | Success or failure of the connection authorization request. |

### 7.x.3	Procedure

The procedure for connection authorisation with an MC gateway UE is shown in figure 7.x.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services by using a non-3GPP device.

-	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC client has been provided with an appropriate MC service ID.

Editor's note:	Whether separate MC gateway UE authentication with the MC server and specific MC gateway UE configuration is needed requires further study.

Editor's note: How the MC client discovers the MC gateway UE is FFS.

Figure 7.x.3-1: Connection authorisation with the MC gateway UE

1.	The MC client requests connection authorization with the MC gateway UE. The MC client of the MC service user provides the MC service ID.

2.	The MC gateway UE performs an authorization check by using the provided MC service ID to verify that the MC service user is permitted to use the MC gateway UE to access MC services.

NOTE:	The criteria for the authorization check is outside the scope of the present document, but for example could be based on a pre-configured list of users who are expected to request connection authorization with the MC gateway UE.

3.	The MC gateway UE sends the connection authorization response to the MC client, confirming successful connection with the MC gateway UE.

The MC client has now access to an MC server and may continue with user authentication and service authorization.

### 7.x.4	Solution evaluation

Editor's note:	This clause will provide the evaluation of this solution.

* * * End of Change * * * *