3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201904

e-meeting, 12th – 20th October 2020	(revision of S6-201820)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.764 | CR | 0001 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to solution #16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eV2XAPP |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Solution #16 uses the Network monitoring information that is directly inherited from TS 23.286 for LTE. For 5GS the parameter “Overload level” is not provided. For the parameter “Congestion level”, according to TS 23.502: Based on the user data congestion analytics information the NEF receives from the NWDAF, the NEF derives and reports the network status for the geographical area or for the UE as Network Status Result (NSR) to the AF. When reporting to the AF, the NSR shall not include any 3GPP location information. NOTE 1: Either exact values for congestion status, as reported by NWDAF to NEF or abstracted values e.g. (High, Medium, Low) can be reported by the NEF to the AF. The calculation and the reporting of the NSR to the AF depends on operator configuration (e.g. SLAs, usage etc.). Hence for 5G case the congestion values can be abstracted or relevantly concrete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in the usage of congestion level and overload level. Change the parameter name to congestion informaiton. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misuse of paramters with 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.16.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.16.1	Solution description

This solution addresses the key issue #10.

The solution enhances the procedure specified in clause 9.7 of 3GPP TS 23.286 [7] for the V2X UE to receive the network monitoring information (e.g. downgrade event notifications) with 5GC events. This information can be used for switching modes and application behaviour adaptation.

The changes to clause 9.7.4 of 3GPP TS 23.286 [7] are highlighted in bold italics as follows:

9.7.4 Notifications for network monitoring information 9.7.4.1 General This subclause describes the procedure for notification of network monitoring information from the VAE server. 9.7.4.2 Procedure Figure 9.7.4.2-1 illustrates the procedure where the VAE server sends notification of the network monitoring information to the V2X UEs. Pre-conditions: 1. The VAE server acting as a SCS/AS is configured with the SCEF information and is authorized to exchange information with the SCEF as specified in subclause 4.9 in 3GPP TS 23.682 [8]. The VAE server has subscribed to QoS notification service from 5GS (e.g. PCF/NWDAF). The notification may either include the request for QoS sustainability events as specified in 3GPP TS 23.288 [9] or can include a QoS change notification requests as provided by SMF and specified in 3GPP TS 23.287 [8]; 2. The V2X UE1 and V2X UE2 have subscribed for the network monitoring information at the VAE server. Figure 9.7.4.2-1: Notifications for network monitoring information 1. The VAE server communicates with the SCEF to receive network monitoring information as per the procedure in subclause 4.9 of 3GPP TS .23.682 [8]. The VAE server subscription to QoS monitoring service from 5GS (e.g. PCF/NWDAF) may be active for a certain period of time or a given geographical area. The monitoring may either include the request for QoS sustainability events as specified in 3GPP TS 23.288 [9], or can include a QoS change notification requests as provided by SMF and specified in 3GPP TS 23.287 [8]. The reporting may be configured by the application enabler layer for a given area, time, periodicity etc taking into account the service requirement and other parameters (e.g. expected congestion in certain area, time of the day, road conditions). Based on the subscription, as specified in 3GPP TS 23.287 [8], 5GS provides the extended QoS monitoring report, over N33 interface. This report may come either from NWDAF or SMF via PCF/NEF. 2. The network monitoring information (e.g. uplink or downlink link degradations, congestions, overload) available at the VAE server about the on-going V2V session(s) is based on the original network monitoring information provided by the SCEF, and QoS monitoring report from 5GS as specified in step 1. The VAE server determines the network monitoring information to send to the UE. 3, The monitoring information is sent to the subscribed V2X UEs via network monitoring information notification.

The changes to clause 9.7.2.3 of 3GPP TS 23.286 [7] are highlighted in bold italics as follows:

9.7.2.3 Network monitoring information notification Table 9.7.2.3-1 describes the information flow network monitoring information notification from the VAE server to the VAE client. Table 9.7.2.3-1: Network monitoring information notification Information element; Status; Description. V2X UE ID; M; Identity of the subscribed V2X UE. Network monitoring information; M; This includes information on network status for the triggering criteria, and may include the following parameters: - Uplink quality level - Congestion information (NOTE) - Geographical Area (cell area or TA for which the monitoring applies) - Time Validity (the period for which the monitoring applies) - Coverage level and bearer level events (optionally, for MBMS) - QoS Sustainability. NOTE : The congestion information may be exact values for congestion status reported by NWDAF to NEF or abstracted values e.g. (High, Medium, Low) which can be reported by the NEF to the AF.. Editor’s note: It’s FFS if the slice load level can be supported in Rel-17.