3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201756

e-meeting, 12th – 20th October 2020	(revision of S6-20xxxx)

Source:	Sony

Title:	Pseudo-CR on ECS Discovery from 5GC

Spec:	3GPP TS 23.558

Agenda item:	7.6

Document for:	Approval

Contact:	Svante Alnås, Svante.Alnas@Sony.com

2. Reason for Change

SA2 has not finalized theirs study and has not started the normative work for supporting Edge Enabling layer in the core network. So far they have not specified a solution that allows deployment of EEC and AC on existing smartphones. Currently the different solutions require modifications in the mobile OS/kernel or DNS Client etc. according to TS 23.748. We have specified 3 ways for the EEC to discover the ECS:

- Preconfigured in EEC or AC

- From 5GC and

- an FFS if it is possible to derive that from H-PLMN/V-PLMN

3. Conclusions

The first solution is an existing solution that also allows deployment of AC and EEC’s on existing smartphone OS. At least this solution should be allowed to be used by the EEC regardless of if the Mobile OS/kernel is updated or not so it is possible for the EEC to retrieve the ECS configuration from 5GC. This allows deployment of EEC’s without dependencies that mobile OS/kernel etc. supports the new feature.

Re1: Updated according to proposal from Catalina

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 1.0.0

* * * First Change * * * *

#### 8.3.2.2	Edge Configuration Server discovery

ECS address information can be pre-configured with the EEC, configured by an edge-aware Application Client, configured by the user, provisioned by MNO through 5GC procedure, or derived from HPLMN identifier for non-roaming scenario or from VPLMN identifier for roaming scenario.

Editor’s Note: 5GC provision of ECS configuration information to the UE is in scope of SA2.

NOTE: How the ECS address information is configured to the EEC by the AC, user, or pre-configuration is out of scope of the present specification.

When multiple ECSs are discovered by or provisioned to a specific EEC, how the EEC selects the ECS(s) for targeting Service Provisioning requests is outside the scope of this specification

Editor’s Note: It is FFS whether ECS address can be derived from HPLMN or VPLMN identifier when no ECS address information is available.

* * * Next Change * * * *