3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201799

e-meeting, 12th – 20th October 2020	(revision of S6-20xxxx)

Source:	Samsung Electronics, AT&T, Deutsche Telekom, Intel

Title:	EDGE-1 and EDGE-4 APIs

Spec:	3GPP TS 23.558 v1.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

The contribution provides API summary of the procedures on EDGE-1 and EDGE-4 interface.

2. Reason for Change

Relationship between the UE functional entity (EEC) and network functional entities (EES and ECS) in EDGEAPP is fairly heterogeneous:

- EEC can be developed by – Application providers, ECSPs, OEM vendors, OS vendors

- EES can be developed by – ECSPs, OEM vendors

- ECS can be developed by – ECSPs, OEM vendors

In a typical deployment, these functional entities will be provided by different vendors. Given the involvement of multiple and possibly unrelated developers, development of these functional entities cannot be tightly coordinated. Therefore providing APIs exposed by the ECS and EES, towards the EEC will help ease the development complexities.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.0.0.

* * * First Change * * * *

#### 8.3.4	APIs

##### 8.3.4.1	General

Table 8.3.4.1-1 illustrates the API for service provisioning.

Table 8.3.4.1-1: Service provisioning API

| API Name | API Operations | Operation Semantics | Known Consumer(s) |
| --- | --- | --- | --- |
| EDGE_Service_Provisioning API | Request | Request/Response | Edge Enabler Client |
|  | Subscribe | Subscribe/Notify | Edge Enabler Client |
|  | Notify | Subscribe/Notify | Edge Enabler Client |
|  | Subscription_update | Subscribe/Notify | Edge Enabler Client |
|  | Unsubscribe | Subscribe/Notify | Edge Enabler Client |

##### 8.3.4.2	Request operation

API operation name: Request

Description: The consumer requests for one time service provisioning information.

Inputs: See clause 8.3.3.1.

Outputs: See clause 8.3.3.2.

See clause 8.3.2.3 for details of usage of this operation.

##### 8.3.4.3	Subscribe operation

API operation name: Subscribe

Description: The consumer subscribes for service provisioning information.

Inputs: See clause 8.3.3.3.

Outputs: See clause 8.3.3.4.

See clause 8.3.2.4.2 for details of usage of this operation.

##### 8.3.4.4	Notify operation

API operation name: Notify

Description: The consumer is notified with service provisioning information.

Inputs: See clause 8.3.3.5.

Outputs: None.

See clause 8.3.2.4.3 for details of usage of this operation.

##### 8.3.4.5	Subscription_update operation

API operation name: Subscription_update

Description: The consumer updates an existing subscription for service provisioning information.

Inputs: See clause 8.3.3.6.

Outputs: See clause 8.3.3.7.

See clause 8.3.2.4.4 for details of usage of this operation.

##### 8.3.4.6	Unsubscribe operation

API operation name: Unsubscribe

Description: The consumer cancels an existing subscription for service provisioning information.

Inputs: See clause 8.3.3.8.

Outputs: See clause 8.3.3.9.

See clause 8.3.2.4.5 for details of usage of this operation.

* * * Next Change * * * *

#### 8.4.2.4	APIs

##### 8.4.2.4.1	General

Table 8.4.2.4.1-1 illustrates the API for EEC registration.

Table 8.4.2.4.1-1: EEC registration API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| EDGE_EEC_registration API | Request | Request/Response | Edge Enabler Client |
|  | Registration_update | Request/Response | Edge Enabler Client |
|  | Deregistration | Request/Response | Edge Enabler Client |

##### 8.4.2.4.2	Request operation

API operation name: Request

Description: The consumer requests to register the EEC on the EES.

Inputs: See clause 8.4.2.3.1.

Outputs: See clause 8.4.2.3.2.

See clause 8.4.2.2.2 for details of usage of this operation.

##### 8.4.2.4.3	Registration_update operation

API operation name: Registration_update

Description: The consumer requests to update the registered information of the EEC on the EES.

Inputs: See clause 8.4.2.3.3.

Outputs: See clause 8.4.2.3.4.

See clause 8.4.2.2.3 for details of usage of this operation.

##### 8.4.2.4.4	Deregistration operation

API operation name: Deregistration

Description: The consumer requests to de-register the EEC from the EES.

Inputs: See clause 8.4.2.3.5.

Outputs: See clause 8.4.2.3.6.

See clause 8.4.2.2.4 for details of usage of this operation.

* * * Next Change * * * *

#### 8.5.4	APIs

##### 8.5.4.1	General

Table 8.5.4.1-1 illustrates the API for EAS discovery.

Table 8.5.4.1-1: EAS discovery API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| EDGE_EAS_discovery API | Request | Request/Response | Edge Enabler Client |

##### 8.5.4.2	Request operation

API operation name: Request

Description: The consumer requests for one time EAS discovery information.

Inputs: See clause 8.5.3.1.

Outputs: See clause 8.5.3.2.

See clause 8.5.2 for details of usage of this operation.