3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201806

e-meeting, 12th – 20th October 2020	(revision of S6-20xxxx)

Source:	Samsung Electronics

Title:	Target EES and target EAS selection

Spec:	3GPP TS 23.558 v1.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution closes the following ENs related to target EES and target EAS selection:

Editor’s Note:	How Edge Enabler Server is selected from the list of Edge Enabler servers and whether the EEC needs to be notified when this selection happens is for further study.

Editor’s Note:	How the target Edge Application Server is selected from the list of Edge Application Servers received in step 4 is for further study.

2. Reason for Change

On receiving the discover target Edge Application Server request from the Edge Application Server, the source Edge Enabler Server discovers the potential target Edge Enabler Servers and further the potential target Edge Application Servers.

Note that:

-	The source EES is not aware of the parameters used by the Edge Enabler Client for initial selection of the source EES and source EAS (e.g. AC preferences/requirements, EEC/user preferences etc.)

-	Source Edge Enabler Server can choose the target Edge Enabler Server provided by the same ECSP as its own, however it locks the user within the offerings of the initially selected ECSP. Also, in typical deployments, potential target EES(s) available at UE's location may not have an EES provided by the same ECSP which offers the source EES.

-	Source Edge Enabler Server can choose the target Edge Application Server with the same characteristics (e.g. available KPIs) as the source EAS, but in typical deployments, a target EAS(s) with same characteristics may not be available at UE's location.

Given the above constraints, the source Edge Enabler Server may not be the best entity to select the target EES and target EAS for application context relocation.

This contribution proposes that the EEC should select the target EES and target EAS from the list of potential targets discovered by the source EES. This allows selection of the best target EES and EAS taking into account the requirements and preferences of the AC and the EEC/user.

The selection request also acts as early indications for imminent application context relocation, using which the AC can adjust user offerings as required or throttle back its communication towards the S-EAS until relocation is completed.

Also, in cases where the source EES can effectively select the target EES and target EAS (e.g. an EES from the same ECSP as the source EES and an EAS with same characteristics as the source EAS are available), it can share only the selected targets in the requests; effectively informing the EEC of the selected targets.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.0.0.

* * * First Change * * * *

#### 8.8.3.2	Discover target Edge Application Server

Figure 8.8.3.2-1 illustrates the interactions between the source Edge Application Server and the source Edge Enabler Server and optionally between the source Edge Enabler Server and the Edge Configuration Server and between the source Edge Enabler Server and target Edge Enabler Server for fetching target Edge Application Server information. This procedure may be utilized by a source Edge Application Server which undertakes the transfer of application context information to a target Edge Application Server directly.

Pre-conditions:

1.	Information related to the Edge Enabler Server is available with the source Edge Application Server.

Figure 8.8.3.2-1: Discover target Edge Application Server

1.	The Edge Application Server invokes the Edge Application Server Discovery request on the source Edge Enabler Server. The Edge Application Server discovery request includes the requestor identifier [EAS ID] along with the security credentials and includes EAS discovery filter matching its Edge application Server profile. If target DNAI is available at the source Edge Application Server via UP path management event notification, the source Edge Application Server provides the source Edge Enabler Server with the target DNAI.

NOTE:	The trigger condition to invoke the Discover target Edge Application Server API is up to application service logic, which is out of scope of this specification.

2.	Upon receiving the request, the source Edge Enabler Server checks if the requesting Edge Application Server is authorized to perform the discovery operation. If it is authorized, the source Edge Enabler Server checks if there exists a target Edge Application Server information (registered or cached) that can satisfy the requesting Edge Application Server information and additional query filters. If the source Edge Enable Server discovers the target Edge Application Server(s), the flow continues with step 5, else the source Edge Enabler Server retrieves the potential target Edge Enabler Server address(es) from the Edge Configuration Server as specified in clause 8.8.3.3.

3.	The source Edge Enabler Server invokes the Edge Application Server Discovery request on the potential target Edge Enabler Server(s) received in step 2. The source Edge Enabler Server discovery request includes the requestor identifier [EES ID] along with the security credentials and includes EAS discovery filter obtained in step 1.

NOTE 1:	If the source EES requires to select the target EES on its own, it sends only the EAS discovery message only to the selected target EES.

4.	The potential target Edge Enabler Server(s) discover the potential target Edge Application Server(s) and responds with the discovered Edge Application Server information to the source Edge Enabler Server. The source Edge Enabler Server may cache the Edge Application Server information.

5.	The source Edge Enabler Server sends the target EAS selection request to the Edge Enabler Client. The request includes the list or subset of the list of the potential target Edge Application Servers and related EES information retrieved in step 4.

NOTE 2:	If the source EES requires to select the target EAS on its own, it sends only the selected target EAS information to the Edge Enabler Client in step 5.

NOTE 3:	The target EAS selection request acts as a notification of imminent application context relocation to the Edge Enabler Client, which can inform the Application Client for necessary actions. The actions taken by the Application Client on receiving this notification are out of scope.

6.	The Edge Enabler Client on receiving the target EAS selection request, selects the target EAS from the received list of potential target EASs. The Edge Enabler Client sends the target EAS selection response to the source Edge Enabler Server, including the information of the selected target EAS.

7.	The source Edge Enabler Server responds to the Edge Application Server with the selected target Edge Application Server information.