3GPP TSG-SA WG6 Meeting #39-bis-e	S6-20xxxx

e-meeting, 12th – 20th October 2020	(revision of S6-201919)

Source:	Ericsson

Title:	EAS DNAI in EES profile and selected EAS

Spec:	3GPP TS 23.558 v1.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution introduces EAS DNAI in EES profile and EAS info in service provisioning subscription.

2. Reason for Change

The EAS DNAI may be sent from EAS to EES during EAS registration procedure and may be used to discover the candidate target EAS(es) in the EAS discovery procedure triggered by the network. In the EAS discovery procedure as specified in clause 8.8.2, ECS may be involved to offer the candidate EES(es) which can serve the EAS ID and such EAS DNAI (if provided in the request towards the ECS) is useful to narrow down the number of candidate EES(es) during the query by matching the EAS DNAI stored in the ECS:

[Proposal-01] Therefore, the EAS DNAI is useful for ECS and should be added in EES profile.

3. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

### 8.2.6	Edge Enabler Server Profile

The Edge Enabler Server profile includes information about the EES and the services it provides.

Table 8.2.6-1: Edge Enabler Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EES Provider Identifier | O | The identifier of the EES Provider (such as ECSP) |
| EAS information | M | List of EAS information |
| > EAS ID | M | Edge Application Server ID registered with the EES. |
| > list of EAS DNAI(s) | O | DNAI(s) associated to the EAS. This IE may be used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |

Editor's Note: Whether there is a need to specify a Service Area for the EES is FFS.

g subscription request

Table 8.3.3.6-1: Service provisioning subscription update request

* * * End of Changes * * * *