3GPP TSG-SA WG6 Meeting #39 BIS-e	S6- 201755

e-meeting, 12nd– 20th October 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	User plane connectivity modification in S-EES executed ACR

Spec:	3GPP TS 23.558 v1.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

It has been discussed how user plane connectivity for the UE and AC is modified in the application context relocation procedure.

2. Reason for Change

This pCR is proposed to resolve the following ENs :

Editor’s Note:	How the EEC modifies user plane connectivity for the AC is FFS.

It is out of scope the EEC modifies user plane connectivity for the AC. This is up to implementation of EEC and AC.

Editor's Note:	How the EES influences 3GPP Core Network in this procedure to modify user plane connectivity for the UE is FFS.

If the UE is connected to the 5GC, the EES acting as AF may influence 3GPP Core Network after receiving the notification for UP path management event. The timing of AF request to influence traffic routing varies with the followings: (i) type of subscription to UP path management event notification service, (ii) the use of indication of EAS acknowledgement, (iii) local configuration in the EES.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.0.0.

* * * First Change * * * *

## 8.8	Service continuity

### 8.8.1	General

When a UE moves to a new location, different Edge Application Servers can be more suitable for serving the Application Clients in the UE. Such transitions can result from a non-mobility event also, requiring support from the enabling layer to maintain the continuity of the service.

This clause describes the features that support service continuity for Application Clients in the UE to minimize service interruption while replacing the serving Edge Application Server, called source Edge Application Server, with a target Edge Application Server.

Generally, the source Edge Application Server is associated with an application context. To support service continuity, this application context from the source Edge Application Server is transferred to a target Edge Application Server.

The capabilities for supporting service continuity provided at the Edge Enabler Layer may consider various application layer scenarios in which there may be involvement of Application Client and one or more Edge Application Server(s).

The following scenarios are supported for service continuity:

-	UE mobility, including predictive or expected UE mobility for the following cases:

-	inter-EDN UE mobility; and

-	LADN-related UE mobility.

Editor's note:	It is FFS whether intra-EDN UE mobility is an applicable scenario under application service continuity.

-	Overload situations in source Edge Application Server or EDN for the following cases:

-	intra-EDN;

-	inter-EDN; and

-	LADN-related.

Editor's Note: How the architecture is affected by load balancing (i.e. EAS Proxy) or multiple reachable EAS instances is FFS.

-	Maintenance aspects for the following case:

-	graceful shutdown of an Edge Application Server.

To support the need of application context relocation, following entity roles are identified:

-	detection entity, detecting the need of application context relocation;

-	decision-making entity, deciding that the application context relocation is required; and

-	execution entity, executing application context relocation.

A detection entity detects the probable need for application context relocation by monitoring various aspects, such as UE's location and indicates to the decision-making entity to determine if the application context relocation is required. The following entities can potentially perform the detection role:

-	Application Client;

-	Edge Enabler Client;

-	Edge Enabler Server; and

-	Edge Application Server.

A decision-making entity determines that application context relocation is required and instructs the execution entity to perform application context relocation.

An execution entity performs application context relocation as and when instructed by the decision-making entity.

Editor's note:	It is FFS if there is more than one entity involved in the decision-making, and if decision-making is a distributed process.

The Edge Application Server may utilize the following capabilities provided by the Edge Enabler Server for supporting service continuity at the application layer:

-	Provide notifications of service continuity related events;

-	Fetch the target Edge Application Server; and

-	Application context relocation from a source Edge Application Server to a target Edge Application Server.

Editor's note:	The details of the APIs for the above capabilities is FFS. Each of the above capabilities to the Edge Application Server may be provided as individual API, or combined to be offered as a new service API with higher level of abstraction. Example, application context relocation service API which fulfils all the above capabilities to hide the implementation complexity from the application layer.

The Edge Enabler Server can utilize the following capabilities provided by the Edge Configuration Server for supporting service continuity at the application layer:

-	Fetch the target Edge Enabler Server.

The Edge Enabler Client may determine if the application context relocation is required by detecting that the UE moved outside the service area (i.e., EAS service area if the EAS is not global accessible and/or EDN service area if LADN is configured). For the PDU Session of SSC mode 3, if the UE receives PDU Session Modification Command as specified in clause 4.3.5.2 of 3GPP TS 23.502 [3], the Edge Enabler Client may determine that the application context relocation is required. For IPv6 multi-homed PDU Session of SSC mode 3, the Edge Enabler Client may determine that application context relocation is required if the UE is notified of the existence and availability of a new IPv6 prefix as specified in clause 4.3.5.3 of 3GPP TS 23.502 [3].

NOTE:	For IPv6 multi-homed PDU Session of SSC mode 3, the Edge Enabler Client can be aware of the notification about the IPv6 prefix configuration due to change of PSA UPF based on the UE implementation.

Editor's note:	It is FFS whether and what additional service area information can be used by the UE to determine if the application context relocation is required.

After successful application context relocation:

-	The Edge Enabler Server is informed of the completion by the Edge Application Server; and

-	The Edge Enabler Client is informed of the completion by the Edge Enabler Server.

Editor's note:	It is FFS what information is required in the Application Context Relocation completion. It is also FFS when and which entity triggers the AF influence for the EAS traffic in the 3GPP CN.

In general, a number of steps are required in order to perform the application context relocation procedure. The potential roles of an edge enablement layer in the application context relocation procedure include:

-	providing detection events;

-	selecting the target Edge Application Server(s); and

-	supporting the transfer of the Application context from the source Edge Application Server(s) to the target Edge Application Server(s).

Editor's note:	It is FFS whether the potential roles of an edge enablement layer include the cutover of application traffic from the source Edge Application Server(s) to the target Edge Application Server(s).

If the UE is connected to the 5GC, the EES/EAS acting as AF may influence EAS traffic routing in the 3GPP Core Network. Such AF influence may be in runtime coordination after receiving the notification for UP path management event (early and/or late) or pre-provisioned, according to type of subscription to UP path management event notification service, the use of indication of EAS acknowledgement, or local configuration in the EES/EAS.

A high level overview of application context relocation is illustrated in Figure 8.8.1-1.

Figure 8.8.1-1: High level overview of application context relocation procedure