3GPP TSG-SA WG6 Meeting #39-e	S6-201510

# e-meeting, 31st August – 8th September 2020								(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0026 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NRM unicast procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  | Date: |  |  | 2020-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There exists unicast procedures with SIP core for "Request for unicast resources at VAL service communication establishment" and the corresponding procedure for without SIP core is missing. For V2XAPP, the request for unicast resources without SIP core at VAL service communication establishment is required and the network resource adaptation procedure does not fulfill unicast resource request at communication establishment. It is also required to modify the network resource adapation procedure as "Request for modification of unicast resources" to align with similar procedure for unicast with SIP core. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing request for unicast resources procedure without SIP core and align the modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 implementation will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.1, 14.3.2.2, 14.3.2.8, 14.3.2.9, 14.3.3.3.0 (new), 14.3.3.3.0.1 (new), 14.3.3.3.0.2 (new), 14.3.3.3.1, 14.3.3.3.1.1, 14.3.3.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.3.2.1	Void

#### 14.3.2.2	Void

* * * Next Change * * * *

#### 14.3.2.8	Resource modification request

Table 14.3.2.8-1 describes the information flow for the resource modification request from VAL server to NRM server for unicast resources.

Table 14.3.2.8-1: Resource modification request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL user ID(s) or VAL UE ID(s) | M (NOTE) | List consisting of one or more VAL UE IDs for whom the resource modification is requested. |
| VAL group ID | M (NOTE) | The VAL group ID for whom the resource modification is requested. |
| VAL service requirement information | M | VAL service requirements for unicast resource (e.g. Bitrate, bandwidth, resource) and corresponds to the VAL service QoS requirements as applied for a UE or group of UEs. |
| NOTE: Either of the information elements should be present. |  |  |

#### 14.3.2.9	Resource modification response

Table 14.3.2.9-1 describes the information flow for the resource modification response from NRM server to VAL server for unicast resources.

Table 14.3.2.9-1: Resource modification response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the resource modification operation. The response can also include an updated value for some of the parameters included in the network resource modification request (e.g. negotiation of resource offering) |

* * * Next Change * * * *

##### 14.3.3.3.0	Request for unicast resources at VAL service communication establishment

###### 14.3.3.3.0.1	General

The procedure defined in this subclause specifies how network resources are requested at VAL service communication establishment. If concurrent sessions are used the NRM server may utilize the capability of resource sharing specified for underlying network policy and charging functions. The request for resources includes application type, bandwidth, priority, application identifier and resource sharing information.

###### 14.3.3.3.0.2	Procedure

The procedure is generic to any type of session establishment that requires requests for network resources.

Procedures in figure 14.3.3.3.0.2-1 are the signalling procedures for the requesting resource at session establishment.

Pre-condition:

-	The VAL client has requested VAL service communication with the VAL server.

Figure 14.3.3.3.0.2-1: Resource request at VAL service communication establishment

1.	The VAL server sends request for resources to the NRM server.

2.	The NRM server evaluates the need for network resources and use of resource sharing.

3.	PCC procedures are initiated from NRM server.

4.	The NRM server sends a resource response to the VAL server.

5.	The VAL service communication is established and resources have been allocated.

##### 14.3.3.3.1	Request for modification of unicast resources

###### 14.3.3.3.1.1	General

This subclause describes the procedure for request for modification of unicast resources using PCC procedures. This procedure satisfies the requirements for requesting unicast resources and modification to already allocated unicast resources to VAL communications.

###### 14.3.3.3.1.2	Procedure

Figure 14.3.3.3.1.2-1 illustrates the procedure for the request for modification of unicast resources.

Figure 14.3.3.3.1.2-1: Network resource adaptation

1.	The VAL server sends a resource modification request to the NRM server for one or more users belonging to one or more VAL services, and may comprise one or more VAL UEs that will have updated resource requirement. This requirement may be in the form of exact resources /resource pools to be used or indication of bandwidth increase/decrease for the corresponding VAL UEs or set of VAL UEs.

2.	The NRM server processes the request and applies / enforces the resource modification/adaptation per VAL UE.

3.	The NRM server initiates the PCC procedures for each VAL UE.

4.	The NRM server provides a resource modification response to the VAL server, providing information on the fulfilment of the resource modification request. This will include information either per VAL UE or per set of VAL UEs, as indicated by the request of the VAL server in step 1.