3GPP TSG-SA WG6 Meeting #39-e	S6-201xxx

- E-meeting, 31st Aug. – 8th Sep. 2020										(revision of S6-201446, 1593)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0025 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct SEAL location API operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  |  | Date: |  | 2020-08-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For on-demand location retrieval procedure defined in cl.9.3.9, current procedure requires the LMS to retrieve target LMC location as defined in cl.9.3.4 which involves notification to the user and asking for permission to share its location. This process may be time consuming, that is why cl.9.3.4 uses a separate operation to notify the LMS for the location report. In addtion, CR0023 agreed in 38e meeting is missing the update for API operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make the user permission option in clause 9.3.4. For V2XAPP it is not required that user permission is mandatory. Add more operations for SS_LocationReporting API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect operation semantic for on-demand location retrieval leads to failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4, 9.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.3.4	On-demand location reporting procedure

The location management server can request UE location information at any time by sending a location information request to the location management client, which may trigger location management client to immediately send the location report.

Figure 9.3.4-1: On-demand location information reporting procedure

1.	Based on configurations such as periodical location information timer, or location information request from other entities (e.g., another location management client, VAL server), location management server initiates the immediately request location information from the location management client.

2.	The location management server sends a location information request to the location management client.

3.	VAL user or VAL UE may be notified and asked about the permission to share its location. VAL user can accept or deny the request

4.	The location management client immediately responds to the location management server with a report containing location information identified by the location management server and available to the location management client.

5.	Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client.

* * * Next Change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting | VAL server | Request /Response |
|  | Update_Trigger_Location_Reporting |  |  |
|  | Cancel_Trigger_Location_Reporting |  |  |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |

* * * End of Changes * * * *