3GPP TSG-SA WG6 Meeting #39-e	S6-201446

- E-meeting, 31st Aug. – 8th Sep. 2020											(revision of S6-200xxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0025 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct SEAL location API operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  |  | Date: |  | 2020-08-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For on-demand location retrieval procedure defined in cl.9.3.9, current procedure requires the LMS to retrieve target LMC location as defined in cl.9.3.4 which involves notification to the user and asking for permission to share its location. This process may be time consuming, that is why cl.9.3.4 uses a separate operation to notify the LMS for the location report. Besides, the procedure for on-demand location retrival defined in cl.9.3.9 supports a list of VAL users or VAL UEs, making LMS to aggregate the location reports from all VAL user/UEs and send it in one response message towards the requesting LMC does not work in the current “request/response” operation semantic. Note: a similar communication model can be found in TS 23.280, cl.10.9.3.6.2 “On-demand usage of location information procedure” where the step 3 is “Location Information Notification”. In addtion, CR0023 agreed in 38e meeting is missing the update for API operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add location information notification in clause 9.3.9. Use “subscribe/notify” operation semantics for SS_LocationInfoRetrieval API. Add more operations for SS_LocationReporting API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect operation semantic for on-demand location retrieval leads to failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.9, 9.4.1, 9.4.4.2. 9.4.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.3.9	On-demand usage of location information procedure

The VAL server can request UE location information at any time by sending a location information request to the location management server, which may trigger location management server to immediately send the location report.

Figure 9.3.9-1 illustrates the high level procedure of on demand usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE.

Figure 9.3.9-1: On-demand usage of location information procedure

1.	VAL server sends a location information request to the location management server.

2.	The location management server acquires the latest location of the UEs being requested, by triggering an on-demand location report procedure as described in subclause 9.3.4, or from PLMN operator.

3.	Then, location management server immediately sends a location information response and may include the location information report including the latest location information acquired of one or more VAL users or VAL UEs.

4.	The location management server may further send the location information report including the latest location information acquired of one or more VAL users or VAL UEs in one or more location information notification.

5.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 5 can be skipped if not needed.

* * * Next Change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting | VAL server | Request /Response |
|  | Update_Trigger_Location_Reporting |  |  |
|  | Cancel_Trigger_Location_Reporting |  |  |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info |  |  |

* * * Next Change * * * *

#### 9.4.4.2	Obtain_Location_Info operation

API operation name: Obtain_Location_Info

Description: Request UEs location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.3

Outputs: Refer subclause 9.3.2.7

See subclause 9.3.9 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.4.x	Notifiy_Location_Info operation

API operation name: Notifiy_Location_Info

Description: Location information notification to the existing subscription.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.7

Outputs: Refer subclause 9.3.2.7

See subclause 9.3.9 for the details of usage of this API operation.

* * * End of Changes * * * *