3GPP TSG-SA WG6 Meeting #39-e	S6-201413

e-meeting, 31st August – 8th September 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	EEC-executed application context transfer

Spec:	3GPP TS 23.558

Agenda item:	7.5

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

2. Reason for Change

An Edge Enabler Client can perform detection, decision, and execution of the application context relocation. Also, the Edge Enabler Client can obtain and provide the source EAS with the target EAS information required for context transfer over edge enabling layer (via EDGE-1 and EDGE-3). Specifically, the Edge Enabler Client can detect that application context relocation is required and sequentially select target EES and target EAS. Then, the Edge Enabler Client informs the source Edge Enabler Server of context relocation to be required with providing information for target Edge Enabler Server and target Edge Application Server, of which information is eventually delivered to the source Edge Application Server via EDGE-3.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.4.0.

* * * First Change * * * *

8.8.2.X Application Context Relocation executed by the target Edge Enabler Server

Pre-conditions:

- The Application Client at the UE already has a connection to the source Edge Application Server.

- The Edge Enabler Client is able to communicate with the source Edge Enabler Server.

Figure 8.8.2.X-1: Application context relocation executed by Edge Enabler Client

1.	The Edge Enabler Client detects that application context relocation may be required.

2.	The Edge Enabler Client decides to proceed required procedures for application context relocation.

3.	Upon making a decision, the Edge Enabler Client determines target Edge Enabler Server by using the provisioned information or performing service provisioning procedure. Upon deciding target Edge Enabler Server, the Edge Enabler Client discovers and selects target Edge Application Server.

4.	The Edge Enabler Client informs the source Edge Enabler Server of information for the target Edge Enabler Server and target Edge Application Server, so that the source Edge Enabler Server notifies the source Edge Application Server of application context relocation to be performed and target Edge Application Server address for context transfer. Upon receiving the notification, the source Edge Application Server transfers application context to the target Edge Application Server.

NOTE X: If supported, the Edge Enabler Client can provide target Edge Application Server address to the Application Client, so that the source Edge Application Server can be informed by the Application Client of the application context relocation to be performed. This method to inform the necessity of application context relocation over application layer is implementation specific and out of scope of this specification.

NOTE X: The methods to transfer directly between the source Edge Application Server and the target Edge Application Server are implementation specific and out of scope of this specification.

5.	The Edge Enabler Client is informed by the source Edge Enabler Server of the completion. The Edge Enabler Client updates binding between Application Client and its corresponding Edge Application Server information.

Editor's note:	Information flows of the procedure is FFS.

* * * End of Change * * * *