3GPP TSG-SA WG6 Meeting #39-e	S6-201415

e-meeting, 31st August – 8th September 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	tEES-executed application context transfer

Spec:	3GPP TS 23.558

Agenda item:	7.5

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

This pCR provides an Edge Enabler Layer execution procedure that is aligned with an outcome of pre-meeting conference calls.

2. Reason for Change

A target EES can execute the application context relocation if it is informed by the Edge Enabler Client. The Target Edge Enabler Server decides to proceed required procedures for application context relocation (e.g. based on the information provided by the Edge Enabler Client). Upon making a decision, the target Edge Enabler Server determines target Edge Application Server and provides the target Edge Application Server address for context transfer the source Edge Application Server via the source Edge Enabler Server via EDGE-9.

The EEC-detected and tEES-executed context relocation can be used when the UE is not reachable to sEES (e.g. LADN) and/or AF Request (e.g. for subscribing UP path change notification service) is not supported.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.4.0.

* * * First Change * * * *

8.8.2.X Application Context Relocation executed by the target Edge Enabler Server

Pre-conditions:

- Edge Enabler Client is not able to communicate with source Edge Enabler Server.

Figure 8.8.2.X-1: Application context relocation executed by target Edge Enabler Server

1.	The Edge Enabler Client detects that application context relocation may be required.

2.	The Edge Enabler Client selects the target Edge Enabler Server by using the provisioned available Edge Enabler Server information or by performing service provisioning. Then, the Edge Enabler Client informs the target Edge Enabler Server that application context relocation is required.

3.	The Target Edge Enabler Server decides to proceed required procedures for application context relocation (e.g. based on the information provided by the Edge Enabler Client).

4.	Upon making a decision, the target Edge Enabler Server determines target Edge Application Server. The target Edge Application Server is informed of application context relocation to be performed.

5.	The target Edge Enabler Server provides the target Edge Application Server address for context transfer to the source Edge Enabler Server. The source Edge Enabler Server notifies the source Edge Application Server of application context transfer and the target Edge Application Server address for context transfer. Upon receiving the notification, the source Edge Application Server transfers application context to the target Edge Application Server.

NOTE X: The methods to transfer directly between the source Edge Application Server and the target Edge Application Server are implementation specific and out of scope of this specification.

6.	After successful application context relocation, the target Edge Enabler Server is notified of the application context relocation completion. Also, the target Edge Enabler Server informs the Edge Enabler Client of the application context relocation completion.

7.	The Edge Enabler Client is informed of the completion and target Edge Application Server information. The Edge Enabler Client stores the target Edge Application Server information (e.g. address) and updates binding between Application Client and its corresponding Edge Application Server information.

Editor's note:	More detailed description on the procedure to capture complete detection/decision/execution/notification is FFS.

* * * End of Change * * * *