3GPP TSG-SA WG6 Meeting #40-e	S6-202160

e-meeting, 16th – 24th November 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	EES profile in service provisioning

Spec:	3GPP TS 23.558 v1.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution replaces the EES endpoint and ECSP provider info in the service provisioning response and notification with the EES profile.

2. Reason for Change

EES profile contains the EES endpoint and ECSP provider info along with other information required by the EEC for downselect the EES(s).

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.1.0.

* * * First Change * * * *

#### 8.3.3.2	Service provisioning response

Table 8.3.3.2-1 describes the information elements for service provisioning response from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.2.-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| List of EDN configuration information | M | List of Edge Data Networks. |
| > EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the Edge Data Network. |
| >> DNN/APN | M | Data Network Name/Access Point Name |
| >> S-NSSAI | O | Network Slice information |
| >> EDN Topological Service Area (NOTE 2) | O | A list of Cell IDs or List of TAIs. The EDN serves UEs that are connected to the Core Network from one of these cells. |
| >List of EES profiles (NOTE 3) | M | Profiles of the Edge Enabler Servers of the Edge Data Network, as specified in clause 8.2.6. |
| > Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: The EDN Topological service area IE is included only if the EDN is a LADN. NOTE 3: EAS information included in the EES profile is limited to the EEC requested applications. If no Application Client profiles were present in the service provisioning request, the EAS information included in the EES profile is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). |  |  |

* * * Next Change * * * *

#### 8.3.3.5	Service provisioning notification

Table 8.3.3.5-1 describes the information elements for service provisioning notification from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.5.-1: Service provisioning notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the ECS for the request |
| List of EDN configuration information | M | List of Edge Data Networks. |
| > EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the Edge Data Network. |
| >> DNN/APN | M | Data Network Name/Access Point Name |
| >> S-NSSAI | O | Network Slice information |
| >> EDN Topological Service Area (NOTE 2) | O | A list of Cell IDs or List of TAIs. The EDN serves UEs that are connected to the Core Network from one of these cells. |
| >List of EES profiles (NOTE 3) | M | Profiles of the Edge Enabler Servers of the Edge Data Network, as specified in clause 8.2.6. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: The EDN Topological service area IE is included only if the EDN is a LADN. NOTE 3: EAS information included in the EES profile is limited to the EEC requested applications. If no Application Client profiles were present in the service provisioning subscription request, the EAS information included in the EES profile is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). |  |  |