3GPP TSG-SA WG6 Meeting #40-e	S6-202161

e-meeting, 16th – 24th November 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	DNAIs

Spec:	3GPP TS 23.558 v1.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution closes multiple ENs related to DNAIs.

2. Reason for Change

DNAI is an identifier of a user plane access to one or more DN(s) where applications are deployed. Its mapping with UPF is preconfigured at the SMF and the UPF in the form of traffic steering policy and is not necessarily a one-to-one mapping. One UPF can map to one or more DNAIs; multiple UPFs can map to the same DNAI.

DNAI can point to anything that can be targeted by an N6 connection, e.g. a data centre, an mobile edge host, a server etc. And usage of DNAI is deployment specific.

With this definition of the DNAI, the EAS and EES within the same EDN can associate with multiple DNAIs, which are associated with the EDN.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.1.0.

* * * First Change * * * *

#### 7.3.3.5	Edge Application Server service area

An EAS may have both, Geographical Service Area and Topological Service Area.

For some applications, ASPs may use EAS geographical service area to define an area where Edge Application Server's service is available.

The Topological Service Area of an Edge Application Server is limited by the Topological Service Area of the EDN it resides in. I.e. the EAS Topological Service Area is either same as the EDN service area or a subarea of the EDN service area it resides in.

NOTE:	While planning the EAS and EES deployments in an EDN, it needs to be ensured that the EES (where the EAS is registered at), is reachable by the UE when the EAS is reachable.

* * * Next Change * * * *

### 8.2.4	Edge Application Server Profile

Table 8.2.4-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| Application Client ID(s) | O | Identifies the Application Client(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The topological service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE may also indicate whether the EAS supports Application Context Transfer. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

* * * Next Change * * * *

### 8.2.6	Edge Enabler Server Profile

The Edge Enabler Server profile includes information about the EES and the services it provides.

Table 8.2.6-1: Edge Enabler Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| Edge Application Server IDs | M | List of Edge Application Server IDs registered with the EES. |
| EES Provider Identifier | O | The identifier of the EES Provider (such as ECSP) |
| EES Topological Service Area | O | The list of Cell IDs (or TAIs) serviced by this EES. EECs of UEs that are connected to the Core Network from a cell that is not in this list shall not be served by this EES. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in subclause 7.3.3.3) |
| List of EESDNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EES resides. |

Editor's Note:	Whether to include EAS DNAI(s) so that the ECS can utilize it for monitoring EAS traffic and initiate ACR is FFS.