3GPP TSG-SA WG6 Meeting #40-e	S6-202088

e-meeting, 16th – 24th November 2020	(revision of S6-20xxxx)

Source:	Ericsson

Title:	Add N6 routing info in EAS profile

Spec:	3GPP TS 23.558 v1.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution adds N6 routing info in EAS profile.

2. Reason for Change

Currently, the different edge entity can trigger AF traffic influence but only the EAS address (IP 3-tuple) and EAS DNAI are available. It is more flexible to provide routing profile ID and/or specific N6 routing info as part of the EAS profile.

3. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

### 8.2.4	Edge Application Server Profile

Table 8.2.4-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| Application Client ID(s) | O | Identifies the Application Client(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The topological service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE may also indicate whether the EAS supports Application Context Transfer. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Editor's Note: Should the EAS DNAIs be limited to a subset of DNAIs of the EES it is registered at is FFS.

* * * Next Change * * * *

#### 8.8.4.2	Application Context Relocation request

Table 8.8.4.2-1 describes information elements for the Application Context Relocation request sent from the EEC to the S-EES.

Table 8.8.4.2-1: Application Context Relocation request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| UE identifier | M | The identifier of the UE (i.e. GPSI). |
| Target EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the target EAS. |
| DNAI of the target EAS | O | DNAI information associated with the target EAS. |
| N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the target EAS DNAI. |
| EAS notification indication | M | Indicates whether to notify the EAS about the need of application context relocation. |

* * * End of Changes * * * *