3GPP TSG-SA6 Meeting #40-e	S6-202128

- Online, , 16th Nov 2020 - 24th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0028 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-to-UE broadcast/groupcast configuration by VAE layer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2020-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces the support capability at the VAE layer for the UE-to-UE broadcast configuration and delivery by VAE layer (based on the approved Solution #8 of TR 23.764). This capability avoids the V2X message flood / interference to surrounding V2X-UEs, by properly configuring transmission parameters from application layer (number Re-tx, QoS attributes, relay activation, bundling of messages to reduce the transmissions), and at the same time guarantees the successful reception of a safety/efficiency related messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause (9.x) is added for introducing the new VAE layer capability (clause 9.x.2 provides the information flows,clause 9.x.3 provides the procedure for the configuration and clause 9.x.4 shows the procedure for the broadcast/groupcast delivery via the VAE layer). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this feature is not enabled, V2X broadcast/groupcast message delivery will be uncoordinated (via V5-APP layer) and may either have negative impact on interference in co-channel deployments (e.g. in dense urban scenarios), or on excess of required resources for the msg deliveries, or could also impact the KPI fulfilment (in cases of urban scenarios with NLOS like intersections, the proposed configuration may help meeting the requirements by increasing the number of Re-Tx or QoS attributes (e.g. range) or activating relay UEs) . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

## 9.x	V2X groupcast/broadcast configuration by VAE layer

### 9.x.1	General

The VAE client configures policies for the UE-to-UE broadcast/groupcast delivery, based on the requirement received by the VAE server, to reduce V2X broadcast flooding, while ensuring meeting the KPIs for one or more V2X services in a VAE service area. Two procedures are provided for this feature: the UE-to-UE broadcast/groupcast configuration by VAE client (9.x.3) and the UE-to-UE delivery which is handled by VAE clients over V5-AE (9.x.4).

### 9.x.2	Information flows

#### 9.x.2.1	V2V configuration requirement request

Table 9.x.2.1-1 describes the information flow V2V configuration requirement request from the V2X application specific server to the VAE server.

Table 9.x.2.1-1: V2V configuration requirement request

| Information element | Status | Description |
| --- | --- | --- |
| V2X group ID | M (NOTE) | Identity of the V2X group for which the V2X application requirement is initiated. |
| V2X service ID | M (NOTE) | The V2X service ID for which application requirement corresponds to. |
| Candidate Relay V2X-UE ID list | O | List of identities of the V2X UEs, which are candidate to serve as application layer relays. |
| V2X application QoS requirements | O | The application QoS requirements (reliability, delay, jitter) for the V2X service. |
| NOTE: Either of the information element should be present. |  |  |

#### 9.x.2.2	V2V configuration requirement response

Table 9.x.2.2-1 describes the information flow V2V configuration requirement response from the VAE server to the V2X application specific server.

Table 9.x.2.2-1: V2V configuration requirement response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result corresponding to the V2V configuration requirement request. |

#### 9.x.2.3	V2X groupcast/broadcast configuration request

Table 9.x.2.3-1 describes the information flow V2X groupcast/broadcast configuration request from the VAE server to the VAE client.

Table 9.x.2.3-1: V2X groupcast/broadcast configuration request

| Information element | Status | Description |
| --- | --- | --- |
| VAE Server ID | M | Identity of the VAE server which is requester of the V2X groupcast/broadcast configuration |
| V2X group ID | M (NOTE) | Identity of the V2X group for which the V2X groupcast/broadcast configuration is requested. |
| V2X service ID | M (NOTE) | The V2X service ID for which the groupcast/broadcast configuration is requested |
| PC5 Provisioning policies/ parameters | O | PC5 provisioning policies/parameters to be used by the V2X-UEs within the V2X service (list provided in TS 23.287) |
| Relay V2X-UE ID list | O | List of identities of the V2X UEs to serve as application layer relays. |
| Minimum number of re-transmissions | O | Minimum Number of allowed re-transmissions for the V2X message delivery. |

#### 9.x.2.4	V2X groupcast/broadcast configuration response

Table 9.x.2.4-1 describes the information flow V2X groupcast/broadcast configuration response from the VAE client to the VAE server.

Table 9.x.2.4-1: V2X groupcast/broadcast configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result corresponding to the V2X groupcast/broadcast configuration request. |

#### 9.x.2.5	UE-to-UE groupcast/broadcast configuration request

Table 9.x.2.5-1 describes the information flow UE-to-UE groupcast/broadcast configuration request from a VAE client to another VAE client within the group or the V2X service.

Table 9.x.2.5-1: UE-to-UE groupcast/broadcast configuration request

| Information element | Status | Description |
| --- | --- | --- |
| VAE client ID | M | Identity of the VAE client which is requester of the UE-to-UE groupcast/broadcast configuration |
| V2X group ID | M (NOTE) | Identity of the V2X group for which the UE-to-UE groupcast/broadcast configuration is requested. |
| V2X service ID | M (NOTE) | The V2X service ID for which the UE-to-UE groupcast/broadcast configuration is requested |
| UE-to-UE groupcast/broadcast configuration policies/ parameters | M | The requested configuration policies to be applied for the UE-to-UE broadcast/groupcast communications. The policies / parameters may be the following: - activation / de-activation of the application relaying capability - adaptation of the application QoS parameters of the PC5 session - adaptation of the frequency and number of re-transmissions of the application messages - activation / de-activation of bundling of V2X messages to containers |
| NOTE: Either of the information element should be present. |  |  |

#### 9.x.2.6	UE-to-UE groupcast/broadcast configuration response

Table 9.x.2.6-1 describes the information flow UE-to-UE groupcast/broadcast configuration response from a VAE client to another VAE client within the group or the V2X service.

Table 9.x.2.6-1: UE-to-UE groupcast/broadcast configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result corresponding to the UE-to-UE groupcast/broadcast configuration request. |

#### 9.x.2.7	VAE message

Table 9.x.2.7-1 describes the information flow for a VAE client to deliver a VAE message to another VAE client.

Table 9.x.2.7-1: VAE message

| Information element | Status | Description |
| --- | --- | --- |
| VAE client ID | M | Identity of the VAE client which provides the V2X message |
| V2X group ID | M (NOTE 1) | Identity of the V2X group (for groupcast delivery) |
| V2X service ID | M (NOTE 1) | Identity of the V2X service (for broadcast delivery) |
| V2X Message ID(s) | O | Identity of the V2X message(s) within the VAE message (CPM, PCM, MCM, DENM) |
| Management Container | O | Management Container provides basic information about the transmitter, including the V2X-UE type (e.g. vehicle or RSU) and position |
| VAE payload | M (NOTE 2) | V2X message(s) payload carried by the VAE message. |
| NOTE 1: Either of the information element should be present. |  |  |
| NOTE 2: One or more V2X messages may be carried within a VAE payload (e.g. CPM, PCM, MCM, DENM) depending on whether V2X message bundling is activated |  |  |

### 9.x.3	V2X groupcast/broadcast configuration by VAE layer

#### 9.x.3.1	General

This subclause describes the procedure for V2X groupcast/broadcast configuration by the VAE layer.

#### 9.x.3.2	Procedure

Figure 9.x.3.2-1 illustrates the procedure where the VAE client configures policies for the UE-to-UE broadcast/ groupcast delivery to other VAE clients (within the V2X group or the V2X service), based on the requirement received by the VAE server.

Pre-conditions:

- The V2X UEs have connected to the V2X application server.

Figure 9.x.3.2-1: V2X groupcast/broadcast configuration by VAE layer

1.	The V2X application specific server provides an V2V configuration requirement request to the VAE server to manage the UE-to-UE broadcast/groupcast communication.

2. The VAE server sends a V2V configuration requirement response to the V2X application specific server.

3.	The VAE server provides a V2X broadcast/groupcast configuration request to VAE client 1.

NOTE:	How the V2X-UE 1 (or more V2X-UEs) is selected in step 2 is up to implementation.

4.	The VAE client 1 selects and stores one or more UE-to-UE groupcast/broadcast configuration policies per V2X application for the UE-to-UE broadcast/groupcast communications.

5.   The VAE client 1 sends a V2X broadcast/groupcast configuration response to VAE server to notify on the result.

6.	The VAE client 1 sends a request to VAE client N (can be any VAE client within the V2X group or V2X service) with the requested UE-to-UE groupcast/broadcast configuration policies for the UE-to-UE broadcast/groupcast communications.

NOTE:  	When V2X-UE 1 distributes the UE-to-UE groupcast/broadcast configuration message and how V2X-UE N (or more V2X UEs) is selected is up to implementation.

7.	The VAE client 1 receives a response from the VAE client 2, which includes either a positive or a negative acknowledgement of the message.

### 9.x.4	V2X groupcast/broadcast delivery by VAE layer

#### 9.x.4.1	General

This subclause describes the procedure for V2X groupcast/broadcast delivery by the VAE layer.

#### 9.x.4.2	Procedure

Figure 9.x.4.2-1 illustrates the procedure where the VAE client delivers the VAE message based on the configured policies (based on the procedure in clause 9.x.3).

Pre-conditions:

-	The VAE clients of the V2X-UEs have received and stored the UE-to-UE broadcast/groupcast configuration policies per V2X application.

Figure 9.x.4.2-1: V2X groupcast/broadcast delivery by VAE layer

1.	The V2X application specific client of the V2X UE 1 sends to the VAE client, the V2X message to be broadcasted/groupcasted. This message may be for example a PCM, CPM, MCM, DENM.

2.	The VAE client 1 performs the mapping between the V2X message and the configuration policy per application as provided in 9.x.3. The VAE client 1 forms a VAE message, based on the V2X message(s) and the applied configuration policy.

3.	The VAE message is sent from the transmitting V2X-UE 1 to the receiving V2X-UE N (which may be application relay or destination V2X-UEs). For the case when a V2X-UE  acts as application relay, the message is relayed to the destination VAE client.

4.	The V2X message is provided from the destination VAE client to the corresponding V2X application specific client. In case of message bundling, the receiving VAE client first un-bundles the V2X message and then sends to the V2X application specific client.