3GPP TSG-SA WG6 Meeting #40-e	S6-202172

e-meeting, 16th – 24th November 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	Deployments in relation with CAPIF

Spec:	3GPP TS 23.558 v1.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

The contribution provides deployment options in relation with CAPIF.

2. Reason for Change

TS cuurently does not depict the depoyments in reation with CAPIF. Based on solution #10 of TR 23.758, the contribution introduces distributed and centralized CAPIF deployments.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.1.0.

* * * First Change * * * *

## A.x	Deployments in relation with CAPIF

### A.x.1	General

### A.x.2	Distributed CAPIF

The Edge Enabler Server can support Edge Application Server's access to northbound APIs exposed by SCEF/NEF by providing distributed CAPIF functions as shown in Figure A.x.2-1.

Figure A.x.2-1: Edge Enabler Server supporting distributed CAPIF functions

The Edge Data Networks reside outside the PLMN trust domain as shown in Figure A.x.2-1. In Edge Data Network 2, the Edge Application Server and Edge Enabler Server are within the same ECSP trust domain. While in Edge Data Network 1, the Edge Enabler Server and the Edge Application Server are in the different ECSP trust domain.

The Edge Enabler Server of an Edge Data Network provides the following functions for network capability exposure:

-	the CAPIF core function as specified in 3GPP TS 23.222 [6] to support onboarding of Edge Application Servers (API invokers), publish of service APIs, discovery of service APIs and charging of service APIs invocations; and

-	the API exposing function as specified in 3GPP TS 23.222 [6] to expose the service APIs from SCEF/NEF to the Edge Application Servers via proxy or gateway function.

The following procedures are performed as specified in 3GPP TS 23.222 [6]:

-	The SCEF and NEF act as API exposing function and the service APIs from SCEF (T8) and NEF (Nnef) are published to the CAPIF core function 1. The service APIs are published to the Edge Enabler Servers (CAPIF core function 2 and CAPIF core function 3) from the CAPIF core function 1.

-	The Edge Application Server acts as an API invoker and is onboarded to the Edge Enabler Server (CAPIF core function 2 or CAPIF core function 3) within the Edge Data Network.

-	The Edge Application Servers (API invokers) are authenticated with Edge Enabler Server (CAPIF core function 2 or CAPIF core function 3).

NOTE:	The trusted Edge Application Servers can utilize the services of a centralized CAPIF core function deployed by the PLMN operator instead of the CAPIF core function of Edge Enabler Server deployed within the Edge Data Network.

-	The Edge Application Server discovers the service APIs published by the SCEF and NEF via the Edge Enabler Server (CAPIF core function 2 or CAPIF core function 3) within the Edge Data Network including the end point address of the API exposing function where the service API invocation is to be performed.

-	The Edge Application Server obtains authorization to invoke the service APIs of the SCEF and NEF from the Edge Enabler Server (CAPIF core function 2 or CAPIF core function 3).

-	The Edge Application Server invokes the service APIs of the SCEF and NEF after authorization by the Edge Enabler Server (API exposing function) and obtaining the UE identifier as specified in clause 8.6.5. The Edge Enabler Server (API exposing function) further invokes the service APIs of the SCEF or NEF in the 3GPP core network. EDGE-2 supports CAPIF-7e interactions corresponding to T8 (for SCEF) and N33 (for NEF).

### A.x.3	Centralized CAPIF

The Edge Enabler Server can support Edge Application Server (owned by 3rd party or by PLMN operator) access to northbound APIs exposed by SCEF/NEF by providing centralized CAPIF functions as shown in Figure A.x.3-1.

Figure A.x.3.-1: Edge Enabler Server supporting centralized CAPIF functions

The Edge Data Networks reside outside the PLMN trust domain as shown in Figure A.x.3-1. In Edge Data Network 2, the Edge Application Server and Edge Enabler Server are within the same ECSP trust domain. While in Edge Data Network 1, the Edge Enabler Server and the Edge Application Server are in the different ECSP trust domain.

The Edge Enabler Server of an Edge Data Network provides the following functions for network capability exposure:

-	the centralized CAPIF core function as specified in 3GPP TS 23.222 [6] to support onboarding of Edge Application Servers (API invokers), publish of service APIs, discovery of service APIs and charging of service APIs invocations; and

-	the API exposing function as specified in 3GPP TS 23.222 [6] to expose the service APIs from SCEF/NEF to the Edge Application Servers via proxy or gateway function.

The following procedures are performed as specified in 3GPP TS 23.222 [6]:

-	The SCEF and NEF act as API exposing function and the service APIs from SCEF (T8) and NEF (Nnef) are published to the centralized CAPIF core function. The service APIs exposed by the Edge Enabler Servers are published to the centralized CAPIF core function.

-	The Edge Application Server acts as an API invoker and is onboarded to the centralized CAPIF core function residing outside of the Edge Data Network.

-	The Edge Application Servers (API invokers) are authenticated with the centralized CAPIF core function.

-	The Edge Application Server discovers the service APIs published by the SCEF and NEF via the centralized CAPIF core function including the end point address of the API exposing function where the service API invocation is to be performed.

-	The Edge Application Server obtains authorization to invoke the service APIs of the SCEF and NEF from the centralized CAPIF core function.

-	The Edge Application Server invokes the service APIs of the SCEF and NEF after authorization by the Edge Enabler Server (API exposing function) and obtaining the UE identifier as specified in clause 8.6.5. The Edge Enabler Server (API exposing function) further invokes the service APIs of the SCEF or NEF in the 3GPP core network. EDGE-2 supports CAPIF-7e interactions corresponding to T8 (for SCEF) and N33 (for NEF).