3GPP TSG-SA WG6 Meeting #40-e	S6-202279

e-meeting, 16th – 24th November 2020	(revision of S6-202089)

Source:	Ericsson, Huawei

Title:	AF influence in ACR event

Spec:	3GPP TS 23.558 v1.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution makes clarifications for the two events in UPPM API.

2. Reason for Change

There is an EN left in UPPM API:

Editor's note:	With the “application context relocation monitoring” Event, the AF traffic influence executed by EAS/EES is for FFS.

Basically, the “ACR monitoring” event in UPPM API provides the possibility for the EAS to be notified about the need of ACR with T-EAS information. As described in 8.6.3.2.3 step 1, the EES, upon receipt of the UP path change notification from 3GPP CN, discovers the T-EAS and reports to the EAS with the T-EAS information.

The Edge Enabler Server detects the user plane path management event of the UE (e.g. receiving User plane path management event notification for the UE from the 3GPP core network). … If application context relocation monitoring Event is subscribed, the Edge Enabler Server checks the target DNAI is not in the EAS profile of the subscribing EAS, and further checks a target Edge Application Server is available at the target DNAI as described in steps 2-4 of clause 8.8.3.2.

For the traffic influence part, the EAS may

- Do it by itself upon receipt of the notification; or

- Ask the EES to do it.

It is then proposed to have such AF influence indication for the “application context relocation monitoring” Event.

Revision history:

R2: separate the two events in UPPM API with specific description & IEs. Fix small errors.

R3: remove “AF traffic influence indication”. Let EES influence AF traffic in the “ACR monitoring” event.

R4: remove the “AF influence action” in the sEES. Remove change on change.

Merging content from 2312:

- 8.6.3.2.2. add example in step 1b;

- 8.6.3.2.3, add notifying tEAS endpoint in step 1b;

- 8.6.3.3.3, add tEAS endpoint for “ACR monitoring” event.

3. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

##### 8.6.3.2.2	Subscribe

Figure 8.6.3.2.2-1 illustrates the subscribe operation between the Edge Application Server and the Edge Enabler Server for user plane path management event notifications.

Figure 8.6.3.2.2-1: User plane path management event API: Subscribe operation

1.	The Edge Application Server sends user plane path management event subscribe request for tracking the UE's user plane path change continuously. The Edge Application Server shall include UE Identifier or UE group Identifier.

a.	The Edge Application Server may include the "user plane path change" event to indicate the EES to notify the Edge Application Server when the EES detects there is a user plane path chanage for the application traffic and the Edge Application Server may include Subscription Type (Early and/or Late notification defined in clause 5.6.7 of 3GPP TS 23.501 [2]) and/or Indication of EAS Acknowledgement in the event subscription.

b.	The Edge Application Server may include the "application context relocation monitoring" event to indicate the EES to notify the Edge Application Server when the EES detects there is a need for the application context relocation (e.g. when target EAS is available at the target DNAI). The Edge Application Server may also include the Event Filter to specify the conditions to match for notifying the event, e.g., inter-EDN mobility.

2.	The Edge Enabler Server checks if the Edge Application Server is authorized for this operation. If authorized, Edge Enabler Server checks if there exists a subscription with the 3GPP core network for the user plane path management event notifications corresponding to the UE information obtained in step 1 as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], which may be triggered by other Edge Application Server for the same UE. The Edge Enabler Server checks the availability of the user plane path management event service for the UE.

a.	if a subscription with 3GPP core network does not exist, then the Edge Enabler Server subscribes with the 3GPP core network (PCF, NEF or SCEF+NEF) for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3] If the Edge Application Server provides Subscription Type and/or Indication of EAS Acknowledgement, the Edge Enabler Server include the type of subscription and/or the indication of "AF acknowledgement to be expected" as information on AF subscription to corresponding SMF events within the AF Request;

b.	if a subscription with 3GPP core network exists, then the Edge Enabler Server uses the locally cached user plane path management event notification information of the UE to respond to the Edge Application Server.

The Edge Enabler Server stores the subscription related to the Edge Application Server.

3.	If the event is "user plane path change", the Edge Enabler Server may subscribe to UE expected behaviour analytics (UE mobility and UE communication) for the group of UEs as described in 3GPP TS 23.288 [18].

4.	If Edge Application Server is authorized, the Edge Enabler Server responds with user plane path management event subscribe response. If Edge Application Server is not authorized, the Edge Enabler Server provides a rejection response with cause information.

If the target UE and the 3GPP network support mobility between 5GC and EPC, the Edge Enabler Server monitors the availability of the user plane path management event notification from the 3GPP network by utilizing Nnef_APISupportCapability or Availability of service APIs event notifications provided by the CAPIF core function.

* * * Next Change * * * *

##### 8.6.3.2.3	Notify

Figure 8.6.3.2.3-1 illustrates the notify operation between the Edge Enabler Server and the Edge Application Server for continuous User plane path management event notifications.

Figure 8.6.3.2.3-1: User plane path management event API: Notify operation

1.	The Edge Enabler Server detects the user plane path management event of the UE (e.g. receiving User plane path management event notification for the UE from the 3GPP core network).

a.	If "user plane path change" Event is subscribed, the Edge Enabler Server may cache the detected User plane path management event notification locally with timestamp as the latest information of the UE(s) and start the notification aggregation for a group of UEs. The Edge Enabler Server decides whether to aggregate and the aggregation period based on the analytics result received from the 3GPP Core Network, local policy and User Plane path management subscription information received from the Edge Application Server. The Edge Enabler Server determines to notify the user plane path management event notification information (e.g., DNAI) to the Edge Application Servers which has subscribed for the "user plane path management" event.

b.	If "application context relocation monitoring" Event is subscribed, based on the detected user plane path change report sent from the 3GPP core network, the Edge Enabler Server checks whether the target DNAI is in the EAS profile of the subscribing EAS, if not it further checks whether a target Edge Application Server is available at the target DNAI as described in steps 2-4 of clause 8.8.3.2. If a target Edge Application Server is available, the Edge Enabler Server notifies the Edge Application Server with target EAS endpoint.

Editor's note:	With the “application context relocation monitoring” Event, the AF traffic influence executed by EAS/EES is for FFS.

2.	The Edge Enabler Server sends user plane path management event notification to the Edge Application Server. The Edge Enabler Server includes the user plane path management event notification information of the UE(s) and optionally the timestamp. If the event triggering the notification is DNAI change, the timestamp can be included to indicate the age of the user plane path management event notification information. The Edge Enabler Server may only provide part of information included in the user plane path management event notification from 3GPP network (e.g. target DNAI). If the Edge Application Server had provided "Indication of EAS acknowledgement", the Edge Enabler Server waits for acknowledgement from the EAS before it sends AF acknowledgement to the 3GPP core network.

3.	If the Edge Application Server had included Indication of EAS Acknowledgement within the user plane path management event subscribe request described in clause 8.6.3.2.1, the Edge Application Server sends EAS Acknowledgement as a response to user plane path management event notification to the Edge Enabler Server either immediately or after the required application context transfer is completed. The Edge Application Server may reply in negative, e.g., the Edge Application Server may determine not to perform application context relocation. Then, the Edge Enabler Server sends the AF acknowledgement to the 3GPP core network.

Editor's Note:	It is FFS whether the EAS can provide N6 traffic routing details, target EAS address, and notification target address of the target EAS to the EES.

Editor's Note:	The relationship between the step 3 and the application relocation notify in clause 8.8.2.3 and 8.8.2.5 is FFS.

* * * Next Change * * * *

##### 8.6.3.3.1	User plane path management event subscribe request

Table 8.6.3.3.1-1 describes the information elements for a user plane path management event subscribe request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.3.3.1-1: User plane path management event subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID (NOTE) | O | The identifier of the UE (i.e. GPSI or identity token) |
| UE Group ID (NOTE) | O | Identifies a group of UEs (i.e. internal group ID or external group ID) |
| Event ID(s) | M | Event ID: - user plane path change - application context relocation monitoring |
| Event Report | M | Event Reporting Information as specified in 3GPP TS 23.502 [3] |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Type of subscription | O | Indicates Early and/or Late notification to inform if the notification needs to be received before and/or after UP path configuration. Applicable for the "user plane path change" event. |
| Indication of EAS acknowledgement | O | This IE indicates the EES to include indication of "AF acknowledgement to be expected" within the AF request for subscribing UP path management events to 3GPP network and that the EAS will provide an acknowledgement as a response for the notifications of UP path management events to the EES. Applicable for the "user plane path change" event. |
| Event Filter | O | Event filter as specified in 3GPP TS 23.501 [2] |
| NOTE: Either UE ID or UE Group ID shall be provided. |  |  |

Editor's note:	The detailed applicable information in "Event Report" is FFS.

* * * Next Change * * * *

##### 8.6.3.3.3	User plane path management event notification

Table 8.6.3.3.3-1 describes the information elements for a user plane path management event notification from the Edge Enabler Server to the Edge Application Server.

Table 8.6.3.3.3-1: User plane path management event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| List of event notifications | M | A list of event notifications for one or more UEs. |
| > Event report | M | Event reporting information as specified in clause 5.2.8.3.1 of 3GPP TS 23.502 [3] |
| > Timestamp | O | The timestamp of each event report. |
| > Target EAS endpoint | O | The target EAS endpoint. Applicable for the "application context relocation monitoring" event. |

* * * End of Changes * * * *