3GPP TSG-SA WG6 Meeting #40-e	S6-202218_Rev1

e-meeting, 16th – 24th November 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	Pseudo-CR on EN additional APIs

Spec:	3GPP TR 23.700-24 V1.1.0

Agenda item:	8.5

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan (basavarajjp@samsung.com)

1. Introduction

This pCR proposes resolving some of the editor's notes as per the discussion during [Pre-SA6#40-e conference call] 5GMARCH call #2 held on 5th Nov, 2020.

2. Reason for Change

Following editor's notes is resolved in this pCR:

| EN # | Clause # |  | EN | Reasoning |
| --- | --- | --- | --- | --- |
| 52 | 6.23.1 | Description [Solution 23: APIs provided by MSGin5G server] | Editor's Note: More APIs are FFS. | Further APIs can be added in normative phase. |
| 53 | 6.23.2.3 | Group Message Delivery API | Editor's Note: Group Message Delivery API is FFS. | Need for this API is not yet clear. |

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-24 v1.1.0.

* * * First Change * * * *

## 6.23	Solution 23: APIs provided by MSGin5G server

### 6.23.1	Description

This solution addresses the following identified issues and gaps related with the interaction between AS and MSGin5G Server under Key Issue #9.

6.23.2	APIs provided to Application Server

MSGin5G server provides restful APIs to Application Server.

#### 6.23.2.1 Point-to-Point Message Delivery API

MSGin5G server provides Point-to-Point Message Delivery API to Application Server.

The Application Server may use this API with the information elements in Table 6.11.1.4-1 for delivering a message to a recipient (i.e. a UE).

#### 6.23.2.2 Broadcast Message Delivery API

MSGin5G server provides Broadcast Message Delivery API to Application Server.

The Application Server may use this API with the information elements in Table 6.23.2.2-1 for delivering broadcast message to UEs in a Service Area.

Table 6.23.2.2-1: Message Delievery API request

| Information element | Status | Description |
| --- | --- | --- |
| Originator ID | M | The identifier of Application Server |
| Service Area ID | M | The identifier of Group if the message is broadcast message. |
| Application ID | O | The identifier of application |
| Message Content | M | Message content that will be used as a payload of a MSGin5G message |
| Security Credentials | O | Security information required by the MSGin5G Server. |

#### 6.23.2.3 Application Server Registration API

MSGin5G server provides Application Server Registration API to Application Server, the Application Server may use this API to register on the MSGin5G Server. The API request includes the information elements in Table 6.11.1.3-1, the API response includes the information elements in Table 6.11.1.3-2.

### 6.23.3	Solution evaluation

This solution defines the MSGin5G server exposing APIs to the consumers i.e. application server. Currently Application-to-Point Message Delivery API, Broadcast Message Delivery API and Application Server Registration API are identified. Need for Group message delivery API is yet to be identified. And more such APIs are expected to be identified during the normative phase.

* * * Next Change * * * *